3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201433

Electronic Meeting, 17 – 25 January 2022

Agenda Item:	8.9.6.2.2

Source:	Ericsson

Title:	View on Rel-17 NB-IoT BS demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN#94-e confirmed WI Additional enhancements for NB-IoT and LTE-MTC is completed from RAN1 perspective and set the total completion level of core part to 85% [1].

According to the latest WID of Additional enhancements for NB-IoT and LTE-MTC [2], we list the objectives for NB-IoT affecting to RAN4 BS demodulation requirements as follows:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4]

  - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4]

# 2	NPUSCH demodulation

## 2.1	RAN1 agreements

RAN1 agreed to support 16QAM for NPUSCH format 1:

- With Multi-tone: 3, 6, and 12

- Without repetition

- With maximum TBS with 2,536 bits

Table 1 shows the TBS table agreed in RAN1 to support NPUSCH with 16QAM. According to RAN1 agreements, NPUSCH format 1 with 16QAM is scheduled with 14 ≤ ITBS ≤ 21.

Table 1	TBS table applicable for NPUSCH format 1 with 16QAM

|  | IRU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ITBS | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

Since the existing NPUSCH format 1 demodulation requirements are specified for BPSK and QPSK, RAN4 need to define new NPUSCH format 1 demodulation requirements with 16QAM.

Proposal 1: RAN4 define NPUSCH format 1 demodulation requirements with 16QAM.

## 2.2	Simulation assumption

RAN4 has already defined several multi-tone NPUSCH format 1 demodulation requirements in TS36.104 8.5.1, however, the existing requirements assume repetitions. Considering the purpose of introducing 16QAM, we think it is natural to verify the 12 tones. We therefore propose to assume the following assumption by reusing A16-5:

- Subcarrier spacing: 15kHz

- Number of allocated subcarriers: 12

- TBS: 280bits / IRU: 0 (ITBS =15, target CR=0.53, according to Table 1)

- Repetition number: 1

- Propagation condition: ETU 1Hz Low

- Antenna configuration: 1x2

Proposal 2: RAN4 assumes the simulation assumption below for NPUSCH format 1 with 16QAM.

| Test number | Subcarrier spacing | Number of allocated subcarriers | Propagation condition | TBS / ITBS | IRU | Repetition number | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | 12 | ETU 1Hz low | [280] bits / [15] | [0] | 1 | 1x2 |

# 3	Summary

Proposal 1: RAN4 define NPUSCH format 1 demodulation requirements with 16QAM.

Proposal 2: RAN4 assumes the simulation assumption below for NPUSCH format 1 with 16QAM.

| Test number | Subcarrier spacing | Number of allocated subcarriers | Propagation condition | TBS / ITBS | IRU | Repetition number | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | 12 | ETU 1Hz low | [280] bits / [15] | [0] | 1 | 1x2 |

# References

- RP-213365, “Status report for WI: Additional enhancements for NB-IoT and LTE-MTC”, Rapporteur (Huawei).

- RP-211340, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon.