3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201436

Electronic Meeting, 17 January – 25 January 2022

Agenda Item:	6.20.4

Source:	Ericsson

Title:	UE demodulation and CSI reporting requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN#94-e confirmed that the core part of WI Support of reduced capability NR devices is completed from RAN1 perspective and the core part completion level is set to 75% [1]. This contribution discusses our view on UE demodulation and CSI reporting requirements for RedCap.

# 2	Common configurations for UE demodulation and CSI reporting requirements

According to the WID objectives [2], RedCap UEs have reduced capabilities as follows:

- Reduced maximum UE bandwidth

  - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20MHz.

  - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100MHz.

- Reduced minimum number of Rx branches

  - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1.

  - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1.

- Maximum number of DL MIMO layers

  - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported.

  - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

- Relaxed maximum modulation order

  - Support of 256QAM in DL is optional (instead of mandatory) for an FR1 RedCap UE, that is, the maximum modulation order for RedCap UE is 64QAM.

- Duplex operation

  - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

- CA/DC is not supported, SA only

From the WID, it is observed RedCap UE is characterized by 1Rx/2RX, maximum CBW with 20MHz/100MHz for FR1/FR2, and HD-FDD for FDD band. Since RAN4 requirements ensures the minimum performance requirements, we propose to define UE demodulation and CSI reporting requirements for RedCap UE with 1Rx/2Rx with reduced maximum bandwidth.

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for RedCap UE with:

- Number of receive antennas: 1Rx and 2Rx

- Both FR1 and FR2

- Modulation order: Up to 64QAM

- No requirements for CA, EN-DC, NE-DC, and NR-DC scenarios

Proposal 2: RAN4 defines the RedCap UE demodulation requirements with:

- FR1 HD-FDD: SCS=15kHz and CBW=10MHz

- FR1 TDD: SCS=30kHz and CBW=20MHz

- FR2 TDD: SCS=120kHz and CBW=100MHz

# 3	UE demodulation requirements

## 3.1	Scope of requirements

Since RAN4 has defined UE demodulation and CSI reporting requirements with 2Rx and 4Rx, RAN4 need to define new demodulation requirements for PDSCH, PDCCH, and PBCH, with 1Rx. For RedCap with 2Rx, we also need new requirements because of the reduced maximum bandwidth and HD-FDD. We think it is also important to verify the peak rate, i.e., SDR test, considering the use case of wearable requiring the DL peak rate of 150Mbps, according to WID.

Proposal 3: Define the UE demodulation requirements of PDSCH, SDR, PDCCH, and PBCH with 1Rx/2Rx.

## 3.2	DL/UL scheduling for HD-FDD

In Rel-15/16 RAN4 UE demodulation requirements are defined based on full-duplex FDD for FDD bands, where both DL and UL slots can be used simultaneously for UE reception/transmission. On the other hand, RedCap UE assumes HD-FDD operation type A where either UL slot or DL slot can be used. This assumption affects to the UE demodulation and CSI reporting requirements such as FRC and CQI reporting delay.

RAN4 has defined the UE demodulation requirements for LTE LTE-MTC Cat-M1/M2 with HD-FDD type B in TS36.101 8.11. For LTE-MTC, RAN4 assumes the scheduling pattern ‘DDDGUUUG’, where ‘D’ means DL subframe, ‘U’ means UL subframe, and ‘G’ means RF switching period. One option is to reuse the LTE-MTC HD-FDD pattern for RedCap HD-FDD scheduling, as shown in Figure 1. Since RedCap UE can use HD-FDD type A, we can configure the pattern ‘DDDDUUUU’.

Figure 1	HD-FDD DL/UL scheduling by reusing the LTE-MTC HD-FDD pattern.

The reason RAN4 used this pattern for LTE-MTC is because HARQ-ACK feedback timing is fixed to 4ms. However, since NR can set different the HARQ-ACK feedback delay with DCI, we can allocate more DL slots to reduce the UE test time. Another issue with this pattern is the periodicity of 8ms. Since RAN4 NR UE demodulation requirements set SSB/TRS periodicity to 20ms, UE cannot receive SSB/TRS when the slots of SSB/TRS is overlapped with UL transmission like HARQ-ACK or CSI reporting.

Figure 2	HD-FDD DL/UL scheduling applicable for RedCap UE with HD-FDD.

Figure 2 shows our proposal for RedCap HD-FDD pattern where we can reuse the existing TDD UL-DL pattern FR1.15-1. We think this pattern is more suitable for NR compared with Figure 1 because we can keep 4 HARQ processes, more DL slots are allocated, and the periodicity is 5ms.

Proposal 4: HD-FDD SCS=15kHz uses the DL/UL pattern with ‘DDDDU’, where ‘D’ denotes a DL slot and ‘U’ denotes a UL slot.

- The number of slots between PDSCH and corresponding HARQ-ACK information for slot index i (i = {0, 1, …, 9}) per frame is:

  - 4 if mod(i, 5) = 0

  - 3 if mod(i, 5) = 1

  - 2 if mod(i, 5) = 2

  - 6 if mod(i, 5) = 3

Although RedCap UE assumes HD-FDD type A, UE still requires RF switching period corresponding to 1 OFDM symbol in SCS=15kHz, according to RAN1 assumption. Since the UE demodulation requirements verify the downlink signal reception performance, we propose to allocate 14 symbols in DL slots regardless it is before/after the UL transmission as shown in Figure 3. This means the start symbol index is 1 in the UL slots just after the DL slots and the last symbol index is 12 in the UL slots just before the DL slots.

Figure 3	DL/UL symbol allocation during the DL-UL switching for FD-FDD type A.

Proposal 5: For UE demodulation requirements for RedCap with HD-FDD SCS=15kHz, PDCCH/PDSCH are scheduled with the assumption 14 symbols are available in all the DL slots. PUSCH/PUCCH are scheduled with the assumption the start symbol index is 1 for slots just after the DL scheduling, and the last symbol index is 12 for slots just before the DL scheduling.

## 3.3	PDSCH demodulation requirements

In Rel-15/16, RAN4 has defined many PDSCH demodulation requirements considering the RAN1 physical layer specification. Since RedCap UE targets the low complexity/cost devices including single Rx and limited bandwidth, we propose PDSCH demodulation requirements for RedCap are based on the PDSCH mapping type A defined in Rel-15 specified in TS38.101-4 subclauses 5.2.2.1.1, 5.2.2.2.1, and 7.2.2.2.1.

If we consider the supported modulation order, we propose to define three test cases for 1Rx/2Rx for each FR1 HD-FDD, FR1 TDD and FR2 TDD.

Proposal 6: Define PDSCH demodulation requirements for RedCap UE with the following test setup.

FR1 HD-FDD

| Test number | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 5.2.2.1.1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x1 low | 70% of peak rate | Test 1-1 |
| 1-2 | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | Test 1-4 |
| 1-3 | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | Test 2-1 |
| 2-1 | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x2 ULA low | 70% of peak rate | Test 1-1 |
| 2-2 | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | Test 1-4 |
| 2-3 | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | Test 2-1 |
| Note 1: Use ‘DDDDU’ for DL/UL scheduling. |  |  |  |  |  |  |

FR1 TDD

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 5.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x1 low | 70% of peak rate | Test 1-1 |
| 1-2 | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | Test 1-4 |
| 1-3 | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | Test 2-1 |
| 2-1 | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x2 low | 70% of peak rate | Test 1-1 |
| 2-2 | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 low | 70% of peak rate | Test 1-4 |
| 2-3 | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 low | 70% of peak rate | Test 2-1 |

FR2 TDD

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x1 low | 70% of peak rate | Test 1-1 |
| 1-2 | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA30-300 | 2x1 low | 70% of peak rate | Test 2-2 |
| 1-3 | 100MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDLA30-75 | 2x1 low | 70% of peak rate | Test 2-6 |
| 2-1 (Note 1) | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | Test 1-1 |
| 2-2 (Note 1) | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | Test 2-2 |
| 2-3 (Note 1) | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | Test 2-6 |
| Note 1 Tests 2-1, 2-2, and 2-3 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |  |  |

The detailed simulation parameters are provided in Appendix A.1.

## 3.4	PDCCH demodulation

There is no limitation for PDCCH CORESET configuration for RedCap UE. We therefore propose to define PDCCH with all the possible aggregation levels 2, 4, 8, and 16 by reusing the existing test case specified in TS38.101-4 5.3.2.1, 5.3.2.2, and 7.3.2.2.

Proposal 7: Define PDCCH demodulation requirements for RedCap UE with the following test setup.

FR1 HD-FDD SCS=15kHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 5.3.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x1 | 1% | 1Tx Test 1 |
| 1-2 | 10 | 48 | 2 | 4 | TDLA300-100 | 1x1 | 1% | 1Tx Test 2 |
| 1-3 | 10 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 2Tx Test 1 |
| 1-4 | 10 | 48 | 2 | 16 | TDLC300-100 | 1x1 | 1% | 1Tx Test 5 |
| 2-1 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x2 low | 1% | 1Tx Test 1 |
| 2-2 | 10 | 48 | 2 | 4 | TDLA300-100 | 1x2 low | 1% | 1Tx Test 2 |
| 2-3 | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 2Tx Test 1 |
| 2-4 | 10 | 48 | 2 | 16 | TDLC300-100 | 1x2 low | 1% | 1Tx Test 5 |
| Note 1 Tests 2-1, 2-2, 2-3, and 2-4 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |  |  |  |

FR1 TDD SCS=30kHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 5.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 20 | 48 | 2 | 2 | TDLA30-10 | 1x1 | 1% | 1Tx Test 1 |
| 1-2 | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 1% | 1Tx Test 2 |
| 1-3 | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 2Tx Test 1 |
| 1-4 | 20 | 48 | 2 | 16 | TDLC300-100 | 1x1 | 1% | 1Tx Test 3 |
| 2-1 | 20 | 48 | 2 | 2 | TDLA30-10 | 1x2 low | 1% | 1Tx Test 1 |
| 2-2 | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1% | 1Tx Test 2 |
| 2-3 | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 2Tx Test 1 |
| 2-4 | 20 | 48 | 2 | 16 | TDLC300-100 | 1x2 low | 1% | 1Tx Test 3 |

FR2 TDD SCS=120kHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 7.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100 | 60 | 1 | 2 | TDLA30-75 | 1x1 | 1% | 1Tx Test 1 |
| 1-2 | 100 | 60 | 1 | 4 | TDLA30-300 | 1x1 | 1% | 1Tx Test 2 |
| 1-3 | 100 | 60 | 1 | 8 | TDLA30-75 | 2x1 low | 1% | 2Tx Test 1 |
| 1-4 | 100 | 60 | 2 | 16 | TDLA30-75 | 2x1 low | 1% | 2Tx Test 2 |
| 2-1 | 100 | 60 | 1 | 2 | TDLA30-75 | 1x2 low | 1% | 1Tx Test 1 |
| 2-2 | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 1Tx Test 2 |
| 2-3 | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 2Tx Test 1 |
| 2-4 | 100 | 60 | 2 | 16 | TDLA30-75 | 2x2 low | 1% | 2Tx Test 2 |
| Note 1 Tests 2-1, 2-2, 2-3, and 2-4 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |  |  |  |

The detailed simulation parameters are provided in Appendix A.2.

## 3.5	PBCH demodulation

RAN4 defines the PBCH demodulation requirements (i.e., MIB decoding performance) in Rel-15 although it is not tested in RAN5. We think it is also beneficial for RedCap UE to define PBCH demodulation requirements to understand the SIB index acquisition performance or MIB decoding performance in RRM. To minimum the workload in RAN4, we propose to limit the configuration only for SCS=15/30/120kHz with the assumption SS/PBCH block is not known to consider the minimum performance requirements.

Proposal 8: Define PBCH demodulation requirements with the following test setup by assuming SS/PBCH block is not known.

| Test number | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Table 5.4.2.1-2 |
| 1-2 | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Table 5.4.2.2-2 |
| 1-3 | 100MHz / 120kHz | TDLA30-300 | 1x1 | 1% | Table 7.4.2.2-2 |
| 2-1 | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Table 5.4.2.1-2 |
| 2-2 | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Table 5.4.2.2-2 |
| 2-3 | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Table 7.4.2.2-2 |
| Note 1 Tests 2-1, 2-2, and 2-3 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |

The detailed simulation parameters are provided in Appendix A.3.

## 3.6	SDR

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. Although RedCap targets low complexity/cost UE implementation, it also requires DL peak rate of 150Mbps according to WID. Therefore we think RAN4 need to define SDR requirements to ensure UE can achieve the peak rate.

According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for RedCap UEs such and rank 1/2 and QPSK/16QAM/64QAM as shown in Table 1. RAN4 only need to set the applicability rule for the RedCap UE.

Table 1	MCS indexes for indicated UE capabilities applicable for RedCap UE in FR1 and FR2.

| Received antenna | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 1Rx/2Rx RedCap UE | 1 | 6 | 1 | 27 |
|  | 1 | 6 | 0.8 | 23 |
|  | 1 | 6 | 0.75 | 22 |
|  | 1 | 6 | 0.4 | 14 |
|  | 1 | 4 | 1 | 16 |
|  | 1 | 4 | 0.8 | 16 |
|  | 1 | 4 | 0.75 | 16 |
|  | 1 | 4 | 0.4 | 10 |
|  | 1 | 2 | 1 | 9 |
|  | 1 | 2 | 0.8 | 9 |
|  | 1 | 2 | 0.75 | 9 |
|  | 1 | 2 | 0.4 | 4 |
| 2Rx RedCap UE | 2 | 6 | 1 | 27 |
|  | 2 | 6 | 0.8 | 23 |
|  | 2 | 6 | 0.75 | 22 |
|  | 2 | 6 | 0.4 | 14 |
|  | 2 | 4 | 1 | 16 |
|  | 2 | 4 | 0.8 | 16 |
|  | 2 | 4 | 0.75 | 16 |
|  | 2 | 4 | 0.4 | 10 |
|  | 2 | 2 | 1 | 9 |
|  | 2 | 2 | 0.8 | 9 |
|  | 2 | 2 | 0.75 | 9 |
|  | 2 | 2 | 0.4 | 4 |

Proposal 9: Define SDR test for RedCap UE assuming both 1Rx and 2Rx.

# 4	CSI reporting requirements

## 4.1	Scope of requirements

According to the WID, the RedCap UE can be used for wearable. In this scenario, we can expect the devices are moving and it is important UE should report the correct CSI according to the channel condition. RAN4 has defined CQI, PMI and RI reporting requirements in Rel-15. For RedCap UE, we propose to define CQI reporting requirements only for RedCap UEs. Since RedCap UE mainly assumes single Rx, we don’t need to verify RI because it is clear the maximum MIMO layer is 1. It may be beneficial to define the PMI reporting test with Type-I precoder. However since RedCap UE assumes low complexity baseband processing, PMI reporting with 16 or 32 CSI-RS ports is heavy.

Regarding the CQI reporting, we propose to define both the static condition and fading condition requirements for RedCap UE with 1Rx/2Rx. Since the existing CQI reporting tests verifies the CQI tables 2 (256QAM), we need to define CQI reporting requirements with CQI table 1 (64QAM) not only for 1Rx but also for 2Rx.

Proposal 10: Define the CQI reporting definition test for RedCap 1Rx/2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

For the CQI reporting under fading condition, considering the maximum CBW is 20MHz for FR1 and 100MHz for FR2, we think it is sufficient to define wideband CQI reporting test only.

Proposal 11: Define the wideband CQI reporting under fading condition for RedCap 1Rx/2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

## 4.2	CQI report scheduling

For the HD-FDD SCS=15kHz case, the existing CQI reporting tests with static/fading conditions for (full duplex) FDD with SCS=15kHz configure NZP-CSI-RS/ZP-CSI-RS/CSI-IM with (periodicity, offset) = (5, 1) slots, and the periodic CSI reporting is configured with (periodicity, offset) = (5, 0) slots. With this configuration, the CQI delay becomes 8ms. If we use the UL/DL scheduling in Figure 2, it is observed the existing CSI-RS and CQI reporting configuration can apply as shown in Figure 4.

Figure 4	Test setup of CQI report in HD-FDD with SCS=15kHz

Proposal 12: CQI reporting tests for RedCap UE with HD-FDD SCS=15kHz uses the following configurations:

- DL/UL pattern with ‘DDDDU’.

- NZP-CSI-RS/ZP-CSI-RS/CSI-IM is configured with (periodicity, offset) = (5, 1) slots.

- Periodic CSI reporting is configured with (periodicity, offset) = (5, 0) slots.

- CQI delay: 8ms

# 5	Specification structure

Table 2 through Table 5 shows the possible specification structures needed for RedCap UE demodulation requirements. RAN4 need to discuss further when RAN4 agree with the scope of UE demodulation and CSI reporting for RedCap UE. We also need to add new FRC tables and applicability sections.

Table 2	Specification structure for RedCap UE Demodulation performance requirements (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 5.2 | PDSCH demodulation requirements |  |
| 5.2.1 | 1RX requirements |  |
| 5.2.1.1 | FDD |  |
| 5.2.1.1.1 | Minimum requirements for RedCap UEs | New section |
| 5.2.1.2 | TDD |  |
| 5.2.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 5.2.2 | 2RX requirements |  |
| 5.2.2.1 | FDD |  |
| 5.2.2.1.[16] | Minimum requirements for RedCap UEs | New section |
| 5.2.2.2 | TDD |  |
| 5.2.2.2.[15] | Minimum requirements for RedCap UEs | New section |
| 5.3 | PDCCH demodulation requirements |  |
| 5.3.1 | 1RX requirements |  |
| 5.3.1.1 | FDD |  |
| 5.3.1.1.1 | Minimum requirements for RedCap UEs | New section |
| 5.3.1.2 | TDD |  |
| 5.3.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 5.3.2 | 2RX requirements |  |
| 5.3.2.1 | FDD |  |
| 5.3.2.1.[4] | Minimum requirements for RedCap UEs | New section |
| 5.3.2.2 | TDD |  |
| 5.3.2.2.[4] | Minimum requirements for RedCap UEs | New section |
| 5.4 | PBCH demodulation requirements |  |
| 5.4.1 | 1RX requirements |  |
| 5.4.1.1 | FDD | New section |
| 5.4.1.2 | TDD | New section |
| 5.4.2 | 2RX requirements |  |
| 5.4.2.1 | FDD | Add minimum requirement tables for RedCap UEs |
| 5.4.2.2 | TDD | Add minimum requirement tables for RedCap UEs |

Table 3	Specification structure for RedCap UE CSI reporting requirements (Conducted requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 6.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 6.2.1 | 1RX requirements |  |
| 6.2.1.1 | FDD |  |
| 6.2.1.1.1 | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.1.1.2 | CQI reporting under fading conditions for RedCap UEs | New section |
| 6.2.1.2 | TDD |  |
| 6.2.1.2.1 | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.1.2.2 | CQI reporting under fading conditions for RedCap UEs | New section |
| 6.2.2 | 2RX requirements |  |
| 6.2.2.1 | FDD |  |
| 6.2.2.1.[3] | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.2.1.[4] | CQI reporting under fading conditions for RedCap UEs | New section |
| 6.2.2.2 | TDD |  |
| 6.2.2.2.[3] | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 6.2.2.2.[4] | CQI reporting under fading conditions for RedCap UEs | New section |

Table 4	Specification structure for RedCap UE Demodulation performance requirements (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 7.2 | PDSCH demodulation requirements |  |
| 7.2.1 | 1RX requirements |  |
| 7.2.1.2 | TDD |  |
| 7.2.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 7.2.2 | 2RX requirements |  |
| 7.2.2.2 | TDD |  |
| 7.2.2.2.[4] | Minimum requirements for RedCap UEs | New section |
| 7.3 | PDCCH demodulation requirements |  |
| 7.3.1 | 1RX requirements |  |
| 7.3.1.2 | TDD |  |
| 7.3.1.2.1 | Minimum requirements for RedCap UEs | New section |
| 7.3.2 | 2RX requirements |  |
| 7.3.2.2 | TDD |  |
| 7.3.2.2.[4] | Minimum requirements for RedCap UEs | New section |
| 7.4 | PBCH demodulation requirements |  |
| 7.4.1 | 1RX requirements |  |
| 7.4.1.2 | TDD | New section |
| 7.4.2 | 2RX requirements |  |
| 7.4.2.2 | TDD | Add minimum requirement tables for RedCap UEs |

Table 5	Specification structure for RedCap UE CSI reporting requirements (Radiated requirements).

| Section number | Section name | Note |
| --- | --- | --- |
| 8.2 | Reporting of Channel Quality Indicator (CQI) |  |
| 8.2.1 | 1RX requirements |  |
| 8.2.1.2 | TDD |  |
| 8.2.1.2.1 | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 8.2.1.2.2 | CQI reporting under fading conditions for RedCap UEs | New section |
| 8.2.2 | 2RX requirements |  |
| 8.2.2.2 | TDD |  |
| 8.2.2.2.[3] | CQI reporting definition under AWGN conditions for RedCap UEs | New section |
| 8.2.2.2.[4] | CQI reporting under fading conditions for RedCap UEs | New section |

# 6	Summary

Proposal 1: RAN4 defines the UE demodulation and CSI reporting requirements for RedCap UE with:

- Number of receive antennas: 1Rx and 2Rx

- Both FR1 and FR2

- Modulation order: Up to 64QAM

- No requirements for CA, EN-DC, NE-DC, and NR-DC scenarios

Proposal 2: RAN4 defines the RedCap UE demodulation requirements with:

- FR1 HD-FDD: SCS=15kHz and CBW=10MHz

- FR1 TDD: SCS=30kHz and CBW=20MHz

- FR2 TDD: SCS=120kHz and CBW=100MHz

Proposal 3: Define the UE demodulation requirements of PDSCH, SDR, PDCCH, and PBCH with 1Rx/2Rx.

Proposal 4: HD-FDD SCS=15kHz uses the DL/UL pattern with ‘DDDDU’, where ‘D’ denotes a DL slot and ‘U’ denotes a UL slot.

- The number of slots between PDSCH and corresponding HARQ-ACK information for slot index i (i = {0, 1, …, 9}) per frame is:

  - 4 if mod(i, 5) = 0

  - 3 if mod(i, 5) = 1

  - 2 if mod(i, 5) = 2

  - 6 if mod(i, 5) = 3

Proposal 5: For UE demodulation requirements for RedCap with HD-FDD SCS=15kHz, PDCCH/PDSCH are scheduled with the assumption 14 symbols are available in all the DL slots. PUSCH/PUCCH are scheduled with the assumption the start symbol index is 1 for slots just after the DL scheduling, and the last symbol index is 12 for slots just before the DL scheduling.

Proposal 6: Define PDSCH demodulation requirements for RedCap UE with the following test setup.

FR1 HD-FDD

| Test number | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 5.2.2.1.1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x1 low | 70% of peak rate | Test 1-1 |
| 1-2 | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | Test 1-4 |
| 1-3 | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | Test 2-1 |
| 2-1 | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB400-400 | 2x2 ULA low | 70% of peak rate | Test 1-1 |
| 2-2 | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | Test 1-4 |
| 2-3 | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | Test 2-1 |
| Note 1: Use ‘DDDDU’ for DL/UL scheduling. |  |  |  |  |  |  |

FR1 TDD

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 5.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x1 low | 70% of peak rate | Test 1-1 |
| 1-2 | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | Test 1-4 |
| 1-3 | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | Test 2-1 |
| 2-1 | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB400-400 | 2x2 low | 70% of peak rate | Test 1-1 |
| 2-2 | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 low | 70% of peak rate | Test 1-4 |
| 2-3 | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 low | 70% of peak rate | Test 2-1 |

FR2 TDD

| Test number | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 7.2.2.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x1 low | 70% of peak rate | Test 1-1 |
| 1-2 | 100MHz / 120kHz | 16QAM 0.48 Rank 1 | FR2.120-1 | TDLA30-300 | 2x1 low | 70% of peak rate | Test 2-2 |
| 1-3 | 100MHz / 120kHz | 64QAM 0.43 Rank 1 | FR2.120-2 | TDLA30-75 | 2x1 low | 70% of peak rate | Test 2-6 |
| 2-1 (Note 1) | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | Test 1-1 |
| 2-2 (Note 1) | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | Test 2-2 |
| 2-3 (Note 1) | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | Test 2-6 |
| Note 1 Tests 2-1, 2-2, and 2-3 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |  |  |

Proposal 7: Define PDCCH demodulation requirements for RedCap UE with the following test setup.

FR1 HD-FDD SCS=15kHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 5.3.2.1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x1 | 1% | 1Tx Test 1 |
| 1-2 | 10 | 48 | 2 | 4 | TDLA300-100 | 1x1 | 1% | 1Tx Test 2 |
| 1-3 | 10 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 2Tx Test 1 |
| 1-4 | 10 | 48 | 2 | 16 | TDLC300-100 | 1x1 | 1% | 1Tx Test 5 |
| 2-1 | 10 | 24 | 2 | 2 | TDLA30-10 | 1x2 low | 1% | 1Tx Test 1 |
| 2-2 | 10 | 48 | 2 | 4 | TDLA300-100 | 1x2 low | 1% | 1Tx Test 2 |
| 2-3 | 10 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 2Tx Test 1 |
| 2-4 | 10 | 48 | 2 | 16 | TDLC300-100 | 1x2 low | 1% | 1Tx Test 5 |
| Note 1 Tests 2-1, 2-2, 2-3, and 2-4 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |  |  |  |

FR1 TDD SCS=30kHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 5.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 20 | 48 | 2 | 2 | TDLA30-10 | 1x1 | 1% | 1Tx Test 1 |
| 1-2 | 20 | 48 | 2 | 4 | TDLC300-100 | 1x1 | 1% | 1Tx Test 2 |
| 1-3 | 20 | 48 | 2 | 8 | TDLC300-100 | 2x1 low | 1% | 2Tx Test 1 |
| 1-4 | 20 | 48 | 2 | 16 | TDLC300-100 | 1x1 | 1% | 1Tx Test 3 |
| 2-1 | 20 | 48 | 2 | 2 | TDLA30-10 | 1x2 low | 1% | 1Tx Test 1 |
| 2-2 | 20 | 48 | 2 | 4 | TDLC300-100 | 1x2 low | 1% | 1Tx Test 2 |
| 2-3 | 20 | 48 | 2 | 8 | TDLC300-100 | 2x2 low | 1% | 2Tx Test 1 |
| 2-4 | 20 | 48 | 2 | 16 | TDLC300-100 | 1x2 low | 1% | 1Tx Test 3 |

FR2 TDD SCS=120kHz

| Test number | CBW (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 7.3.2.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100 | 60 | 1 | 2 | TDLA30-75 | 1x1 | 1% | 1Tx Test 1 |
| 1-2 | 100 | 60 | 1 | 4 | TDLA30-300 | 1x1 | 1% | 1Tx Test 2 |
| 1-3 | 100 | 60 | 1 | 8 | TDLA30-75 | 2x1 low | 1% | 2Tx Test 1 |
| 1-4 | 100 | 60 | 2 | 16 | TDLA30-75 | 2x1 low | 1% | 2Tx Test 2 |
| 2-1 | 100 | 60 | 1 | 2 | TDLA30-75 | 1x2 low | 1% | 1Tx Test 1 |
| 2-2 | 100 | 60 | 1 | 4 | TDLA30-300 | 1x2 low | 1% | 1Tx Test 2 |
| 2-3 | 100 | 60 | 1 | 8 | TDLA30-75 | 2x2 low | 1% | 2Tx Test 1 |
| 2-4 | 100 | 60 | 2 | 16 | TDLA30-75 | 2x2 low | 1% | 2Tx Test 2 |
| Note 1 Tests 2-1, 2-2, 2-3, and 2-4 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |  |  |  |

Proposal 8: Define PBCH demodulation requirements with the following test setup by assuming SS/PBCH block is not known.

| Test number | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | Reference from TS38.101-4 |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Table 5.4.2.1-2 |
| 1-2 | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Table 5.4.2.2-2 |
| 1-3 | 100MHz / 120kHz | TDLA30-300 | 1x1 | 1% | Table 7.4.2.2-2 |
| 2-1 | 10MHz / 15kHz | TDLC300-100 | 1x2 low | 1% | Table 5.4.2.1-2 |
| 2-2 | 20MHz / 30kHz | TDLA30-10 | 1x2 low | 1% | Table 5.4.2.2-2 |
| 2-3 | 100MHz / 120kHz | TDLA30-300 | 1x2 low | 1% | Table 7.4.2.2-2 |
| Note 1 Tests 2-1, 2-2, and 2-3 could be the same requirements as the existing TS38.101. RAN4 may only need to add applicability rule for RedCap UE with 2Rx. |  |  |  |  |  |

Proposal 9: Define SDR test for RedCap UE assuming both 1Rx and 2Rx.

Proposal 10: Define the CQI reporting definition test for RedCap 1Rx/2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

Proposal 11: Define the wideband CQI reporting under fading condition for RedCap 1Rx/2Rx UE with CQI table 1 (64QAM) by reusing the existing test setup and metrics.

Proposal 12: CQI reporting tests for RedCap UE with HD-FDD SCS=15kHz uses the following configurations:

- DL/UL pattern with ‘DDDDU’.

- NZP-CSI-RS/ZP-CSI-RS/CSI-IM is configured with (periodicity, offset) = (5, 1) slots.

- Periodic CSI reporting is configured with (periodicity, offset) = (5, 0) slots.

- CQI delay: 8ms

# References

- RP-212802, “Status report for WI: Support of reduced capability NR devices; rapporteur: Ericsson”, RAN1.

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDSCH

### A.1.1	FR1

| Parameter |  | Unit | Value | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD | TDD |
| SCS |  | kHz | 15 | 30 |
| Active DL BWP index |  |  | 1 | 1 |
| PDSCH configuration | Mapping type |  | Type A | Type A |
|  | k0 |  | 0 | 0 |
|  | Starting symbol (S) |  | 2 | 2 |
|  | Length (L) |  | 12 | 12 for DL slots 4 for special slots |
|  | PDSCH aggregation factor |  | 1 | 1 |
|  | PRB bundling type |  | Static | Static |
|  | PRB bundling size |  | 2 | 2 |
|  | Resource allocation type |  | Type 0 | Type 0 |
|  | RBG size |  | Config2 | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 4 | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

### A.1.2	FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| SCS |  | kHz | 120 |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 for DL slots 9 for special slots |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

## A.2	NPDCCH

### A.2.1	FR1

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |
| REG bundle size |  | 6 |  |
| Shift index |  | 0 |  |
| TDD UL-DL pattern (applicable only for TDD) |  | FR1.30.1 |  |

Also refers to TS38.101-4 Table 5.3-1.

### A.2.2	FR2

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR2.120-1 |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1 6 for test 2 | 2 |
| Interleaver size |  | 3 for test 1 2 for test 2 | 3 |
| Shift index |  | 0 |  |

Also refer to TS38.101-4 Table 7.3-1

## A.3	PBCH

### A.3.1	FR1

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern (Only applicable for TDD) |  | FR1.30-1 |
| Note 1: as specified in clause 4.1 of TS 38.213 [11] |  |  |

### A.3.2	FR2

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| TDD UL-DL pattern |  | FR2.120-1 |
| Note 1: as specified in clause 4.1 of TS 38.213 [11] Note 2: as specified in clause 11.1 of TS 38.213 [11] |  |  |

## A.4	CQI reporting in static condition

### A.4.1	Test metric

- The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

- If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

### A.4.2	FR1

| Parameter |  |  | Unit | Value | Value |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 20 |
| Duplex Mode |  |  |  | FDD | TDD |
| Subcarrier spacing |  |  | kHz | 15 | 30 |
| SNR |  |  | dB | TBD | TBD |
| Propagation channel |  |  |  | AWGN | AWGN |
| Antenna configuration |  |  |  | 2×1 and 2x2 with static channel specified in Annex B.1 | 2×1 and 2x2 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 2 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3,(6) | Row 3,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (4, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| ReportConfigType |  |  |  | Periodic | Periodic |
| CQI-table |  |  |  | Table 1 | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | 8 |
| Csi-ReportingBand |  |  |  | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | 000001 |
|  |  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  |  | PUCCH | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 |
| Maximum number of HARQ transmission |  |  |  | 1 | 1 |
| Measurement channel |  |  |  | TBD | TBD |
| TDD Slot Configuration |  |  |  | N/A | FR1.30-1 |

### A.4.3	FR2

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |
| Subcarrier spacing |  |  | kHz | 120 |
| Duplex Mode |  |  |  | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |
| SNRBB |  |  | dB | TBD |
| Propagation channel |  |  |  | AWGN |
| Antenna configuration |  |  |  | 2×1and 2x2 with static channel specified in Annex B.1 |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | fd-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 8/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 8/1 |
| ReportConfigType |  |  |  | Periodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | 8/3 |
| aperiodicTriggeringOffset |  |  |  | Not configured |
| Codebook configuration |  | Codebook Type | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUCCH |
| CQI/RI/PMI delay |  |  | ms | 8.375 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| Measurement channel |  |  |  | TBD |

## A.5	CQI reporting in fading condition

### A.5.1	Test metric

- A CQI index not in the set {median CQI -1, median CQI, median CQI +1} shall be reported at least α% of the time.

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ γ.

- When transmitting the transport format indicated by each reported wideband CQI index, the average BLER for the indicated transport formats shall be greater than or equal to [0.02].

### A.5.2	FR1

| Parameter |  | Unit | Value | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 | 20 |
| Subcarrier spacing |  | kHz | 15 | 30 |
| Duplex Mode |  |  | FDD | TDD |
| SNR |  | dB | TBD | TBD |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 |
| Antenna configuration |  |  | 2×1 and 2x2 | 2×1 and 2x2 |
| Correlation configuration |  |  | ULA high | ULA high |
| Beamforming Model |  |  | As specified in Annex B.4.1 | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3,(6) | Row 3,(6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) | (4, 9) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 |
| ReportConfigType |  |  | Periodic | Periodic |
| CQI-table |  |  | Table 1 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 8 |
| Csi-ReportingBand |  |  | 1111111 | 1111111 |
| CSI-Report periodicity and offset |  | slot | 5/0 | 10/9 |
| aperiodicTriggeringOffset |  |  | Not configured | Not configured |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  | CodebookSubsetRestriction |  | 000001 | 000001 |
|  | RI Restriction |  | N/A | N/A |
| Physical channel for CSI report |  |  | PUCCH | PUCCH |
| CQI/RI/PMI delay |  | ms | 8 | 9.5 |
| Maximum number of HARQ transmission |  |  | 1 | 1 |
| Measurement channel |  |  | TBD | TBD |
| TDD Slot Configuration |  |  | N/A | FR1.30-1 |

### A.5.3	FR2

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |
| Subcarrier spacing |  |  | kHz | 120 |
| Duplex Mode |  |  |  | TDD |
| TDD Slot Configuration |  |  |  | FR2.120-2 Annex A.1.3 |
| SNRBB |  |  | dB | TBD |
| Propagation channel |  |  |  | TDLA30-35 |
| Antenna configuration |  |  |  | 2×1 and 2x2 ULA High |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 8 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | CSI-RS periodicity and offset |  | slot | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | fd-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | Not configured |
|  | aperiodicTriggeringOffset |  |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Aperiodic |
|  | CSI-IM RE pattern |  |  | 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (8, 13) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | Not configured |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 1 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 111111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 6 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 1.375 |
| Maximum number of HARQ transmission |  |  |  | 1 |
| Measurement channel |  |  |  | TBD |