3GPP TSG-RAN WG4 Meeting#101-bis-e                              R4-2201447

E-meeting,17th Jan– 25th Feb, 2022

Agenda item:	5.3.2

Source: 	ZTE Corporation

Title: 	Discussion on system parameters for 6425-7125MHz

Document for:	Approval

Introduction

In RAN#94e meeting, the WID on introduction of 6GHz NR licensed bands was kicked off based on the RCC LS. In the contribution, we want to share some initial analysis from system parameters perspectives based on the discussion in SI phase.

Discussion

## 2.1 Band number

Since this frequency range 6425-7125MHz is newly introduced in 3GPP, a new band number should be defined correspondingly for this frequency range. Based on the legacy Rel-15 principle “first come first served” and ongoing the discussions in other spectrum related WIDs, it’s proposed to use band number n103 for 6425-7125MHz.

Proposal 1: to use band number n103 for 6425-7125MHz.

## 2.2 CBW/SCS

Based on the discussion in SI phase for ACLR requirement definition and UE SEM requirement, we propose to define the CBW 20,25,30,40,50, 60,70,80,90 and it’s FFS for 35MHz and 45MHz.

Proposal 2: to use the following CBW/SCS configurations for 6425-7125MHz.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n103 | 15 |  |  |  | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 20 | 25 | 30 | 40 | 50 | 60 | 704 | 80 | 904 | 100 |
|  | 60 |  |  |  | 20 | 25 | 40 | 40 | 50 | 60 | 704 | 80 | 904 | 100 |

## 2.3 Spectral utilization

Regarding the spectrum utilization, indeed during coexistence study in SI phase, the existing spectral utilization of 100MHz/30KHz has already been reused. In addition, based on the outcome of ACLR and ACS requirement, BS OBUE and UE SEM in TR 38.921, it seems reasonable to reuse the existing spectral utilization for 6425-7125MHz.

Proposal 3: to reuse the existing FR1 spectral utilization for 6425-7125MHz.

## 2.4 Chanel raster, sync raster

Regarding the channel raster for 6425-7125MHz, we propose to use the same approach as n79, therefore channel raster 15KHz and 30KHz (step size is equal to 2) could be both supported.

Proposal 4: to define channel raster 15KHz and 30KHz (step size is equal to 2) for 6425-7125MHz.

Regarding sync raster for 6425-7125MHz, based on the maximum sync raster design principle agreed in Rel-15 shown as following equation:

Maximum sync raster<=BWconfig+channel raster-BWSSB

For 6425-7125MHz with minimum channel bandwidth as 20MHz, then

maximum sync raster<=51*360KHz+30KHz-20*360KHz=11190KHz=7.7708*1.44MHz>4*1.44MHz.

Based on the above analysis, the existing step size 4 of n79 could also been applied for 6425-7125MHz.

Proposal 5: to reuse the step size 4 for 6425-7125MHz;

Conclusions

In this contribution, we want to initial analysis from system parameters perspectives based on the discussion in SI phase and made the following proposals:

Proposal 1: to use band number n103 for 6425-7125MHz.

Proposal 2: to use the following CBW/SCS configurations for 6425-7125MHz.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n103 | 15 |  |  |  | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |
|  | 30 |  |  |  | 20 | 25 | 30 | 40 | 50 | 60 | 704 | 80 | 904 | 100 |
|  | 60 |  |  |  | 20 | 25 | 30 | 40 | 50 | 60 | 704 | 80 | 904 | 100 |

Proposal 3: to reuse the existing FR1 spectral utilization for 6425-7125MHz.

Proposal 4: to define channel raster 15KHz and 30KHz (step size is equal to 2) for 6425-7125MHz.

Proposal 5: to reuse the step size 4 for 6425-7125MHz.

References

[1] RP-213696, “Revised WID: Introduction of 6GHz NR licensed bands”, Huawei, HiSilicon

[2] RP-213694, “	Way for [94e-57-R17-6GHz-LS]”, Apple

[3] R4-2103104, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)”, TSG RAN WG4

[4] R4-2016905, WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz, Nokia, CATT, Ericsson, ZTE, Huawei, Qualcomm