3GPP TSG-RAN WG4 Meeting#101-bis-e                              R4-2201448

E-meeting,17th Jan– 25th Feb, 2022

Agenda item:	5.3.3

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirements for 6425-7125MHz

Document for:	Approval

Introduction

In RAN#94e meeting, the WID on introduction of 6GHz NR licensed bands was kicked off based on the RCC LS. In the contribution, we want to share some initial analysis from UE RF requirements perspectives based on the discussion in SI phase.

Discussion

## 2.1 Tx requirements

### 2.1.1 UE Transmitter power

#### 2.1.1.1 UE maximum output power

In the incoming LS from RCC regulation, user equipment should not exceed the limits established by Article 21of the Radio Regulations for the radio frequency band 6425-7125. To be more specifically according to

the article 21 of the RR as following, UE maximum output power should not exceed 43dBm. In addition, based on the discussion during the SI phase, UE PC3 23dBm was used for co-existence study, therefore we think that UE PC3 could be used as starting point, for other UE power class, it depends on operator and market demand under the compliance with regulatory requirement.

The power delivered by a transmitter to the antenna of a station in the fixed or mobile services shall not exceed +13 dBW in frequency bands between 1 GHz and 10 GHz, or +10 dBW in frequency bands above 10 GHz.

Proposal 1: to use UE PC3 as starting point for 6425-7125MHz.

#### 2.1.1.2 UE MPR or A-MPR

As ACLR requirement and UE SEM requirement was agreed to be relaxed based on simulation results of coexistence study. To be specifically, for ACLR requirement was agreed as 26dBc for 6425-7125MHz which is 4dB less than that of PC3 in other licensed band and SEM requirement is also relaxed general NR FR1 spectrum at the FOOB edge ± 0 – 5 MHz by 3 dB.

For 6425-7125MHz, a new band should be defined, and there are new ACLR and SEM requirements for this band. For MPR requirement, it was simulated based on the ACLR and SEM requirements, where the ACLR was used for the PA operating point calibration and SEM requirements was used as the criteria for the unwanted emission. When the ACLR and SEM are modified, the MPR would be re-evaluated. However, for 6425-7125MHz, both ACLR and SEM are relaxed compared with the other PC3 bands.

Actually, MPR was used for a UE to meet the unwanted emission when UE PA operates at a high operating point due to PA non-linear characteristic. For the same transmission power, relaxation of ACLR and SEM means less MPR would be needed. Meanwhile, considering the current MPR requirements are defined as band agnostic, so we think the current MPR requirements can also apply to 6425-7125MHz to meet the more relaxed ACLR and SEM requirements.

From the above perspective, MPR requirement requirement might be necessary to be revisited. Regarding A-MPR requirement, it would depend on the regulatory requirement.

Proposal 2: Current MPR requirement for NR PC3 can apply to 6425-7125MHz PC3.

#### 2.1.1.3 Configured transmitted power

Since configured transmit power is a bit generic defined in the 38.101-1 spec, therefore the existing requirement could be also reused for 6425-7125MHz.

Proposal 2: the existing requirement for configured transmitted power could be also reused for 6425-7125MHz.

### 2.1.2 Output power dynamics

#### 2.1.2.1 Minimum output power

The existing minimum output power could be reused, we don’t see the reasoning why this could be revisited again. In addition, it should be noted that -33dBm minimum output power for 100MHz, 30KHz has already been during the coexistence study.

Proposal 3: the existing requirement for minimum output power could be reused for 6425-7125MHz;

#### 2.1.2.2 Transmit OFF power

Similar as minimum output power, we also don’t see the reasoning why the transmit OFF power could be revisited again.

Proposal 4: the existing requirement for Transmit OFF power could be reused for 6425-7125MHz;

2.1.2.3 Transmit ON/OFF time mask

Similar as minimum output power, we also don’t see the reasoning why the transmit ON/OFF time mask could be revisited again.

Proposal 5: the existing requirement for Transmit ON/OFF time mask could be reused for 6425-7125MHz;

### 2.1.3 Transmit signal quality

Similar as before, the transmit signal quality is defined in band agnostic way, therefore the existing requirement could be also reused for 6425-7125MHz.

Proposal 6: the existing requirement for transmit signal quality could be reused for 6425-7125MHz;

### 2.1.4 Output RF spectrum emissions

2.1.4.1 OBW

For OBW requirement, the existing requirement to follow ITU regulation could be reused for 6425-7125MHz.

Proposal 7: the existing OBW requirement could be reused for 6425-7125MHz;

2.1.4.2 SEM

Regarding the SEM requirement, we propose to follow the existing requirement defined in TR 38.921 for 6425-7125MHz.

Proposal 8: to use the existing SEM requirement defined in TR 38.921 for 6425-7125MHz;

2.1.4.3 ACLR

Similar as the SEM requirement, we propose to follow the existing ACLR requirement defined in TR 38.921 for 6425-7125MHz.

Proposal 9: to use the existing ACLR requirement defined in TR 38.921 for 6425-7125MHz;

2.1.4.4 Spurious emissions

Similar as the SEM and ACLR requirement, we propose to follow the existing spurious emission requirement defined in TR 38.921 for 6425-7125MHz.

Proposal 10: to use the existing spurious emission requirement defined in TR 38.921 for 6425-7125MHz;

In addition, similar as other high band as n77, n78, n79 and n41, UL MIMO should also been supported for 6425-7125MHz.

Proposal 11: UL MIMO should also been supported for 6425-7125MHz;

## 2.2 Rx requirements

### 2.2.1	Reference sensitivity

Regarding REFSENS for 6425-7125MHz, this is highly dependent on the UE noise figure. In Rel-17 SI phase, it was agreed to use 9dB as baseline and 13dB as optional assumption for coexistence study. If noise figure 10dB is agreeable, then UE REFSENS for n78 or n79 could be reused for 6425-7125MHz except for channel bandwidth not supported for 6425-7125MHz.

Observation 2: If noise figure 10dB is agreeable, then UE REFSENS for n78 or n79 could be reused for 6425-7125MHz except for channel bandwidth not supported for 6425-7125MHz

### 2.2.2	Maximum input level

Since maximum input power is defined in band agnostic way, therefore the existing requirement for maximum input power could be reused for 6425-7125MHz.

Proposal 12: the existing maximum input power could also been reused for 6425-7125MHz.

### 2.2.3	Adjacent channel selectivity

For ACS requirement, similar as transmitter SEM and ACLR requirement, it’s proposed to use the existing requirement in TR 38.921 for 6425-7125MHz.

Proposal 13: to use the existing ACS requirement in TR 38.921 for 6425-7125MHz.-

### 2.2.4	Blocking characteristics

The blocking characteristic specified in clause 7.6 of TS 38.101-1 [4] for frequency larger than 3300 MHz could be applied for 6425 – 7125 MHz.

Proposal 14: to use the existing blocking requirement above 3300MHz in TS 38.101-1 for 6425-7125MHz.

### 2.2.5	Spurious response

Similar as blocking requirement, the spurious response requirement for frequency higher than 3300MHz could be applied for 6425-7125MHz.

Proposal 15: to use the existing spurious response requirement above 3300MHz in TS 38.101-1 for 6425-7125MHz.

### 2.2.6	Intermodulation characteristics

Similar as blocking requirement and spurious response requirement, the existing intermodulation requirements for frequency higher than 3300MHz could be applied for 6425-7125MHz.

Proposal 16: to use the existing intermodulation requirement above 3300MHz in TS 38.101-1 for 6425-7125MHz.

### 2.2.7	Spurious emissions

Since the spurious emission requirement of UE receiver is also defined in band agnostic way, therefore we think that the existing requirement could also been applied for 6425-7125MHz.

Proposal 17: to reuse the existing spurious emission requirements for 6425-7125MHz.

Conclusions

In this contribution, we want to share some initial views on UE RF requirements for 6425-7125MHz based on Rel-17 SI and proposals/observations are made as following:

Proposal 1: to use UE PC3 as starting point for 6425-7125MHz.

Observation 1: MPR requirement for NR PC3 in 6425-7125MHz might be necessary to be revisited

Proposal 2: the existing requirement for configured transmitted power could be also reused for 6425-7125MHz.

Proposal 3: the existing requirement for minimum output power could be reused for 6425-7125MHz;

Proposal 4: the existing requirement for Transmit OFF power could be reused for 6425-7125MHz;

Proposal 5: the existing requirement for Transmit ON/OFF time mask could be reused for 6425-7125MHz;

Proposal 6: the existing requirement for transmit signal quality could be reused for 6425-7125MHz;

Proposal 7: the existing OBW requirement could be reused for 6425-7125MHz;

Proposal 8: to use the existing SEM requirement defined in TR 38.921 for 6425-7125MHz;

Proposal 9: to use the existing ACLR requirement defined in TR 38.921 for 6425-7125MHz;

Proposal 10: to use the existing spurious emission requirement defined in TR 38.921 for 6425-7125MHz;

Proposal 11: UL MIMO should also been supported for 6425-7125MHz;

Observation 2: If noise figure 10dB is agreeable, then UE REFSENS for n78 or n79 could be reused for 6425-7125MHz except for channel bandwidth not supported for 6425-7125MHz

Proposal 13: the existing maximum input power could also been reused for 6425-7125MHz.

Proposal 14: to use the existing ACS requirement in TR 38.921 for 6425-7125MHz.

Proposal 15: to use the existing blocking requirement above 3300MHz in TS 38.101-1 for 6425-7125MHz.

Proposal 16: to use the existing spurious response requirement above 3300MHz in TS 38.101-1 for 6425-7125MHz.

Proposal 17: to use the existing intermodulation requirement above 3300MHz in TS 38.101-1 for 6425-7125MHz.

Proposal 18: to reuse the existing spurious emission requirements for 6425-7125MHz.

References

- TR 38.921

- RP-213696, “Revised WID: Introduction of 6GHz NR licensed bands”, Huawei, HiSilicon

- RP-213694, “	Way for [94e-57-R17-6GHz-LS]”, Apple

- RP-213605