3GPP TSG-RAN WG4 Meeting#101-bis-e                               R4-2201449

E-meeting,17th Jan– 25th Feb, 2022

Agenda item:	5.3.4

Source: 	ZTE Corporation

Title: 	Discussion on BS RF requirements for 6425-7125MHz

Document for:	Approval

Introduction

In RAN#94e meeting, the WID on introduction of 6GHz NR licensed bands was kicked off based on the RCC LS. In the contribution, we want to share some initial analysis from BS RF requirements perspectives based on the discussion in SI phase.

Discussion

As indicated in the following Table, we share some initial analysis on the BS RF requirements due to the introduction of 6425-7125MHz.

Table 1. Summary of related RF requirement

| Tx part |  |
| --- | --- |
| Base station output power | All of these requirements are defined as band or channel bandwidth agnostic except for output power dynamics, therefore it’s not expected to have any impacts with the introduction of licensed band 6425-7125MHz. |
| Output power dynamics |  |
| Transmit ON/OFF power |  |
| Transmitted signal quality |  |
| OBW |  |
| Tx intermodulation |  |
| ACLR | The requirement in TR 38.921 could be specified for licensed band 6425-7215MHz. |
| Operating band unwanted emissions | For Wide BS OBUE requirements, the existing requirement in TR 38.921 could be applied for licensed band 6425-7125MHz. For Medium range and Local area BS OBUE requirement, its detailed analysis could be found in the companion contribution [6]. For Fobue requirement, the requirement ΔfOBUE = 100 MHz in TR 38.921 could be applied for licensed band 6425-7125MHz. |
| Transmitter spurious emissions | The requirement in Table 6.1.4.1 of TR 38.921 could be applied for licensed band 6425-7125MHz. |
| Rx part |  |
| REFSENS | As documented in TR 38.921, for 6.425 - 7.125 GHz, the typical Noise Figure for a Wide Area BS operating at 7 GHz was agreed to be 6 dB (11 dB for Medium Range BS and 14 dB for Local Area BS) which is 1dB higher than other FR1 licensed band. In addition, as analyzed in the companion contribution for system parameters of licensed band 6425-7125MHz, the same spectral utilization could be reused for 6425-7125MHz, therefore the existing FRC of FR1 licensed band could be also reused for 6425-7125MHz. Based on the above background, the existing REFSESNS requirement could be relaxed 1dB to be defined for licensed band 6425-7125MHz. |
| Dynamic range requirement | It should follow the similar approach as FR1 REFSENS, 1dB relaxation is needed due to the increasing noise figure for 6425-7125MHz. |
| ACS, | According to the simulation results in TR 38.921, it is agreed to specify 42 dB ACS for 6.425 - 7.125 GHz. Frequency offset of interfering signal could be reused for 6425-7125MHz |
| Blocking requirement | For blocking requirement, as documented in TR 38.921, the existing requirement could be reused for 6425-7125MHz. |
| OOBB | To follow TR 38.921 agreement to define ΔfOOB = 100 MHz |
| RX IMD | The narrow band blocking or intermodulation requirement is not needed since it’s not expected to have such narrow band blocking in that frequency range. For general intermodulation, the existing requirement in TS 38.104 could also been reused for 6425-725MHz. , |
| Rx spurious emission | The existing receiver spurious emission requirement could be reused for 6425-7125MHz |
| ICS | For ICS requirement, it should be similar as REFSENS and ICS requirements with 1dB relaxation compared with other FR1 licensed band. |

Conclusions

In this contribution, we want to share some initial analysis from BS RF requirements perspectives based on the discussion in SI phase and proposals are made in the above Table 1.

References

- TR 38.921

- RP-213696, “Revised WID: Introduction of 6GHz NR licensed bands”, Huawei, HiSilicon

- RP-213694, “	Way for [94e-57-R17-6GHz-LS]”, Apple

- RP-213605

- R4-22xxx, Discussion on system parameters for 6425-7125MHz.

- R4-22xxx, Discussion on MR and LA OBUE requirements for 6425-7125MHz and 10-10.5GHz.

- R4-22xxxx, draft CR for the introduction of licensed band 6425-7125MHz