3GPP TSG-RAN WG4 Meeting#101-bis-e                               R4-2201455

E-meeting,17th Jan– 25th Feb, 2022

Agenda item:	6.16.5

Source: 	ZTE Corporation

Title: 	Coexistence simulation results for 52.6-71GHz

Document for:	Discussions

Introduction

In the previous RAN4 meetings, there were some discussions how to define ACLR/ACS requirement for 52.6-71GHz and the simulation assumption was approved after extensive offline discussions[1], therefore based on the latest simulation assumption, some updated simulation results for indoor scenario are provided in the this contribution for further discussions.

Discussion

## 2.1. Indoor scenario

### 2.1.1.DL

The DL coexistence simulation results for indoor case are summarized in the following Table 1 and the required DL ACIR requirement to satisfy 5% throughput loss are summarized in Table 2.

Table 1. simulation results for downlink in indoor scenario

| Case | ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60GHz, 100MHz | Average throughput loss | 14.30 | 8.39 | 4.51 | 2.18 | 0.86 | 0.24 | 0.05 | 0.01 |
|  | Cell edge through loss | 49.91 | 31.53 | 15.68 | 5.60 | 0.87 | 0.23 | 0.15 | 0.09 |
| 70GHz, 400MHz | Average throughput loss | 14.36 | 8.48 | 4.59 | 2.23 | 0.88 | 0.24 | 0.05 | 0.01 |
|  | Cell edge through loss | 50.66 | 32.28 | 16.01 | 5.31 | 1.05 | 0.07 | 0.00 | 0.00 |

Table 2. summary of required ACIR requirements for indoor DL case

| Case | Metric | Required ACIR |
| --- | --- | --- |
| 60GHz 100MHz | Average throughput loss | 13.87 |
|  | Cell edge through loss | 23.83 |
| 70GHz 400MHz | Average throughput loss | 14.06 |
|  | Cell edge through loss | 19.98 |

### 2.1.2 UL

The UL coexistence simulation results for indoor case are summarized in the following Table 3 and the required UL ACIR requirement to satisfy 5% throughput loss are summarized in Table 4.

Table 3. simulation results for uplink in indoor scenario

| Case | ACIR [dB] | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60GHz, 100MHz | Average throughput loss | 7.28 | 3.79 | 1.73 | 0.69 | 0.25 | 0.08 | 0.03 | 0.01 |
|  | Cell edge through loss | 45.03 | 24.70 | 10.44 | 3.62 | 0.39 | 0.07 | 0.02 | 0.02 |
| 70GHz, 400MHz | Average throughput loss | 6.39 | 3.28 | 1.48 | 0.59 | 0.21 | 0.07 | 0.02 | 0.01 |
|  | Cell edge through loss | 36.74 | 18.43 | 6.68 | 1.69 | 0.22 | 0.16 | 0.01 | 0.00 |

Table 4. summary of required ACIR requirements for indoor UL case

| Case | Metric | Required ACIR |
| --- | --- | --- |
| 60GHz 100MHz | Average throughput loss | 8.10 |
|  | Cell edge through loss | 22.57 |
| 70GHz 400MHz | Average throughput loss | 8.12 |
|  | Cell edge through loss | 16.68 |

Note: more results could be provided for 60GHz, 400MHz and 70GHz, 100MHz.

## 2.2. Comparison between updated results and TR 38.803

Based on the simulation results in section 2.2 for indoor scenario and data from 38.803, it seems that more stringent requirement are needed for 52.6-71GHz due to different antenna configuration and lower output power limit.

Table 5.5-9S: Interpolated ACIR values for DL to meet the 5% throughput loss criteria at 70GHz

| Scenario |  | Indoor |  | Dense urban |  |
| --- | --- | --- | --- | --- | --- |
| NF [dB] |  | 13 | 15 | 13 | 15 |
| ZTE | Average | 9.25 | 9.10 | 5.00 | 5.00 |
|  | 5%-tile | 19.21 | 18.13 | NA | NA |

Table 5.5-3: Interpolated ACIR values for UL to meet the 5% throughput loss criteria at 70GHz

| Scenario |  | Indoor |  | Dense urban |  |
| --- | --- | --- | --- | --- | --- |
| NF [dB] |  | 13 | 15 | 13 | 15 |
| ZTE | Average | 5.28 | 5.52 | 5.00 | 5.00 |
|  | 5%-tile | 18.15 | 17.77 | NA | NA |

In addition, DL ACIR requirement would be 18.7dB according to the following BS ACLR and ACS value at 70GHz, however it seems that 3.8 dB tighten requirement are needed for 60GHz and 1.2 dB tighten requirement compared with required ACIR value agreed in TR38.803 are needed for 70GHz.

Table 5.5-10: UE ACS

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| UE ACS value [dB] | 22.5 | 21.5 | 20.5 |

Table 5.5-11: BS ACLR

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| BS ACLR value [dB] | 27.5 | 25.5 | 23.5 |

Observation 1: 3.8 dB tighten requirement are needed for 60GHz and 1.2 dB tighten requirement compared with required DL ACIR value agreed in TR38.803 are needed for 70GHz.

In addition, UL ACIR requirement would be 14.1dB according to the following UE ACLR and ACS value at 70GHz, however it seems that 8.4 dB tighten requirement are needed for 60GHz and 2.56dB tighten requirement compared with required UL ACIR value agreed in TR38.803 are needed for 70GHz.

Table 5.5-5: UE ACLR

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| UE ACLR value [dB] | 17 | 16 | 15 |

Table 5.5-6: BS ACS

|  | 30GHz | 45GHz | 70GHz |
| --- | --- | --- | --- |
| BS ACS value [dB] | 23.5 | 22.5 | 21.5 |

Observation 2:8.4 dB tighten requirement are needed for 60GHz and 2.56dB tighten requirement compared with required UL ACIR value agreed in TR38.803 are needed for 70GHz.

Based on summarized results in [1] and technology capabilities captured in TR 38.808 and TR38.803, we are fine to follow the existing requirements in TR 38.803 for 52.6-71GHz;

Conclusions

In this contribution, some initial simulation results for indoor scenario are provided in the this contribution and observation are made as following:

Observation 1: 3.8 dB tighten requirement are needed for 60GHz and 1.2 dB tighten requirement compared with required DL ACIR value agreed in TR38.803 are needed for 70GHz;

Observation 2:8.4 dB tighten requirement are needed for 60GHz and 2.56dB tighten requirement compared with required UL ACIR value agreed in TR38.803 are needed for 70GHz.

Proposal 1: to reuse the existing requirement in TR 38.803 for 52.6-71GHz with round up to nearest integer.

References

- R4-2120001	WF on DL/UL ACIR and BS/UE ACLR/ACS for FR2-2, approved.