3GPP TSG-RAN WG4 Meeting#101-bis-e                               R4-2201456

E-meeting,17th Jan– 25th Feb, 2022

Agenda item:	6.16.4.1

Source: 	ZTE Corporation

Title: 	Further Discussion on BS Tx requirements for 52.6-71GHz

Document for:	Approval

Introduction

In the last RAN4 meeting, there were lots of progress for BS Tx requirements for 52.6-71GHz [2]. In this contribution, we want to share some further considerations on the remaining open issues for BS TX requirements.

Discussion

## 2.1.TAE requirements for MIMO, CA case

As discussed in 60GHz SI phase, it was recognized that TAE 65ns for MIMO transmission might be not valid anymore especially considering the CP length for 960kHz SCS as shown in Table 1. TAE 65ns is almost close to CP length of 960kHz which would result in serious performance degradation at UE receiver side, we provided some initial simulation results are provided in Figure 1 and Figure 2 to show the impacts of different TAE values on downlink performance. Detailed simulation assumptions are listed in Annex 5.

Table 1. Summary for CP and data length for different SCS

| u | SCS | CP[ns] | data[ns] | Sampling Rate [MHz] |
| --- | --- | --- | --- | --- |
| 2 | 60 | 1171.875 | 16666.6 | 245.76 |
| 3 | 120 | 585.9375 | 8333.3 | 491.52 |
| 4 | 240 | 292.96875 | 4166.6 | 983.04 |
| 5 | 480 | 146.484375 | 2083.3 | 1966.08 |
| 6 | 960 | 73.2421875 | 1041.6 | 3932.16 |

Figure 1. the impacts of TAE on downlink link performance [960kHz]

Figure 2. the impacts of TAE on downlink link performance [480kHz]

Based on the initial simulation results in Figure 1 and Figure 2, it could be easily found that 65ns TAE would cause severe performance degradation especially for 960kHz.

For 960kHz SCS, the acceptable TAE requirement should be around 10-20ns for 960kHz and 10-40ns for 480kHz,

Based on the discussion in last RAN4 meeting, we made the following proposal；

Proposal 1: propose MIMO TAE requirement as 16.25ns for 960kHz and 32.5ns for 480kHz;

Regarding the band combinations within FR2-2 and among FR2-1 and FR2-2, it has been postponed to Rel-18 timeframe [8], therefore we think that it is not necessary to define it in Rel-18. In addition, for band combination for FR1 and FR2-2, since it’s different BS equipment deployed at the end, therefore CA related TAE requirement is also not necessary. Based on the above considerations, we made the following proposals:

Proposal 2: postpone CA TAE requirement to Rel-18 timeframe;

Agreement:

- Postpone consideration of the following CA/DC band combinations to the Rel-18 timeframe:

  - Band combinations within FR2-2

  - Band combinations among FR2-1 and FR2-2

- Limit Rel-17 consideration of FR2-2 inter-band scenarios to two bands, before studying higher order configurations, i.e., one FR1 band + one FR2-2 band.

  - Also subject to availability of the regulatory framework for licensed FR2-2 band

## 2.2. Unwanted emission requirements

In the last RAN4 meeting, there were some discussions to further evaluate coexistence performance for 52.6-71GHz or coexistence work in TR 38.803 can be used as reference. As discussed in the companion contribution [7], based on summarized results in [9] and technology capabilities captured in TR 38.808 and TR38.803, we are fine to follow the existing requirements in TR 38.803 for 52.6-71GHz with ACLR and ACS requirement round up to nearest integer.

Proposal 3: to reuse the existing requirement in TR 38.803 for 52.6-71GHz with round up to nearest integer.

## 2.3. EVM requirements for 52.6-71GHz

Regarding EVM window length for 52.6-71GHz, since this requirement is mainly related with waveform used implemented to meet the out of band emissions and in-band EVM requirement, based on the proposal 2 and the agreements made in the previous meetings, therefore we think that current [50%] EVM window length could be reused.

Proposal 4: fine to reuse the 50% EVM window length for FR2-2.

Conclusions

In this contribution, we want to further considerations on each BS Tx requirements for 60GHz and some proposals and observations are made as following:

Proposal 1: propose MIMO TAE requirement as 16.25ns for 960kHz and 32.5ns for 480kHz;

Proposal 2: postpone CA TAE requirement to Rel-18 timeframe;

Proposal 3: to reuse the existing requirement in TR 38.803 for 52.6-71GHz with round up to nearest integer.

Proposal 4: fine to reuse the 50% EVM window length for FR2-2.

References

- R4-2106113,WF on BS RF TX requirements for 52.6 – 71 GHz, approved.

- R4-2108638,WF on BS TX RF requirements for 52.6 – 71 GHz, approved.

- R4- 1713558, Link Level Simulation Results for EVM FR2, Ericsson

- R4-2115643,WF on BS Tx RF requirement, approved.

- R4-2115643,WF on BS Tx RF requirement, approved.

- R4-22xxx, Coexistence simulation results for 52.6-71GHz, ZTE

- R4-2120677,WF on BS RF Tx requirements, Nokia, approved.

- R4-2120061	WF on NR extension to 71 GHz (Part 1) - System Parameters, approved.

- R4-2120001	WF on DL/UL ACIR and BS/UE ACLR/ACS for FR2-2, approved.

Annex: simulation assumption

Table A1-1 Link level simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Waveform | CP-OFDM |
| System Bandwidth | 400MHz |
| Subcarrier spacing | 480kHz/960kHz |
| CP type | Normal CP |
| Channel Model | TDL-A 5ns |
| PN model | 3GPP TR38.803 example 2 |
| DMRS Configuration | 2 DMRS symbols per slot at (2,11) |
| PTRS Configuration | (K = 2, L = 1) |
| SLIV | (S=0, L=14) |
| Channel Estimation | Realistic |
| PN Estimation | Realistic |
| PN compensation | CPE & ICI |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | 2 |
| UE antenna Array configuration | 2 |