3GPP TSG-RAN WG4 Meeting#101-bis-e                               R4-2201457

E-meeting,17th Jan– 25th Feb, 2022

Agenda item:	6.16.4.2

Source: 	ZTE Corporation

Title: 	Further discussion on BS Rx requirements for 52.6-71GHz

Document for:	Approval

Introduction

In the previous RAN4 meetings, there were some initial discussions on BS RX requirements for 52.6-71GHz and some high level agreement has been reached in WF [9]. In this contribution, we want to share some further considerations on each Rx requirements.

Discussion

## 2.1. ACS/IBB requirements

In the last RAN4 meeting, there were some discussions to further evaluate coexistence performance for 52.6-71GHz or coexistence work in TR 38.803 can be used as reference. As discussed in the companion contribution [7], based on summarized results in [9] and technology capabilities captured in TR 38.808 and TR38.803, we are fine to follow the existing requirements in TR 38.803 for 52.6-71GHz with ACLR and ACS requirement round up to nearest integer.

Proposal 1: to reuse the existing ACS requirement in TR 38.803 for 52.6-71GHz with round up to nearest integer and frequency offset updates.

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 100, 200, 400,800,1200,1600,2000 | EISREFSENS + 6 dB | EISREFSENS_50M + 27.7 + ΔFR2_REFSENS (Note 1) EISREFSENS_50M + 26.7 + ΔFR2_REFSENS (Note 2) EISREFSENS_50M + 25.7 + ΔFR2_REFSENS |
| NOTE 1: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz NOTE 2: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz NOTE 3: EISREFSENS is given in clause 10.3.3 NOTE 4: Applicable to bands defined within the frequency spectrum range of 52.6 – 71 GHz |  |  |

In addition, for interfering signal of ACS requirement and IBB requirements, it should be updated as 100MHz instead of 50MHz since it was agreed that minimum channel bandwidth for this frequency range is 100MHz already.

Table 10.5.1.3-2: OTA ACS interferer frequency offset for 60GHz

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 100 | 49.3 | 100 MHz DFT-s-OFDM NR signal,120 kHz SCS, 64 RBs |
| 200 | 49.34 |  |
| 400 | 49.3 |  |
| 800 | 49.34 |  |
| 1200 | 49.38 |  |
| 1600 | 49.3 |  |
| 2000 | 49.34 |  |

It’s same story for IBB interfering power level, we are fine with the existing requirement with in-band blocking level as EISREFSENS_50M + 33 + ΔFR2_REFSENS.

Proposal 2: propose to follow the existing FR2 requirement for 52.6-71GHz IBB requirement;

Table 10.5.2.3-1: General OTA blocking requirement for 60GHz

| BS channel bandwidth of the lowest/highest carrier received (MHz) | OTA wanted signal mean power (dBm) | OTA interfering signal mean power (dBm) | OTA interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of OTA interfering signal |
| --- | --- | --- | --- | --- |
| 100, 200, 400,800,1200,1600,2000 | EISREFSENS + 6 dB | EISREFSENS_50M + 33 + ΔFR2_REFSENS | ±150 | 100 MHz DFT-s-OFDM NR signal, 120 kHz SCS, 64 RBs |
| NOTE: EISREFSENS and EISREFSENS_50M are given in clause 10.3.3. |  |  |  |  |

## 2.4. RX ICS requirements

Based on the simulation results as shown in Figure 1 and Figure 2, IoT levels are within [0-3]dB which is aligned with observed values in Rel-15 and documented in the section 10.9.3 of TR 38.803.

- Figure 1. IoT level for 60GHz NR BS in indoor scenario

- Figure 2. IoT level for 70GHz NR BS in indoor scenario

Proposal 3: to reuse 14dBc in-channel selectivity for 52.6-71GHz BS ICS requirement;

Conclusions

In this contribution, further considerations on each BS Rx requirements for 52.6-71GHz and some proposals and observations are made as following:

Proposal 1: to reuse the existing ACS requirement in TR 38.803 for 52.6-71GHz with round up to nearest integer and frequency offset updates;

Proposal 2: propose to follow the existing FR2 requirement for 52.6-71GHz IBB requirement;

Proposal 3: to reuse 14dBc in-channel selectivity for 52.6-71GHz BS ICS requirement;

References

- R4-2106114, WF on BS RF RX requirements for 52.6 – 71 GHz, approved.

- TR 38.808

- R4-1700302,Way forward on ACLR & ACS for WP5D LS, approved.

- R4-2108639,WF on BS RX RF requirements for 52.6 – 71 GHz, approved

- R4-2107915, WF on 60 GHz coexistence, ACLR, and ACS, approved.

- R4-2115644, WF on BS Rx RF requirement, approved.

- R4-2115643,WF on BS Tx RF requirement, approved.

- R4-211xxx, Coexistence simulation results for 52.6-71GHz, ZTE

- R4-2120678,WF on BS RF Rx requirements and CR worksplit. Approved.