3GPP TSG-RAN WG4 Meeting #101-bis-e 	       R4-2201463

# E-meeting, 17th Jan– 25th Feb, 22

Agenda item:	6.5.3.2

Source: 	ZTE Corporation

Title: 	Further discussions on emission related requirements for radiated repeater

Document for:	Approval

Introduction

In the past RAN4 meetings, there were lots of discussion on repeater conducted output power and emission requirements and there were lots of open issues left for further discussions. Therefore in this contribution, we want to share some further considerations on those remaining open issues.

Discussion

- 2.1. OBUE

Agreement - In the DL apply the BS OBUE requirement for repeaters outside the passband - In the UL BS OBUE applies for UL class with no power limit (wide area UL transmission) outside the passband - UL class with power limit (local area UL transmission) has an OBUE requirement outside the passband, further discuss the appropriate requirement rom the following options: - BS OBUE requirement - UE SEM requirement - Something else - Provision for limitations to the requirement due to noise floor to be considered. - For in-band OBUE the conducted discussion has the following captured: - FFS whether inside passband OBUE requirements or other requirements needed for DL and UL for the case with non-full passband transmission - The same applies to the radiated repeater and the decision on radiated will follow the conducted discussion.

- As agreed in the last RAN4 meeting, for local area class with power limitations, UL ACLR should follow the UE ACLR requirement, therefore we think that it should be reasonable to follow UE SEM requirement for local area repeater.

In the UL there is a tentative Agreement For an UL class with no power limit, apply the BS ACLR requirement. For an UL class with a power limit based on one of the UE classes, apply the UE ACLR.

- Proposal 1: to reuse the UE SEM requirements for local area repeater;

Agreement: on Nov.4 GTW FFS whether inside passband OBUE requirements or other requirements needed for DL and UL for the case with non-full passband transmission.

- As agreed in last RAN4 meeting, it was allowed to have some empty carriers within pass-band from other collaborative operators, however sometimes it might be quite difficult to define the collaborative operators from network deployment perspective. Therefore we think that OBUE requirement within pass-band is still necessary, otherwise there are no insurance on how to guarantee the coexistence performance with other operators.

Proposal 2: to define the OBUE requirement with pass-band if there are any empty carriers within it.

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

- Proposal 1: to reuse UE SEM requirements for local area repeater;

Proposal 2: to define the OBUE requirement with pass-band if there are any empty carriers within it.

References

- R4-2120666, WF on repeater conducted requirements, approved.