3GPP TSG-RAN WG4 Meeting #101-bis-e 	       R4-2201464

# E-meeting, 17th Jan– 25th Feb, 22

Agenda item:	6.5.3.3

Source: 	ZTE Corporation

Title: 	Further discussions on other requirements of radiated repeater

Document for:	Approval

Introduction

In the past RAN4 meetings, there were lots of discussion on repeater conducted output power and emission requirements and there were lots of open issues left for further discussions. Therefore in this contribution, we want to share some further considerations on those remaining open issues.

Discussion

- 2.1. NF equivalent requirements

In last RAN4 meeting, there are some discussion on NF related requirements and testing issues, there are several options left for further discussion, therefore in the following section, we want to share some further understandings on this issue.

NF can be covered by the equivalent requirements with below options: o Option 1: Perform EVM conformance test with minimum input power o Option 2: Absolute maximum output power with no input within part of passband e.g. inside passband OBUE o Only one option should be selected in the end from RAN4 core requirements aspect

From our understandings, both option 1 and option 2 is indirectly related with NF, therefore we are fine with both options.

Proposal 1: for NF equivalent requirements, we are fine with both options. Maybe option 1 is more preferred since its NF might be higher with minimum input power.

- 2.3. Input IMD requirements

The BW of the interfering signals is still open issue (currently opinion is split) between the 2 options: o Option 1: For RX IM, set the modulated signal bandwidth to [50] MHz o Option 2: propose to use two CW signals

We are fine with option 1 if generation of two CW signals simultaneously in the chamber is difficult.

Proposal 2: fine with option 1 with modulated signal to 50MHz;

- 2.4. Out of band gain requirements

In last RAN4 meeting, there are some discussion on out of band requirements, there are several options left for further discussion, therefore in the following section, we want to share some initial understandings on this issue.

These include: - In band gain assumption (FR1 assumes 90dB) - Antennas gain (e.g. BS assumes 10 to 32dBi) - BS side - UE side - BS to repeater MCL/PL including both: - Serving BS to repeater - Other (victim) BS to repeater - UE to repeater MCL/PL including both: - Served UE to repeater - Other (victim) UE(s) to repeater

Based on the initial simulation results as shown in Figure 1/2 with its simulation assumptions in TR38.803, if repeater with maximum in-band gain as 80dB, UE at the cell edge with the coverage extension with FR2 repeater, it can have the same experience as UE with best side conditions.

For urban macro scenario, the coupling loss for cell edge UEs could be improved to 70dBc as shown in Figure 1;

Therefore if NR based repeater with in-band gain as 80dBc, it should be sufficient enough to ensure repeater’s coverage. In addition, assuming FR2 AAS BS with 100MHz and 30dBm TRP, then in-band PSD should be 10dBm/MHz and the emission in the 0.1*BWcontiguous should be -5dBm/MHz, therefore the effective ACLR within BWcontiguous should be 15dBc. If out of band gain within 0.1*BWcontiguous is 60dBc similar as FR1, then the interference signal amplified by repeater should be similar or lower than unwanted emissions.

Unwanted emission generated by itself= wanted signal+80dBi gain- 15dBc in 0.1*BWcontiguous;

interference amplified by repeater= interference signal+60dBi out of band gain in 0.1*BWcontiguous;

Where:

- The interference signal power level is the same as wanted signal power level;

Figure 1. Coupling loss for AAS NR Urban Macro at 30GHz [pathloss+BS antenna gain+UE antenna gain]

Figure 2. Coupling loss for AAS NR Indoor at 30GHz [pathloss+BS antenna gain+UE antenna gain]

To added later on

Figure 2. BS UEM mask

Based on the above considerations, we think that in-band gain for FR2 could be 80dBc at most. For out of band gain requirement, it could be further discussed.

Proposal 3: in-band gain for FR2 could be 80dBc at most;

- 2.5. ACRR

In last RAN4 meeting, there are some discussion on out of band gain and ACRR requirement, there are several options left for further discussion, therefore in the following section, we want to share some initial understandings on this issue

The issue around ACLR/ACRR is if they can (or should) be specified and/or tested together. The following points are for further discussion • Discuss further testability of ACLR and ACRR together • Whether specified/tested together or not, consider total interference to adjacent channel when specifying ALCR and ACRR the requirements. For the ACRR specific requirement value monitor the conducted progress and aim to apply same or similar agreements to radiate. Capture the assumptions which differ between FR1 and FR2. The same assumptions as for oob gain for FR2 need to be made to assess FR2 values.

Since ACRR requirement is basically coming from out of band gain requirement and in-band gain requirement, therefore we would like to postpone the discussion until FR2 out of band gain is agreed

Proposal 4: agree on out of band gain requirement together with ACRR due to its relationship in nature.;

Conclusions

In this contribution, we want to share some further considerations on those remaining open issues of conducted repeater and proposals are made as following:

Proposal 1: for NF equivalent requirements, we are fine with both options. Maybe option 1 is more preferred since its NF might be higher with minimum input power.

Proposal 2: fine with option 1 with modulated signal to 50MHz;

Proposal 3: in-band gain for FR2 could be 80dBc at most;

Proposal 4: agree on out of band gain requirement together with ACRR due to its relationship in nature;

References

- R4-2120666, WF on repeater conducted requirements, approved.