TSG-RAN Working Group 4 (Radio) meeting #101bis-e	R4-2201487

Electronic Meeting, 17th – 25th January 2022

Source:	Ericsson

Title:	TP to TR 38.844: Section 6.1.2 Signalling for Overlapping CA Approach

Agenda item:	7.2.3.3.1

Document for:	Approval

# 1	Introduction

Based upon the LS Reply from RAN2 [1] the following is a text proposal to include inputs for completeness.  The relevant and key parts are shown in the excerpt below for convience:

- For the overlapping CBWs from UE perspective (two cell approach / CA approach):

  - if two different Bandwidth Parts for the UE are overlapping, and both contain a subset of CSI-RS resources that are mapped to the same subset of overlapping RBs for the same UE, please clarify how does UE report CSI for the overlapped part, e.g. does UE report CSI for each cell separately, or just once for the overlapping part, or something else?

  - RAN2 response: RAN2 thinks it is not clear whether legacy UEs would support this kind of "overlapping CA" as this was never discussed in RAN2 before and current UE capabilities do not consider any frequency overlap in CA case.

# 2	Text Proposal

## 6.6	Legacy UE impact

#### 6.6.xx	Overlapping CA (two cells)

It is unclear whether legacy UEs would support a CA configuration with overlapping channel bandwidths as this has not been discussed in RAN2 when designing current UE capabilities and therefore do not consider any frequency overlap in the CA case.

# 4	References

[1]	R2-2111597, “Reply LS on specification impact for methods on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths” RAN2