3GPP TSG-RAN WG4#101bis-e	R4-2201523

Electronic meeting, 15th – 25th January 2022

Source: 	Ericsson

Title:  	PUSCH with UL timing adjustment requirements

Agenda Item:	6.9.5.3.2

Document for:	Discussion

# Introduction

During RAN4#100-e, a WF on BS demodulation requirements was agreed in [1]. This contribution provides simulation results and proposal for PUSCH timing adjustment requirements.

# Discussion

The following parameters have been agreed as the baseline configuration for UL timing adjustment [1].

Table 1: Simulation assumption for UL timing adjustment requirement.

| Parameters | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE speed | 350km/h |
| Waveform | CP-OFDM |
| CP length | Normal |
| A | AWGN |
| Δω | 1T2R |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | 1DMRS+PT-RS(L=1,K=2), 2DMRS+PT-RS(L=1,K=2), 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |

The simulation results (without impairment) are presented in the table below.

Table 2: Simulation result for UL TA.

| CBW | SNR @ 70% of Maximum TP |  |  |
| --- | --- | --- | --- |
|  | 1DMRS | 2DMRS | 3DMRS |
| 50MHz | 6.5 dB | 6.3 dB | 6.4 dB |
| 200MHz | 6.5 dB | 6.4 dB | 6.4 dB |

The main conclusion is that the results are very similar for different bandwidths and that performance difference is negligible for configurations with additional DMRS symbols.

# References

Nokia, Nokia Shanghai Bell, Samsung, “WF on BS demodulation requirement for FR2 HST”, RAN4#100-e, November 2021