3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201548

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson, T-Mobile US

Title:	TP for TR 38.717-03-02: CA_n41-n66-n260

Agenda item:			5.11.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n41A-n66A-n260A, CA_n41A-n66A-n260(2A), CA_n41A-n66A-n260G/H/I.

# 2. Text Proposal

# ---Start of changes---

### 5.3.x	CA_n41-n66-n260

#### 5.3.x.1	Operating bands for CA

Table 5.3.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n66-n260 | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

#### 5.3.x.2	Channel bandwidths per operating band for CA

Table 5.3.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Conf | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A-n260A | CA_n41A-n260A CA_n66A-n260A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |  |
|  |  | n260 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |
| CA_n41A-n66A-n260(2A) | CA_n41A-n260A CA_n66A-n260A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n260G | CA_n41A-n260A CA_n41A-n260G CA_n66A-n260A CA_n66A-n260G | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n260H | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n66A-n260I | CA_n41A-n260A CA_n41A-n260G CA_n41A-n260H CA_n41A-n260I CA_n66A-n260A CA_n66A-n260G CA_n66A-n260H CA_n66A-n260I | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.3.x.3	UE co-existence studies

UL n41-n260 does not affect DL n66.

UL n66-n260 does not affect DL n41.

#### 5.3.x.4	REFSENS requirements

Based on the co-existence studies there is no need to define MSD values.

---End of changes---

# Reference

[1] 		RP-212889, “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL”, ZTE