3GPP TSG-RAN WG4 Meeting # 101-bis-e 	R4-2201585

Electronic Meeting, Jan. 17-25, 2022

Source:	Ericsson

Title:	UE Timing requirements

Agenda item:	6.13.5.4

Document for:	Approval

# 1	Introduction

In RAN4#101-e a WF was approved [1]. We analyze the questions in the WF and continue to discuss timing requirements for NTN WI.

# 2	Discussion

In the approved WF, it is stated [1]:

- Issue 2-3-6: The gradual timing adjustment requirement - The maximum amount of the magnitude of the timing change in one adjustment shall be Tq_NTN. - The minimum aggregate adjustment rate shall be Tp_NTN per [X]ms. - FFS whether X can be different for different types of satellites, altitude, etc - The maximum aggregate adjustment rate shall be Tq_NTN per [Y]ms. - FFS whether Y can be different for different types of satellites, altitude, etc Frequency Range; SCS of uplink signals (kHz); Tq_NTN; Tp_NTN. 1; 15; [Z1]*64*Tc; [Z1]*64*Tc. 30; [Z2]*64*Tc; [Z2]*64*Tc. 60; [Z3]*64*Tc; [Z3]*64*Tc. NOTE: Tc is the basic timing unit defined in TS 38.211. - FFS whether Z1-Z3 can be different for different types of satellites, altitude, etc

There is also an LS in from RAN1 regarding the issue of Combination of open and closed loop TA control in NTN [2]:

... However, double TA adjustment may happen when the parameters that affect open loop control (including at least common TA, ephemeris, and GNSS position) are updated. Specifically, a UE may use old open loop TA control parameters which leads to TA error. This TA error is already corrected by closed loop TA control. When the UE updates open loop TA control parameters, it actually corrects this TA error one more time (since this TA error is already corrected by closed loop TA control). ...

## 2.1	Gradual timing adjustment requirement

We need to consider the TA mechanism, as currently discussed in RAN1:

Agreement RAN1#104-bis-e: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

One way to develop gradual timing adjustment requirement is to consider the different terms. The main adaptation to the dynamics of the satellite orbit is the open loop TA control which corresponds to the sum:

NTA,UE-specific+NTA,common

If the gNB RX demodulation performance or the gNB UL timing needs adjustment, the legacy closed loop can be used by the gNB to issue TAC to do further adjustment in addition to the open loop TA control, rather slowly. The closed loop TA control terms corresponds to the sum:

NTA+NTA,offset

The closed loop terms NTA+NTA,offset already have requirements in existing legacy specification, TS 38.133.

#### 7.1.2.1	Gradual timing adjustment

Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE is required to adjust its timing to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1.

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Observation 1: The closed loop terms NTA+NTA,offset already have requirements in existing legacy specification, TS 38.133. 

Proposal 1: Keep existing gradual timing adjustment requirements for the closed loop terms NTA+NTA,offset.

The open loop NTA,UE-specific+NTA,common uncertainty is dominated by the RTT dynamics of the (LEO) orbit and UE GNSS positioning uncertainty. The best we can do is to put limits based on the characteristics of at least the UE GNSS positioning accuracy part.

All adjustments made to the UE uplink timing, for NTA,UE-specific shall follow these rules:

1)	The UE GNSS position accuracy is 50 meters from true position.

2)	The maximum UE movement from GNSS position updates corresponds to a UE speed < 500 km/h.

3)	The maximum amount of deviation from true displacement between UE GNSS position updates < mink1 ⋅Δt,k2 where Δt is time between UE GNSS position updates.

The first condition, (1), originates from the UE GNSS position accuracy requirement. The second condition (2) is due to an upper bound on UE speed. Condition (3) originates from a desire to keep the deviation between actual and estimated displacement between GNSS updates under control. This is shown in the figure below. Where 
∆´-∆ is the actual requirement. The actual values of k1 and k2 are FFS.

Figure 1: Condition 3) The maximum amount of deviation from true displacement between UE GNSS position updates

Observation 2: The best we can do is to put limits based on the characteristics of at least the UE GNSS positioning accuracy part, for NTA,UE-specific.

Proposal 2: 
All adjustments made to the UE uplink timing, for NTA,UE-specific shall follow these rules:

1)	The UE GNSS position accuracy is 50 meters from true position.

2)	The maximum amount UE GNSS position update rate corresponds to a UE speed < 500 km/h.

3)	The maximum amount of deviation from true displacement between UE GNSS position updates < mink1 ⋅Δt,k2, where Δt is time between UE GNSS position updates.

Proposal 3: The values of k1 and k2 are FFS.

# 3	Summary

Observation 1: The closed loop terms NTA+NTA,offset already have requirements in existing legacy specification, TS 38.133. 

Proposal 1: Keep existing gradual timing adjustment requirements for the closed loop terms NTA+NTA,offset.

Observation 2: The best we can do is to put limits based on the characteristics of at least the UE GNSS positioning accuracy part, for NTA,UE-specific.

Proposal 2: 
All adjustments made to the UE uplink timing, for NTA,UE-specific shall follow these rules:

1)	The UE GNSS position accuracy is 50 meters from true position.

2)	The maximum amount UE GNSS position update rate corresponds to a UE speed < 500 km/h.

3)	The maximum amount of deviation from true displacement between UE GNSS position updates < mink1 ⋅Δt,k2, where Δt is time between UE GNSS position updates.

Proposal 3: The values of k1 and k2 are FFS.

# References

- R4-2120310, WF on GNSS-related and timing requirements for NR NTN, Xiaomi

- R1-2110604, LS on Combination of open and closed loop TA control in NTN, THALES