3GPP TSG-RAN WG4 Meeting #101bis-e                                                     R4-2201593

Electronic Meeting, 17-25 Jan., 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS38.101-1: Addition of PC2 non-contiguous ULCA MPR requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Skyworks Solutions Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-Core |  |  |  |  |  |  | Date: |  | 2022-01-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Addition of PC2 2LO MPR based on agreed WF R4-2114948 - Addition of PC2 1LO MPR based on agreed WF R4-2119955 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct the restriction to dualPA-Architecture IE absent and add applicability of uplinkTxDC-TwoCarrierReport-r16 to bandwidth class C 2. Introduction of delta MPR for 2Tx in tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in signaling options and MPR for contiguous ULCA with or without UL MIMO |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | These changes are also related to proposals in R4-2200498 |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

## << Start of change >>

#### 6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPRIM3 defined in Clause 6.2A.2.2.2 for power class 3 and Clause 6.2A.2.2.4 for power class 2

Else If AND( FIM3,low_block,low > SEM-25,low ,  FIM3,high_block,high < SEM-25,high )

MPRIM3 defined in Clause 6.2A.2.2.1 for power class 3 and Clause 6.2A.2.2.3 for power class 2

Else

MPRIM3 defined in Clause 6.2A.2.2.1 for power class 3 and Clause 6.2A.2.2.3 for power class 2

where

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

-	SEM-25,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -25 dBm / MHz to -30 dBm / MHz, as specified in Clause 6.5A.2.2.2.

##### 6.2A.2.2.1	MPRIM3 to meet -30dBm/MHz for power class 3

MPR in this clause is for intra-band non-contiguous CA power class 3

For UEs indicating IE uplinkTxDC-TwoCarrierReport-r16 supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15; 	0 ≤ B < 1.08

14.5; 	1.08 ≤ B < 2.16

13.5; 	2.16 ≤ B < 3.24

12.5;       3.24 ≤ B < 5.04

11.5; 	5.04≤ B < 10.08

10.5; 	10.08 ≤ B < 16.38

10;        16.38 ≤ B < 21.78

9; 	      21.78 ≤ B

For UEs not indicating IE uplinkTxDC-TwoCarrierReport-r16 and not indicating TxD supported and applicable for bandwidth separation classes  ≤ 200 MHz and when the frequency gap between carriers is less than or equal to the aggregated channel bandwidth of the two carriers

MPR=MA Where MA is defined as follows

MA = 	[17.5];		0 ≤ B < 1.08

[17.0]; 	1.08 ≤ B < 2.16

[16.5]; 	2.16 ≤ B < 3.24

[16.0];   	3.24 ≤ B < 5.04

[15.0]; 	5.04≤ B < 10.08

[14.5]; 	10.08 ≤ B < 36

[10.0];       36 ≤ B < 56.88

[9.0]; 	 56.88 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

##### 6.2A.2.2.2	MPRIM3 to meet -13dBm/MHz for power class 3

MPR in this clause is for intra-band non-contiguous CA power class 3

For UEs indicating IE uplinkTxDC-TwoCarrierReport-r16 supported. The allowed maximum output power reduction is defined as:

MPR=MA

Where MA is defined as follows

MA = 	9	;	 0 ≤ B < 0.54

8	;	 0.54 ≤ B < 1.08

7	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

5.5	; 	3.24 ≤ B < 5.4

4	; 	5.4 ≤ B

For UEs not indicating IE uplinkTxDC-TwoCarrierReport-r16 and not indicating TxD supported, and applicable for bandwidth separation classes  ≤ 200 MHz and when the frequency gap between carriers is less than or equal to the aggregated channel bandwidth of the two carriers. The allowed maximum output power reduction is defined as:

MPR=MA Where MA is defined as follows

MA = 	[11.0]	;	0 ≤ B < 1.08

[10.5]	;	1.08 ≤ B < 2.16

[10.0]	; 	2.16 ≤ B < 3.24

[9.5]	; 	3.24 ≤ B < 5.04

[8.5]	; 	5.04 ≤ B < 10.08

[7.5]	; 	10.08 ≤ B < 36

[TBD]	; 	36 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

##### 6.2A.2.2.3	MPRIM3 to meet -30dBm/MHz for power class 2

MPR in this clause is for intra-band non-contiguous CA power class 2.

For UEs indicating IE uplinkTxDC-TwoCarrierReport-r16 supported. The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	15.5; 	0 ≤ B < 1.44

15.0; 	1.44 ≤ B < 2.88

14.0;    2.88 ≤ B < 5.76

12.0; 	5.76 ≤ B < 10.8

10.5; 	10.8 ≤ B < 23.04

9.0; 	 23.04 ≤ B

For UEs not indicating IE uplinkTxDC-TwoCarrierReport-r16 supported and indicating TxD supported or not, and applicable for bandwidth separation classes  ≤ 200 MHz and when the frequency gap between carriers is less than or equal to the aggregated channel bandwidth of the two carriers . The allowed maximum output power reduction is defined as:

MPR=MAWhere MA is defined as follows

MA = 	[19.5]; 	0 ≤ B < 1.08

[19.0]; 	1.08 ≤ B < 2.16

[18.0]; 	2.16 ≤ B < 5.04

[16.5]; 	5.04≤ B < 10.08

[16.0]; 	10.08 ≤ B < 36

[12.0];   	 36 ≤ B < 56.88

[10.5];	 	56.88 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

##### 6.2A.2.2.4	MPRIM3 to meet -13dBm/MHz for power class 2

MPR in this clause is for intra-band non-contiguous CA power class 2.

For UEs indicating IE uplinkTxDC-TwoCarrierReport-r16 supported. The allowed maximum output power reduction is defined as:

MPR=MA Where MA is defined as follows

MA = 	9.0	;	0 ≤ B < 0.54

8.0	;	0.54 ≤ B < 1.08

7.0	; 	1.08 ≤ B < 2.16

6.5	; 	2.16 ≤ B < 3.24

6.0	; 	3.24 ≤ B < 5.4

5.5	; 	5.4 ≤ B < 10.8

4.0	; 	10.8 ≤ B

For UEs not indicating IE uplinkTxDC-TwoCarrierReport-r16 supported and indicating TxD supported or not, and applicable for bandwidth separation classes  ≤ 200 MHz and when the frequency gap between carriers is less than or equal to the aggregated channel bandwidth of the two carriers. The allowed maximum output power reduction is defined as:

MPR=MA Where MA is defined as follows

MA = 	[14.0]	;	0 ≤ B < 1.08

[12.0]	;	1.08 ≤ B < 2.16

[11.5]	; 	2.16 ≤ B < 3.24

[11.0]	; 	3.24 ≤ B < 5.04

[9.5]	; 	5.04 ≤ B < 10.08

[8.5]	; 	10.08 ≤ B < 36

[6.5]	; 	36 ≤ B

Where:

B=(LCRB_alloc, 1* 12* SCS1 + LCRB_alloc,2 * 12 * SCS2)/1,000,000

## << End change >>