3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201736

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson, Rogers

Title:	TP to TR 37.717-11-21: Addition of DC configurations for DC_66_n2-n78

Agenda item:	8.19.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_66A_n2A-n78A configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_66_n2-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n2A-n78A | DC_66A_n2A DC_66A_n78A | 66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 160 |
|  |  | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3	Co-existence studies

Based on co-existence studies IMD2 and IMD4 from UL 66_n2 may impact DL n78.

Based on co-existence studies IMD2, IMD4 and IMD5 from UL 66_n78 may impact DL n2.

### 6.x.4	∆TIB and ∆RIB values

For DC_66_n2-n78, the TIB,c and RIB,c values are derived from DC_66_n2-n77 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n78 | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n78 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n78 | 0.5 |

### 6.x.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_66_n2-n77 in TS 38.101-3 and TR 37.717-11-21.

Table 6.x.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66_n2-n78 | 66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 34.9 | TDD | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 20.9 | TDD | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 21.1 | FDD | IMD4 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |

##### ---End of changes---

# Reference

[1]		RP-212876, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics