3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201757

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson, Rogers

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_12-66_n2-n78

Agenda item:	8.19.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 to add DC_12A-66A_n2A-n78A configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 7.x	DC_12-66_ n2-n78

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12-66_n2-n78 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n2A-n78A | DC_12A_n2A DC_66A_n2A DC_12A_n78A DC_66A_n78A | CA_12A-66A | CA_n2A-n78A |

### 7.x.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.x.4	∆TIB and ∆RIB values

For DC_12-66_n2-n78, the TIB,c and RIB,c values are derived from DC_2-12-66_n78 and given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n2-n78 | 12 | 0.3 |
|  | 66 | 0.5 |
|  | n 2 | 0.5 |
|  | n78 | 0.8 |

Table 7.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n2-n78 | 12 | 0 |
|  | 66 | 0.3 |
|  | n 2 | 0.3 |
|  | n78 | 0.5 |

### 7.x.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

##### ---End of changes---

# Reference

[1]	RP-202292, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics