3GPP TSG-RAN WG4 Meeting #101-bis-e		R4- 2201783

E-meeting, January 17-25, 2022

Agenda item:	6.6.5.2

Source:	Intel Corporation

Title:	Discussion on PDSCH requirements for DL 1024QAM in FR1

Document for:	Discussion

# Introduction

During the RAN4#101-e meeting most of the simulation parameters for 1024QAM PDSCH tests were agreed [1]. In this paper we discuss the leftover non-agreed parameters and provide corresponding simulation results.

# Discussion

The simulation parameters agreed in [1] are shown in Table 1. The corresponding simulation results can be found in Figure 1 for FDD scenarios and in Figure 2 for TDD scenarios

Table 1. Simulation parameters for PDSCH demodulation for DL1024QAM.

| Parameters | FDD | TDD |
| --- | --- | --- |
| SCS/CBW | 15kHz and 10MHz | 30kHz and 40MHz |
| TDD UL/DL configuration | N/A | 7D1S2U, with S=6:4:4 FFS to schedule PDSCH in the special slots |
| MCS (See appendix) | Option 1: MCS23 Option 2: MCS24 Option 3: MCS25 |  |
| Rank | 1 |  |
| Antenna configuration | 2x2 and 2x4 with ULA Low |  |
| Propagation channel | Option 1: TDLA30-10 Option 2: TDLD30-5 |  |
| PDSCH configuration | Type A mapping, Start symbol 2, Duration 12 |  |
| PDSCH DMRS configuration | Type 1, Single symbol, additional DMRS: pos1 |  |
| Tx EVM assumed for simulation | Option 1: 2.0% Option 2: 2.5% Option 3: 2.8% Note: The assumed Tx EVM should be same or less than the BS Tx EVM requirements. |  |
| SSB configuration | Periodicity 20 ms, Allocated in first slot within 20ms |  |
| TRS configuration | 20 ms periodicity, 2 slots, Offset 10 ms |  |
| Number of HARQ processes | 4 for FDD 15kHz, 8 for TDD 30kHz |  |
| Transform precoding | CP-OFDM |  |
| Allocated RBs | Full BWP |  |
| PRB bundling | 2 |  |
| Precoding model | Random Precoding, per slot, WB granularity (codebook configuration Single panel Type 1) |  |
| Receiver type | MMSE-IRC |  |
| Test metric | 70% of maximum throughput |  |

| Figure 1. 1024QAM PDSCH simulation results for FDD scenarios |  |
| --- | --- |

| Figure 2. 1024QAM PDSCH simulation results for TDD scenarios |  |
| --- | --- |

During the RAN4#101-e meeting two levels of Tx EVM for 1024QAM requirements were agreed in RF session [2]:

- 2.5% requirement is applicable for frequencies equal to or below 4.2 GHz,

- 2.8% requirement is applicable for frequencies above 4.2 GHz,

Considering that the Tx EVM assumed for PDSCH tests should be same or less than the BS Tx EVM requirements we propose to consider Tx EVM equal to 2.5%

Proposal 1: RAN4 to consider Tx EVM equal to 2.5% for 1024QAM PDSCH tests

Considering Proposal 1, in Table 2 and Table 3 we summarized the simulation results for BS Tx EVM equal to 2.5%.

Table 2. Summary of 1024QAM PDSCH simulation results (SNR @ 70% of Max T-put)

|  | TDLA30-10 |  |  |  |  |  | TDLD30-5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCS23 |  | MCS24 |  | MCS25 |  | MCS23 |  | MCS24 |  | MCS25 |  |
|  | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 |
| FDD | 27.1 | 23.6 | 29.1 | 25.4 | 31.8 | 27.9 | 26.6 | 23.4 | 28.6 | 25.0 | 31.4 | 27.4 |
| TDD | 27.3 | 23.7 | 29.3 | 25.4 | 32.0 | 27.8 | 26.9 | 23.6 | 28.8 | 25.0 | 31.2 | 27.3 |

Table 3. Summary of impairments 1024QAM PDSCH simulation results (SNR @ 70% of Max T-put)

|  | TDLA30-10 |  |  |  |  |  | TDLD30-5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MCS23 |  | MCS24 |  | MCS25 |  | MCS23 |  | MCS24 |  | MCS25 |  |
|  | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 | nRx=2 | nRx=4 |
| FDD | 29.6 | 26.1 | 31.6 | 27.9 | 34.3 | 30.4 | 29.1 | 25.9 | 31.1 | 27.5 | 33.9 | 29.9 |
| TDD | 29.8 | 26.2 | 31.8 | 27.9 | 34.5 | 30.3 | 29.4 | 26.1 | 31.3 | 27.5 | 33.7 | 29.8 |

As we can see from Table 3 SNR operating points for MCS24 and MCS25 scenarios are rather high. Thus, we propose to consider MCS23 for 1024QAM PDSCH tests.

Comparing the results for TDLA30-10 and TDLD30-5 propagation conditions we can see that the difference in SNR operating point is only 0.5dB. With such small difference, we prefer to use the same propagation conditions as were used for 256QAM requirements definition – TDLA30-10. The corresponding SNR operating point in Table 2 are highlighted in green.

Proposal 2: RAN4 to consider MCS23 for 1024QAM PDSCH tests

Proposal 3: RAN4 to consider TDLA30-10 propagation conditions for 1024QAM PDSCH tests

# Conclusion

Proposal 1: RAN4 to consider Tx EVM equal to 2.5% for 1024QAM PDSCH tests

Proposal 2: RAN4 to consider MCS23 for 1024QAM PDSCH tests

Proposal 3: RAN4 to consider TDLA30-10 propagation conditions for 1024QAM PDSCH tests

# References

- R4-2120699, WF on DL1024QAM demodulation and CQI reporting requirements, Ericsson, RAN4 #101-e, Nov 1-11, 2021

- R4-2120770, CR for TS 38.104: 1024QAM, Huawei, HiSilicon, RAN4 #101-e, Nov 1-11, 2021