3GPP TSG-RAN WG4 Meeting #101-bis-e		R4- 2201786

E-meeting, January 17-25, 2022

Agenda item:	6.13.6.3

Source:	Intel Corporation

Title:	Discussion on UE demodulation requirements for NR NTN

Document for:	Discussion

# Introduction

In the Rel-17 new WI introducing Solutions for NR to support non-terrestrial networks was approved [1].  The objective captured for the performance part WI is rather general:

- Specify necessary UE and network performance requirements for the specified enhancements

In this paper we discuss the scope for the UE performance requirements.

# Discussion

Scope of requirements

NR NTN system has specific propagation conditions with less scatterers long propagation distance and satellite motion. NTN channel models for link level simulations are introduced in TR38.811: two TDL models, namely NTN-TDL-A and NTN-TDL-B are constructed to represent two different channel profiles for NLOS, while NTN-TDL-C and NTN-TDL-D are constructed for LOS.

Considering the specifics of the propagation conditions, the existing demodulation requirements can not be reused for NR NTN. New requirements for all the physical channels need to be defined for NR NTN

Proposal 1: RAN4 to define the requirements for PDSCH and PDCCH for NTN-TDL channel models

For NTN systems the satellite motion causes additional Doppler shift. RAN1 assumption is that UE will compensate this frequency offset using the information about UE location (from GNSS) and satellite ephemeris (broadcasted in System Information). However, the accuracy of this autonomous pre-compensation may not be perfect so some shift will still take place. RAN4 need to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition

Proposal 2: RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition

Unlike NTN access node, NTN UE is considered as a regular UE (typically handheld) which can also operate in normal NR network. Thus, such UEs are required to pass legacy Rel-15/Rel-16 tests need

Proposal 3: NTN UE is required to pass legacy Rel-15/Rel-16 tests

Due to large RTT HARQ-ACK feedback is not efficient and requires large number of parallel HARQ processes. The following solutions were agreed in RAN1 to avoid stop-and-wait so new data can be transmitted:

- Disabling of HARQ operation per HARQ process

- Increased # of HARQ processes (32)

Based on that we propose to define two sets of requirements: one for disabled HARQ and one for increased number of HARQ processes

Proposal 4: RAN4 to define two sets of requirements – one for disabled HARQ and one for increased number of HARQ processes.

Since NTN UE is required to pass legacy tests a limited set of PDSCH and PDCCH requirements can be defined for new NTN channel models unlike Satellite Access Node where we proposed to define the requirements for almost the same set of scenarios as in legacy case [2].

Proposal 5: There is no need to repeat all the tests from the legacy set for new NTN channel models. Limited set of tests can be defined for NTN PDSCH and PDCCH

# Conclusion

In this paper we discussed the scope for the UE performance requirements. The following proposals were made:

Proposal 1: RAN4 to define the requirements for PDSCH and PDCCH for NTN-TDL channel models

Proposal 2: RAN4 to decide whether additional Doppler shift due to satellite motion should be taken into account during the requirements definition

Proposal 3: NTN UE is required to pass legacy Rel-15/Rel-16 tests

Proposal 4: RAN4 to define two sets of requirements – one for disabled HARQ and one for increased number of HARQ processes.

Proposal 5: There is no need to repeat all the tests from the legacy set for new NTN channel model. Limited set of tests can be defined for NTN PDSCH and PDCCH

# References

- RP-210908, Solutions for NR to support non-terrestrial networks (NTN), Thales, RAN #91e, March 22 – 26th, 2021

- R4-2201785, Discussion on Satellite Access Node demodulation requirements for NR NTN, Intel, RAN4 #101-bis-e, January 17-25, 2022