3GPP TSG-RAN WG4 Meeting #101-bis-e		R4- 2201788

E-meeting, January 17-25, 2022

Agenda item:	6.16.7.3

Source:	Intel Corporation

Title:	Discussion on interruption requirements for NR 52.6 – 71 GHz

Document for:	Discussion

# Introduction

In this paper we discuss the interruption requirements for new numerologies defined for NR 52.6 – 71 GHz operation.

# Discussion

## Interruptions when identifying CGI of an NR cell with autonomous gaps

During the RAN4 101-e meeting the following open issue was captured in the WF [1]:

- For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps: - Option 1: the interruption requirements to be defined as in table below Interruption length X1, Y1 and Y2 during measurements with autonomous gaps NR Slot length (ms) of victim cell; Interruption length X1 (slots); Interruption length Y1 (slots); Interruption length Y2 (slots). 0; 1; 6; 7; 6. 1; 0.5; 12; 13; 10. 2; 0.25; 24; 25; 19. 3; 0.125; 48; 49; 37. 5; 0.03125; 192; 193; 145. 6; 0.015625; 384; 385; 289. - Option 2: do not define interruption requirements

In [2] the agreement to de-prioritizing CGI identification for Rel-17 FR2-2 was made. However, in FR1 + FR2-2 DC scenario, a UE can still be required to identify CGI of a FR1 neighbour cell. In this case the interruption may occur on FR2-2 serving cells. Given that, we support Option 1 from the above.

Proposal 1: For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps the interruption requirements to be defined as in table below

Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

## Interruption requirements for NR-DC

During the RAN4 #100e meeting [3] the interruption requirements for high SCSs were agreed for SCell addition/release (intra-band CA), SCell activation/deactivation (intra-band CA) and NR SRS carrier based switching. However, the agreements were made only for Standalone NR CA case, i.e. changes for Table 8.2.2.2.1-2, Table 8.2.2.2.2-2, Table 8.2.2.2.9-1 and Table 8.2.2.2.9-2 were considered. For NR DC scenario the same approaches as for Standalone NR CA can be used for interruption requirements derivation, so the above mentioned requirements can also be applied to NR DC case.

Proposal 2: The interruptions requirements for SCell addition/release (intra-band CA), SCell activation/deactivation (intra-band CA), NR SRS carrier based switching agreed for Standalone NR Carrier Aggregation during RAN4 #100-e meeting can also be applied to NR-DC case. Tables 8.2.4.2.1-2, 8.2.4.2.2-2, 8.2.4.2.9-1 and 8.2.4.2.9-2 can be changed in the same way as Tables 8.2.2.2.1-2, 8.2.2.2.2-2, 8.2.2.2.9-1 and 8.2.2.2.9-2

# Conclusion

In this paper we discussed the open issues of interruption requirements for FR2-2 SCSs. The following proposals were made:

Proposal 1: For FR2-2 DC with FR1, when a UE is identifying CGI of a FR1 serving cell with autonomous gaps the interruption requirements to be defined as in table below

Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |
| 5 | 0.03125 | 192 | 193 | 145 |
| 6 | 0.015625 | 384 | 385 | 289 |

Proposal 2: The interruptions requirements for SCell addition/release (intra-band CA), SCell activation/deactivation (intra-band CA), NR SRS carrier based switching agreed for Standalone NR Carrier Aggregation during RAN4 #100-e meeting can also be applied to NR-DC case. Tables 8.2.4.2.1-2, 8.2.4.2.2-2, 8.2.4.2.9-1 and 8.2.4.2.9-2 can be changed in the same way as Tables 8.2.2.2.1-2, 8.2.2.2.2-2, 8.2.2.2.9-1 and 8.2.2.2.9-2

# References

- R4-2120317, “WF on NR extension to 71 GHz RRM requirements (Part 2)”, RAN4 #101-e November 01-12, 2021

- R4-2120316, “WF on NR extension to 71 GHz RRM requirements (Part 1)”, RAN4 #101-e November 01-12, 2021

- R4-2115351, “WF on R17 NR_ext_to_71GHz_RRM_1”, RAN4 #100-e August 16 – 27, 2021