3GPP TSG-RAN WG4 Meeting #101b-e	R4-2201794

Electronic Meeting, 17th - 25th Jan, 2022

Source:	ZTE Corporation

Title:	Revision on TR 38.344 Section 6.2.3

Agenda Item:	7.2.3.3.2

Document for:	Approval

# Introduction

In RAN4#101-e, a WF was agreed on overlapping channel approach [1], which proposed to include a statement in TR that the overlapping CA solution requires UE support of intra-band non-contiguous CA for the combination. And one further clarification is that this statement applies only to the overlapping CA from UE perspective.

# Proposed Revisions

### 6.2.3	Overlapping CA (two cells)

#### 6.2.3.1      General Aspects

Using the Overlapping CA approach to support irregular spectrum is one of the methods without the need of introduction of new dedicated channel bandwidths for both UE and BS. Figure 6.2.3-1 shows overlapping CA from network perspective, in which BS supports the intra-band overlapping CA, while UEs only supports the single CC with the existing channel bandwidth. And figure 6.2.3-2 shows overlapping CA from UE perspective, in which both BS and UE support the intra-band overlapping CA to use all RBs, and the UE supports intra-band non-contiguous CA for the combination as well.

Even two CCs are overlapped in frequency domain, the network can configure different CCs with non-overlapping BWP to avoid the conflict of physical signals or channels.

UE1: legacy channel bandwidth

UE2: legacy channel bandwidth

Overlapping CA for gNB

Figure 6.2.3-1: Overlapping CA from network perspective

Overlapping CA for gNB

UE3: overlapping CA of CC1 and CC2

CC1: BWP#1

CC2: BWP#2

Figure 6.2.3-2: Overlapping CA from UE perspective

In summary,

-	No new gNB channel bandwidth is required

-	Legacy UE using existing lower regular NR channel bandwidth can operate in either carrier

-	Overlapping CA approach needs to have PRB grid alignment between overlapping CCs

-	UE perspective, overlapping CA is optional supported in DL only

-	Irregular bandwidth utilizing overlapping CA approach cannot be combined with NR CA combinations

# Reference

- [1] R4-2120011 WF on overlapping channel approach, Huawei