3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201801

Electronic Meeting, January 17-25, 2022

Source:	ZTE Corporation

Title:	Simulation results for PUSCH 256QAM demodulation requirements

Agenda Item:	6.12.3.1

Document for:	Approval

# Simulation Results

PUSCH 256QAM simulation assumption was agreed in RAN4 #100-e meeting [1].

| Parameter | Value |
| --- | --- |
| Transform precoding | Disabled |
| CP | Normal CP |
| Number of Tx | 1 |
| Number of Rx | 2, 4, 8 |
| Number of layers | 1 |
| TDD pattern | 15kHz SCS: 3D1S1U, S=10D:2G:2U 30kHz SCS: 7D1S2U, S=6D:4G:4U |
| DM-RS sequence generation | NID=0, nSCID=0 |
| DMRS type | Type 1 with single-symbol DM-RS |
| Number of DMRS symbols | 1+1 |
| Number of DM-RS CDM groups without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 Low |
| SCS and BW | 15kHz: 5MHz, 10MHz; 30kHz: 10MHz, 40MHz, 100MHz |
| SRS, PT-RS | Not configured |
| Phase noise modelling | No explicit PN modelling (Note 1) |
| Tx EVM | No explicit 3.5% Tx EVM modelling (Note 2) |
| Timing offset, Frequency offset | 0, 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions and RV sequence | 4, {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| Note 1: The phase noise impact can be included in the impairment results, but it is left up to companies Note 2: Consider 3.5% Tx EVM impact in the impaired results, but it is left up to companies. |  |

In this paper, we update the simulation results with type A 100MHz BW and type B results added.

| Mapping Type | Channel | DMRS Config | SCS [kHz] /CBW [MHz] | 1T2R [dB] |  | 1T4R [dB] |  | 1T8R [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal | Impairment | Ideal | Impairment | Ideal | Impairment |
| Type A | TDLA30-10 Low | DMRS 1+1 | 15 /5 | 17.2 | 19.7 | 13.2 | 15.7 | 11.1 | 13.6 |
|  |  |  | 15 /10 | 18.2 | 20.7 | 14.2 | 16.7 | 10.7 | 13.5 |
|  |  |  | 30/10 | 17.7 | 20.2 | 13.2 | 15.7 | 11.4 | 13.9 |
|  |  |  | 30/40 | 18.4 | 20.9 | 14.4 | 16.9 | 10.9 | 12.4 |
|  |  |  | 30/100 | 20 | 22.5 | 15.2 | 17.7 | 11.4 | 13.9 |
| Type B | TDLA30-10 Low | DMRS 1+1 | 15 /5 | 17.8 | 20.3 | 13.8 | 16.3 | 10.3 | 12.8 |
|  |  |  | 15 /10 | 18.5 | 21 | 14.3 | 16.8 | 10.7 | 13.2 |
|  |  |  | 30/10 | 17.1 | 19.6 | 13.9 | 16.4 | 10.6 | 13.1 |
|  |  |  | 30/40 | 18.2 | 20.7 | 14.4 | 16.9 | 10.9 | 13.4 |
|  |  |  | 30/100 | 19.9 | 22.4 | 15.2 | 17.7 | 11.4 | 13.9 |

# Reference

R4-2115748 WF for FR1 PUSCH 256QAM performance requirements_final.doc