3GPP TSG-RAN WG4 Meeting # 100-e	R4-2201809

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	TP to TR 38.853: BS RF requirements

Agenda Item:	5.4.3

Document for:	Approval

# Introduction

In this TP to TR 38.853 additional clarifications to the derivation of the RMR 900 BS RF requirements are added.

# Conclusions

Proposal 1: Approve the attached TP to TR 38.853.

# Annex A: TP to TR 38.853 for RMR900

------------------------------ Modified sections ------------------------------

# 7	RF requirements

## 7.1	BS specific requirements

### 7.1.1	General

The BS RF work for RMR900 was limited to the Wide Area Base Stations, only.

Referring to the ECC 20(02) decision in [1], one can observe that related analyses were done for the non-AAS BS architectures, only. Therefore the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the EC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements. It was agreed to perform EIRP-to-conducted limits conversion assuming a fixed antenna gain value of GantRMR900 = 17 dBi, assuming internal losses (feeder, etc.) of 0 dB.

### 7.1.2	Transmitter characteristics

#### 7.1.2.1	BS maximum output power

Based on EC Decision (20)02 [1], the BS maximum output power for BS operating in band n100 in uncoordinated deployment, shall not exceed the Prated,c,AC derived based on table 4.3.1-1, with the assumption of fixed value of the BS antenna gain GantRMR900 as:

(64.5 – GantRMR900) dBm / 5 MHz + (fDL-922.1) x 40/3 dB = 47.5 dBm/5MHz + (fDL - 922.1) × 40/3 dB

, where GantRMR900 is the antenna gain of the RMR BS.

#### 7.1.2.2	Unwanted emissions

##### 7.1.2.1.1	General

Only Category B emission requirements are applicable to NR operation in n100 band.

##### 7.1.2.1.2	OBUE

It was agreed to capture the following OBUE requirement for band n100 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.2.1.2-1. The requirement is derived based on table 4.3.1-4, with the assumption of fixed value of the BS antenna gain GantRMR900

Table 7.1.2.1.2-1: OBUE requirement derivation for n100

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.1 MHz f_offset < 0.3 MHz | 32.5 dBm – GantRMR900 = 15.5 dBm | 200 kHz |
| 0.2 MHz f < 1 MHz | 0.6MHz f_offset < 1.4 MHz | 14 dBm – GantRMR900 = -3 dBm | 800 kHz |
| 1 MHz f 10 MHz | 1.5 MHz f_offset < 10.5 | 5 dBm – GantRMR900 = -12 dBm | 1 MHz |
| NOTE: GantRMR900 antenna gain value assumed as 17 dBi. |  |  |  |

##### 7.1.2.1.3	Tx spurious emissions

It was agreed to capture additional Tx spurious emissions requirement for band n100 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.2.1.3-1. The requirement is derived based on table 4.3.1-5, with the assumption of fixed value of the BS antenna gain GantRMR900:

Table 7.1.2.1.3-1: Additional Tx spurious emissions requirement derivation for n100

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 880 MHz – 915 MHz | - 49 - GantRMR900 = - 66 dBm (NOTE) | 5 MHz |
| NOTE: GantRMR900 antenna gain value assumed as 17 dBi |  |  |

------------------------------ Next modified sections ------------------------------

#### 7.1.3.2	Rx blocking

It was agreed to capture Rx blocking requirement for band n100 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.3.2-1.

Table 7.1.3.2-1: Rx blocking requirement derivation for n100

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Center Frequency of Interfering Signal (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | PREFSENS + 3 dB = - 98.7 | Wide Area BS: -34 | 870.1 - 874.3 | [200 kHz; signal details: TBD] |

------------------------------ End of modified section ------------------------------