3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201811

# Electronic Meeting, January 17-25, 2022

Source: 	Huawei

Title: 	Consideration of coordinated/un-coordinated deployments for the RMR900 requirements in RAN4 specifications

Agenda Item:	5.4.4

Document for:	Discussion

# Introduction

In this contribution, we discuss aspects related to the coordinated/un-coordinated deployments considerations for RMR BS and related implications on RAN4 specifications.

# Discussion

# Coordinated/un-coordinated deployment considerations

Referring to the ECC decision (20)02 [1], the following decisions related to the deployment aspects and the potential need for coordination procedures was captured (applicable to both RMR900 and RMR1900):

It shall be clarified, that the above mentioned coordination procedures, and/or mitigation measures are out of scope of RAN4 specification and this WI.

Furthermore, the following RMR900 specific technical considerations were captured in ECC decision (20)02 [1]:

The above extract of the technical conditions for RMR BS states that the Block Edge Mask (BEM) was developed on the basis that detailed coordination and cooperation agreements would not be required to be in place prior to network deployment.

Referring to the RAN4 work, the minimum RF performance requirements are defined with the assumptions, that no coordination is necessary for BS deployment. The usual practise in RAN4 for performing the co-existence simulations among two services is based on the set of simulation assumptions, in order to derive ACLR/ACS parameters which reassure their co-existence without causing harmful interference to each other, i.e. those two services shall be safe to co-exist without the need for any dedicated coordination procedures, or interference mitigation measures.

With the above it is proposed to apply the following clarifications to the RMR TR, as well as to the RMR CR, where applicable:

Proposal 1: by default, the RMR BS RF requirements assume an un-coordinated deployment.

Proposal 2: capture related clarifications to the RMR TR, as proposed in the attached TP to TR.

# Conclusion

Based on the above discussion, the following is proposed:

Proposal 1: by default, the RMR BS RF requirements assume an un-coordinated deployment.

Proposal 2: capture related clarifications to the RMR TR, as proposed in the attached TP to TR.

# References

[1]		ECC Decision (20)02, Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR)

[2]		R4-2120725, Email discussion summary for [101-e][314] RAIL_900_1900MHz_BSRF_NWM, RAN4#101-e

# Annex A: TP to TR 38.853

------------------------------ Modified sections ------------------------------

# 9	Deployment aspects

Even though network deployment aspects are out of scope of the RAN4 specifications, it was agreed that in principle, RMR base stations are not expected to be co-located with MFCN BSs. Therefore, for the derivation of the BS RF requirements it was assumed that the RMR and MFCN base stations are not co-located.

The minimum RF performance requirements were defined with the assumptions, that no coordination is necessary for RMR BS deployment.

Referring to the ECC (20)02 [x], the following decision was captured:

that CEPT administrations wishing to allow multiple carriers using wideband technologies or higher e.i.r.p. for RMR BS than stated in the technical conditions should consider the implementation of a coordination procedure or other mitigation measures

It shall be clarified, that the above mentioned coordination procedures, and/or mitigation measures are out of scope of RAN4 specification and this WI.

------------------------------ End of modified section ------------------------------