3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201812

# Electronic Meeting, January 17-25, 2022

Source: 	Huawei

Title: 	Cross-check on the implementation of ECC Decision (20)02 for RMR1900

Agenda Item:	5.5.1

Document for:	Discussion

# Introduction

In this contribution, we analyse progress on translation of the ECC decision (20)02 [1] into RAN4 BS RF requirements for RMR1900.

# Discussion

Referring to the ECC decision (20)02 [1], the following BS RF requirements were outlined for RMR1900 operation (CAB-radio related aspects are excluded in this analysis):

- General in-block requirements

- Baseline requirements

- RX blocking requirements

The above requirements are analysed in the following sub-clauses.

# General in-block requirements

Annex 3 (Least Restrictive Technical Conditions for a Wideband RMR carrier for RMR 1900) of the ECC decision (20)02 [1] specifies general in-block requirements in Table 9:

Referring to [2], the above requirements were addressed by the following agreement on the BS maximum output power:

Option 1 as baseline, i.e. based on R4-2118167: Capture in TS 38.104 that for BS operating in n101 band, [for uncoordinated deployment], the BS rated output power Prated,c,AC shall not exceed 47 dBm/10MHz

Observation: RMR1900 requirements on general in-block requirement for 10 MHz channel mandatory for uncoordinated deployment was agreed to be captured in TS 38.104 as BS rated output power requirement (BS maximum output power).

# Baseline requirements

Annex 3 (Least Restrictive Technical Conditions for a Wideband RMR carrier for RMR 1900) of the ECC decision (20)02 [1] specifies baseline requirements in Table 10:

Referring to [2], the above requirement was addressed by the following agreement on additional Tx spurious emissions limit:

The final limit for the additional Tx spur limit based on ECC decision is agreeable as the baseline (i.e. -61dBm/5MHz).

Observation: RMR1900 requirements on baseline requirement was agreed to be captured as additional Tx spurious emissions limit.

# RX blocking requirements

Annex 3 (Least Restrictive Technical Conditions for a Wideband RMR carrier for RMR 1900) of the ECC decision (20)02 [1] specifies baseline requirements on wideband RMR BS receiver characteristics in Table 11:

Referring to [2], the above requirements were addressed by the following agreement on the RX blocking:

Option 1 agreed as baseline, i.e. based on R4-2118167: BS channel bandwidth of the lowest/highest carrier received (MHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Centre Frequency of Interfering Signal [MHz]; Type of interfering signal. [5], 10; PREFSENS + 3 dB; Wide Area BS: -20; 1807.5 MHz to 1877.5 MHz; 5 MHz NR signal. NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3..

Based on the discussion the above interfering signal characteristics was to be further discussed (enter frequency and the interfering signal type), with the text of the note proposed to be further adjusted.

Observation: RMR1900 requirements on wideband RMR BS receiver characteristics were agreed as BS RX blocking.

Missing details of the interfering signal type are discussed in [3].

# Conclusion

In this contribution, we have analysed remaining issues of the ECC (20)02 [1] regulations for RMR1900. Based on the above analysis of RMR-specific requirements in ECC decision (20)02 [1], all RMR-specific BS RF requirements are considered to be already addressed, subject to implementation in NR BS specification.

# References

[1]		ECC Decision (20)02, Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR)

[2]		R4-2120725, Email discussion summary for [101-e][314] RAIL_900_1900MHz_BSRF_NWM, RAN4#101-e

[3]		R4-2201813, Further discussion on the remaining BS RF requirements for RMR1900