3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2201813

# Electronic Meeting, January 17-25, 2022

Source: 	Huawei

Title: 	Remaining aspects for BS RF requirements for RMR1900

Agenda Item:	5.5.3

Document for:	Discussion

# Introduction

In this contribution, we provide discussion on the remaining issues for the RMR1900-specific BS RF requirements.

# Discussion

# RX blocking for RMR1900

ECC (20)02 captures the following requirement:

Referring to [2], the following was agreed last meeting on the spurious emissions requirement for RMR1900:

Referring to the previous discussion in [2] and [5], the following was considered as the baseline for Rx blocking requirement derivation:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Centre Frequency of Interfering Signal [MHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| [5], 10 | PREFSENS + 3 dB | Wide Area BS: -20 | 1807.5 MHz to 1877.5 MHz | 5 MHz NR signal |
| NOTE: PREFSENS depends on the RAT. For NR, PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. |  |  |  |  |

Referring to related discussion on RX blocking for RMR900 in [6], it is proposed to follow the same approach as for the RMR900 RX blocking requirement based on the following observation in [6]:

Observation: RMR1900 specific RX blocking requirement from ECC (20)02 is best suited to be captured as in-band blocking (narrowband) requirement.

Referring to TS 37.105 section 7.4.2.3, one can find that Additional blocking requirements were defined for the in-band requirements. Those requirements were defined as band-specific, which would also fit the RMR-specific requirement.

With the above, the RX requirement can be adjusted as presented in table below:

Table 1: Additional in-band blocking requirement Operating band; BS channel bandwidth of the lowest/highest carrier received (MHz); Wanted signal mean power (dBm); Interfering signal mean power (dBm); Centre Frequency of Interfering Signal [MHz]; Type of interfering signal. n101; [5], 10; PREFSENS + 3 dB; Wide Area BS: -20; 1807.5 to 1877.5; 5 MHz NR signal. NOTE: PREFSENS depends on the BS channel bandwidth as specified in table 7.2.2-1..

Based on the above analysis, the following is proposed:

Proposal 1: consider the ECC(20)02 RMR1900 RX blocking requirements as additional in-band blocking requirement in TS 38.104.

Proposal 2: approve the adjustments to the RX blocking requirement as outlined in table 1.

Remaining aspects of the interfering signal characteristics are to be further studied.

# Conclusion

Based on the above discussion, the following proposals are formulated:

Proposal 1: consider the ECC(20)02 RMR1900 RX blocking requirements as additional in-band blocking requirement in TS 38.104.

Proposal 2: approve the adjustments to the RX blocking requirement as outlined in table 1.

# References

[1]		ECC Decision (20)02, Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR)

[2]		R4-2120725, Email discussion summary for [101-e][314] RAIL_900_1900MHz_BSRF_NWM, RAN4#101-e

[3]		R4-2119136, Further discussion on the BS RF requirements for RMR900, RAN4#101-e

[4]	ECC Report 314, Co-existence between Future Railway Mobile Communication System (FRMCS) in the frequency range 1900-1920 MHz and other applications in adjacent bands

[5]		R4-2118167, 900MHz RMR band – BS RF, RAN4#101-3, Ericsson

[6]		R4-2201807, Remaining aspects for BS RF requirements for RMR900