3GPP TSG-RAN WG4 Meeting # 100-e	R4-2201814

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	TP to TR 38.852: BS RF requirements

Agenda Item:	5.5.3

Document for:	Approval

# Introduction

In this TP to TR 38.852 additional clarifications to the derivation of the RMR 1900 BS RF requirements are added.

# Conclusions

Proposal 1: Approve the attached TP to TR 38.852.

# Annex A: TP to TR 38.852 for RMR900

------------------------------ Modified sections ------------------------------

# 7	RF requirements

## 7.1	BS specific requirements

### 7.1.1	General

The BS RF work for RMR1900 was limited to the Wide Area Base Stations, only.

Referring to the ECC 20(02) decision in [1], one can observe that related analyses were done for the non-AAS BS architectures, only. Therefore the RAN4 requirements derivation was limited to the BS type 1-C requirements, only.

As the EIRP limits were defined in the EC Decision (20)02 [1], consideration of non-AAS BS architecture required to convert those EIRP limits into the conducted requirements. It was agreed to perform EIRP-to-conducted limits conversion assuming a fixed antenna gain value of GantRMR1900 = 18 dBi, assuming internal losses (feeder, etc.) of 0 dB.

### 7.1.2	Transmitter characteristics

#### 7.1.2.1	BS maximum output power

Based on EC Decision (20)02 [1], the BS maximum output power for BS operating in band n101 in uncoordinated deployment, shall not exceed the Prated,c,AC derived based on table 4.3.1-1, with the assumption of fixed value of the BS antenna gain GantRMR1900 as:

(65 – GantRMR1900) dBm / 10 MHz = 47 dBm / 10 MHz

, where GantRMR1900 is the antenna gain of the RMR BS.

#### 7.1.2.2	Unwanted emissions

##### 7.1.2.1.1	General

Only Category B emission requirements are applicable to NR operation in RMR1900 band.

##### 7.1.2.1.2	Tx spurious emissions

It was agreed to capture additional Tx spurious emissions requirement for band n100 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.2.1.2-1. The requirement is derived based on table 4.3.1-5, with the assumption of fixed value of the BS antenna gain GantRMR1900:

Table 7.1.2.1.1-1: Additional Tx spurious emissions requirement derivation for n101

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 1920 MHz – 1980 MHz | - 43 - GantRMR1900 = - 61 dBm (NOTE) | 5 MHz |
| NOTE: GantRMR900 antenna gain value assumed as 17 dBi. |  |  |

### 7.1.3	Receiver characteristics

#### 7.1.3.1	Enhanced BS selectivity for band n1

It was agreed that RAN4 will not specify any requirement to capture the BS enhanced selectivity (band n1) assumption.

#### 7.1.3.2	Rx blocking

It was agreed to capture Rx blocking requirement for band n101 in TS 38.104 [x], based on conversion of the EIRP limits from EC Decision (20)02 [1], as in table 7.1.3.2-1. Interfering signal characteristic is [5 MHz NR or LTE signal].

Table 7.1.3.2-1: Rx blocking requirement derivation for n101

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Center Frequency of Interfering Signal (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| [5], 10 | PREFSENS + 3 dB | Wide Area BS: -20 | 1807.5 MHz to 1877.5 MHz | 5 MHz NR or LTE signal |

------------------------------ End of modified section ------------------------------