3GPP TSG-RAN WG4 Meeting #101-bis-e				     R4-2201838

Electronic Meeting, 17th-25th of January, 2022

Agenda item:			6.13.1.4

Source:	THALES

Title:	Draft proposal to update TR 38.863 NTN related RF and co-existence aspects

Document for:	Approval

# Introduction

Draft TR skeleton for Non-terrestrial networks (NTN) related RF and co-existence aspects (TR 38.863) summarized in R2115640 Way Forward on NTN_solutions_Part1 has been approved by RAN4 100e meeting.

Meanwhile, R4-2115750 Simulation assumptions for NTN co-existence study has been approved to facilitate the co-existence study between TN and NTN. As agreed in R4-2115749 Way forward on [313] NTN_Solutions_Part2, summary of calibration results and assumptions shown in its Annex 2 and Annex 3 with possible updates received by Sept. 30 2021 will be captured in the new TR 38.863.

In this document, following changes have been proposed as a TP to update TR 38.863:

- Section B.2 Calibration Results (Annex B) of TN components with THALES calibration information provided prior to RAN4#101-e meeting before Sept. 30 2021, together with new variance and mean values.

# Reference

[1] R2115640 Way Forward on NTN_solutions_Part1

[2] R4-2115750 Simulation assumptions for NTN co-existence study

[3] R4-2115749 Way forward on [313] NTN_Solutions_Part2

# Text proposal for TR38.863

------------------------------------------------<Start of TP>-----------------------------------------------

- Annex B:
Calibration results of TN components

## B.1 Calibration assumptions

Assumptions in Section 6.2 are adopted as baseline for calibration. It should be noted there are different parts which are listed in Table B.1-1.

Table B.1-1 TN Assumptions for calibration

| Calibration assumptions |  |  |
| --- | --- | --- |
| TN AAS | Rural | Element gain: 7.1 dBi |
|  |  | 3dB: H 90 / V 54 |
|  |  | Front-back: 30 H/V |
|  |  | Array: 8x8 |
|  |  | Element spacing: H 0.5/V 0.9 |
|  |  | Conducted Tx: 25 dBm |
|  |  | Ohmic loss: 2 dB |
|  |  | Mechanical downtilt: 3 deg |
|  |  | Polarization gain 3 dB |
|  |  | #UE: 1 DL/ 3 UL |
|  |  | 100% Outdoor |
|  |  | ISD 7.5 KM |
|  | Urban | Element gain: 6.4 dBi |
|  |  | 3dB: H 90 / V 65 |
|  |  | Front-back: 30 H/V |
|  |  | Array: 8x8 |
|  |  | Element spacing: H 0.5/V 0.7 |
|  |  | Conducted Tx: 25 dBm |
|  |  | Ohmic loss: 2 dB |
|  |  | Mechanical downtilt: 10 deg |
|  |  | Polarization gain 3 dB |
|  |  | #UE: 1 DL/ 3 UL |
|  |  | 100% Outdoor |
|  |  | ISD 0.75 KM |
| TN non-AAS | Antenna gain | 17 dBi |
|  | Conducted Tx | 46 dBm |
|  | 3dB | R4-2108645 Section 2.4.2 |
|  | Front-back |  |
|  | Mechanical downtilt | Rural 3 / Urban 10 |

## B.2 Calibration results

The calibration results include SINR and coupling loss distributions in DL, and SINR, coupling loss and transmit power distributions in UL. Both rural macro and urban macro propagation models defined in TR 38.901 are considered. NR BS with AAS and non-AAS antennas are calibrated separately and results are summarized in Table B.2-1, 2-2 and 2-3. [NB-IoT scenarios are not calibrated.]

Note: The “-” means the data was not provided by its corresponding contributor.

Table B.2-1 Calibration summary for TN NR AAS DL cases

| Calibration metrics |  | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CDF percentile |  | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 60.05 | 88.58 | 98.98 | 8.47 | 24.01 | 45.77 |
|  | Huawei | 52.84 | 85.79 | 103.53 | 5.75 | 22.23 | 43.40 |
|  | Qualcomm | 57.89 | 87.11 | 100.61 | 5.08 | 19.51 | 43.65 |
|  | Nokia | 59.04 | 87.59 | 99.32 | 12.42 | 20.15 | 36.50 |
|  | Ericsson | - | - | - | 6.66 | 19.58 | 39.57 |
|  | ZTE | 58.76 | 86.90 | 99.74 | 5.63 | 20.56 | 45.14 |
|  | Variance | 6.42 | 0.83 | 2.69 | 6.35 | 2.62 | 10.70 |
|  | Mean | 57.72 | 87.19 | 100.44 | 7.33 | 21.01 | 42.34 |
| Rural Macro | Samsung | 78.09 | 116.25 | 127.96 | 4.52 | 16.53 | 37.71 |
|  | Huawei | 77.53 | 114.99 | 128.13 | 3.02 | 15.15 | 31.35 |
|  | Qualcomm | 78.27 | 116.81 | 131.58 | 0.17 | 12.60 | 35.83 |
|  | Nokia | 78.55 | 116.18 | 127.72 | 5.60 | 14.93 | 28.23 |
|  | Ericsson | - | - | - | 10.03 | 20.17 | 37.72 |
|  | Variance | 0.14 | 0.44 | 2.51 | 10.51 | 6.21 | 14.21 |
|  | Mean | 78.11 | 116.06 | 128.85 | 4.67 | 15.87 | 34.17 |

Table B.2-2 Calibration summary for TN NR non-AAS DL cases

| non-AAS NR DL |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DL Coupling Loss |  |  | DL Geometry SINR |  |  |
|  |  | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 73,23 | 99,36 | 110,33 | -1,76 | 6,99 | 14,85 |
|  | Huawei | 70,73 | 99,50 | 113,81 | -5,02 | 5,47 | 14,44 |
|  | Qualcomm | 71,85 | 99,44 | 113,47 | -4,03 | 4,52 | 14,81 |
|  | ZTE | 71,00 | 99,00 | 112,09 | -2,89 | 5,24 | 14,99 |
|  | THALES | 71,84 | 100,22 | 112,26 | -3,03 | 4,25 | 14,40 |
|  | Variance | 0,95 | 0,04 | 1,88 | 1,50 | 0,81 | 0,04 |
|  | Mean | 71,73 | 99,50 | 112,39 | -3,35 | 5,29 | 14,70 |
| Rural Macro | Samsung | 87,82 | 125,94 | 138,22 | -4,25 | 5,77 | 15,42 |
|  | Huawei | 93,87 | 126,75 | 141,60 | -8,93 | 1,10 | 15,33 |
|  | Qualcomm | 89,63 | 127,43 | 142,65 | -7,37 | 2,54 | 15,47 |
|  | THALES | 88,83 | 127,81 | 139,57 | -5,31 | 2,57 | 15,24 |
|  | Variance | 6,43 | 0,37 | 3,58 | 3,78 | 3,81 | 0,00 |
|  | Mean | 90,04 | 126,98 | 140,51 | -6,47 | 3,00 | 15,37 |

Table B.2-3 Calibration summary for TN NR AAS UL cases

| Calibration metrics |  | UL Coupling Loss |  |  | UL Geometry SINR |  |  | UL UE Tx Power |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDF percentile |  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 60.00 | 88.58 | 99.02 | 3.42 | 14.01 | 14.96 | -25.77 | 2.81 | 13.25 |
|  | Huawei | 52.84 | 85.79 | 103.53 | 7.53 | 13.89 | 14.89 | -31.46 | 4.14 | 16.51 |
|  | Qualcomm | 62.00 | 91.09 | 104.80 | 1.33 | 10.10 | 13.87 | -23.77 | 5.32 | 19.03 |
|  | Nokia | 59.04 | 87.59 | 99.32 | 10.98 | 14.55 | 15.10 | -27.23 | 1.32 | 13.05 |
|  | Ericsson | - | - | - | 6.83 | 12.94 | 15.08 | -28.52 | -1.70 | 13.98 |
|  | ZTE | 58.43 | 87.22 | 98.46 | 5.47 | 13.42 | 14.81 | -27.59 | 1.20 | 12.44 |
|  | Variance | 9.37 | 3.11 | 6.81 | 9.41 | 2.11 | 0.18 | 5.61 | 5.14 | 5.41 |
|  | Mean | 58.46 | 88.05 | 101.03 | 5.93 | 13.15 | 14.78 | -27.39 | 2.18 | 14.71 |
| Rural Macro | Samsung | 78.00 | 116.25 | 127.95 | -4.86 | 7.03 | 14.96 | -7.77 | 23.00 | 23.00 |
|  | Huawei | 77.53 | 114.99 | 128.13 | -10.84 | 4.42 | 14.86 | -3.46 | 23.00 | 23.00 |
|  | Qualcomm | 82.05 | 120.47 | 135.46 | -13.41 | 2.07 | 14.69 | -3.72 | 23.00 | 23.00 |
|  | Nokia | 78.42 | 116.20 | 127.85 | -4.15 | 7.69 | 15.17 | -7.85 | 22.99 | 22.99 |
|  | Ericsson | - | - | - | 3.30 | 13.55 | 15.14 | -8.00 | 21.55 | 23.00 |
|  | Variance | 3.20 | 4.32 | 10.50 | 33.91 | 14.88 | 0.03 | 4.41 | 0.34 | 0.00 |
|  | Mean | 79.00 | 116.98 | 129.85 | -5.99 | 6.95 | 14.96 | -6.16 | 22.71 | 23.00 |

Table B.2-4 Calibration summary for TN NR non-AAS UL cases

| non-AAS NR UL |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | UL Coupling Loss |  |  | UL Geometry SINR |  |  | UL UE Tx Power |  |  |
|  |  | @5% | @50% | @95% | @5% | @50% | @95% | @5% | @50% | @95% |
| Urban Macro | Samsung | 73,18 | 99,31 | 110,33 | -10,99 | 4,31 | 9,49 | -12,59 | 13,54 | 23,00 |
|  | Huawei | 70,73 | 99,50 | 113,81 | -6,46 | 3,08 | 8,88 | -10,26 | 18,51 | 23,00 |
|  | Qualcomm | 71,53 | 99,51 | 113,45 | -3,52 | 3,36 | 8,00 | -14,24 | 13,74 | 23,00 |
|  | ZTE | 70,11 | 98,57 | 111,77 | -1,39 | 3,87 | 8,01 | -16,23 | 12,57 | 23,00 |
|  | THALES | 71,84 | 100,22 | 112,26 | -0,01 | 4,13 | 6,80 | -14,05 | 14,90 | 22,76 |
|  | Variance | 1,32 | 0,15 | 1,94 | 12,98 | 0,22 | 0,39 | 4,79 | 5,32 | 0,00 |
|  | Mean | 71,48 | 99,42 | 112,32 | -4,48 | 3,75 | 8,24 | -13,47 | 14,65 | 22,95 |
| Rural Macro | Samsung | 87,78 | 125,93 | 138,22 | -17,23 | -3,82 | 14,65 | 2,01 | 23,00 | 23,00 |
|  | Huawei | 93,87 | 126,75 | 141,60 | -24,83 | -9,72 | 14,75 | 12,88 | 23,00 | 23,00 |
|  | Qualcomm | 89,53 | 127,64 | 142,62 | -21,39 | -5,65 | 14,33 | 3,76 | 23,00 | 23,00 |
|  | THALES | 88,83 | 127,81 | 139,57 | -17,29 | -5,16 | 14,26 | 3,55 | 22,42 | 22,94 |
|  | Variance | 6,55 | 0,49 | 3,54 | 9,66 | 6,07 | 0,03 | 22,71 | 0,00 | 0,00 |
|  | Mean | 90,00 | 127,03 | 140,50 | -20,18 | -6,09 | 14,50 | 5,55 | 22,86 | 22,99 |

------------------------------------------------<End of TP>-------------------------------------------------