3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201846

Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On UL Timing Adjustment in HST FR2 Deployments

Agenda item:	6.9.4.4 Timing requirements

Document for:	Discussion

Introduction

The topic of UL timing in HST FR2 deployments was actively discussed during the last several RAN4 meeting, including RAN4#101-e [1].

The main challenge in the HST FR2 deployments is how to keep correct UL timing when a UE/CPE is switched between the beams of non-collocated RRHs belonging to the same cell, i.e., with the same Physical Cell ID (PCI). In this case, the propagation delays between the RRHs can be significantly different. However, following the legacy (Rel-15) beam management, the CPE does not need to go through the RACH process like in a regular HO that includes UL timing adjustment in Random Access Reply (RAR). Moreover, the legacy network-controlled UL timing adjustment mechanism used in CONNENTED sate is not applicable either because the network does not have information about the propagation delay to the target RRH before the beam/TCI state switch.

At the RAN4#101-e meeting GtW, it was agreed that it is up to network configuration to enable one shot large uplink timing adjustment mechanism [2]. If one shot large uplink timing adjustment is disabled, existing uplink timing adjustment, i.e., RA based mechanism, and related existing RAN4 requirements will be applied when needed. These agreements allow for two solutions to the problem of a large jump in propagation delays in HST FR2 scenarios:

- UE-based mechanism where the UE is allowed to adjust its timing in one shot for the values above Tq autonomously, i.e., without explicit signaling of the TA value from the network.

- Network-controlled mechanism based on UL timing measurements and signaling performed by the network, i.e., using RACH procedures.

In this contribution we are analyzing conditions that are required for the triggering of these mechanisms, and discuss the crucial open issues.

Discussion

On the configuration of UL TA mechanisms

Firstly, we would like to discuss what mechanisms are available for the compensation of the large jump in propagation delays in HST FR2 deployments and how they can be enabled.

Based on the discussions at the previous RAN4#101-e meeting the following agreements and issues were listed in the WF [2]:

- It is up to network configuration to enable one shot large uplink timing adjustment mechanism - RAN4 will further study if additional flag, e.g., unidirectional flag on top of general FR2 HST scenario flag is needed to enable one shot large uplink timing adjustment - RAN4 will further study the network configuration means to disable one shot large uplink timing adjustment. - If one shot large uplink timing adjustment is disabled, existing uplink timing adjustment, i.e., RA based mechanism, and related existing RAN4 requirements will be applied when needed - Introduce a mechanism for one shot large uplink timing adjustment for FR2 HST scenarios with UE allowed to adjust uplink timing beyond Tq - FFS for conditions and additional network assistance for UE to apply one shot large uplink timing adjustment.

From the agreements above one can understand that there are two mechanisms that can be used as a solution to the problem of large jump in propagation delay when RRHs of the same cell are changed in HST FR2 deployment:

- UE-based mechanism:

  - One-shot large uplink timing adjustment is enabled by the network

  - The UE is allowed to adjust uplink timing beyond Tq in one shot without explicit signaling of the timing adjustment value from the network

- Network-controlled mechanism:

  - One shot large uplink timing adjustment is disabled

  - The solution is based on existing uplink timing adjustment, i.e., RA based mechanism

It is obvious from the agreement that network shall have means to enable and desirable one-short large uplink timing adjustment mechanism i.e., UE-based mechanism. We do not think that enabling/disabling of UE-based mechanism needs to be very dynamic, i.e., we expect that the same mechanism will be used across the whole deployment. Hence, the simplest approach would be to define a binary flag (e.g., RRC) that will clearly indicate when UE-based mechanism is enabled. At the same time, if the flag is disabled, it is straightforward that Network-controlled mechanism shall be used.

There is no need in dynamic signaling for enabling/disabling UE one-shot large UL timing adjustment. Dynamic signaling will require more profound changes in the existing specifications and larger involvement of the other WGs what is likely not possible in terms of Rel-17 HST FR2 WI.

RAN4 to define network signaling flag to enable/disable one-shot large UL timing adjustment beyond Tq at UE.

We also add a TP to our LS [3] on HST FR2 signaling flags in the Appendix to clarify our proposal even further.

Next, we need to discuss whether additional conditions might be needed to enable one-shot large UL timing adjustment mechanism at UE. At least the following two pre-conditions were introduced in the WF [2]:

- General FR2 HST flag

- Uni-directional flag

Note, that we consider here only signaling that is needed to enable the UE-based mechanism as a whole, and not additional signaling that might be used as a part of the mechanize.

The deployments where a large jump in propagation delays among non-collocated RRHs of the same cell were identified only in HST FR2 so far. Hence, we see it logical that UE-based solution is limited to these scenarios only. Moreover, HST FR2 enhanced RRM requirements flag has been already agreed in RAN4.

RAN4 to use FR2 HST scenario flag as a pre-condition for one-shot large UL timing adjustment beyond Tq at UE.

Regarding the uni-directional deployment flag, it is obvious that the jump in propagation delay is typical for uni-directional deployments. However, as we show below, it cannot be guaranteed that the jump cannot happen in other HST FR2 deployment scenarios. Additionally, it was not agreed yet in RAN4 whether signaling of uni-/bi-directional operation is needed.

Below, we analyze a bi-directional Scenario-B deployment in order to check how the RRHs are switching for a moving HST UE.

A bi-directional HST FR2 Scenario-B deployment with RRH indexes and coordinates is shown in Figure 1.

Figure 1: A scheme of HST FR2 Bi-directional Scenario-B deployment.

In Figure 2, we demonstrate RSRP trace recorded for the UE moving from RRH7 towards the RRH10. The trace of corresponding serving and best target RRHs is shown in the bottom of the figure. In this example, we demonstrate L3, i.e., HO-based, mobility.

It can be seen that for the area in between the RRH9 and RRH10 on the right, the serving RRH is switched only once roughly in the middle. The area next to the RRH9 is served by the RRH10, and the area closer to RRH10 is served by the RRH9.

However, on the left, in the area between the RRH7 and RRH8, the RRH5 signal strength becomes high enough so that UE is switching from RRH8 to RRH5 and back.

Figure 2:RSRP traces of serving and best target RRHs (above) and The indexes of serving and best target RRHs (below) for a UE moving in HST FR2 bi-dictional scenario-B (1 Tx beam per RRH), L 3

The switching from RRH5 back to RRH8 happens at coordinate 2700. Therefore, the distance to the RRH5 located at coordinate 1750 is 1050m. Whereases, the distance to the RRH8 located at coordinate 3150 is only 450m. This results in a significant jump in propagation delay of 3.5us – 1.5us = 2 us. The difference in propagation delays is around 4CP length at 120KHz SCS.

A similar behavior but with even more frequent ping-pongs can be observed when L1-based (TCI-state switching) mobility is considered, i.e., DPS scheme is used for all RRHs. This behavior is shown in Figure 3 where the change in serving RRH is significantly more frequent than in the non-DPS case (Figure 2).

Figure 3: RSRP traces of serving and best target RRHs (above) and The indexes of serving and best target RRHs (below) for a UE moving in HST FR2 bi-dictional scenario-B (1 Tx beam per RRH) with DPS.

Therefore, we can conclude that large jumps in propagation delay can happen not only in uni-directional but also in bi-directional deployments.

The beam management/switching algorithms are network implementation specific. It cannot be guaranteed that in bi-directional deployments the switch in between the non-collocated RRHs cannot result in a significant jump in propagation delays.

We observe that the simplified assumption that the significant jump in propagation delay can happen only in uni-directional HST FR2 deployments cannot be used. The solution to the problem shall be generic enough so that there are no limitations on the implementation of beam management procedures.

RAN4 not to exclude bi-directional deployments while considering the solution to the problem of a jump in propagation delays.

Network signaling flag to enable/disable one-shot large UL timing adjustment beyond Tq at UE should be generic, i.e., not limited to a particular FR2 HST deployment and/or scenario.

Network-controlled mechanism

In this section we are focusing on the analysis of the network-controlled UL timing adjustment mechanism at inter-RRH beam switch when a large jump in propagation delay happens. This mechanism is used when the one-shot large uplink timing adjustment is disabled.

In our opinion, the most efficient and fail-safe approach in term of HST FR2 Rel-17 WI is to use already existing and standardized mechanisms for UL timing adjustment. Therefore, the preferred solution is to use random access (RA) towards the target RRH to provide a possibility for the network to measure the propagation delay from the RA preamble and to signal the necessary timing adjustment within random access reply (RAR). Transmission of RA preamble itself can be triggered with the PDCCH order.

Without the loss of generality, we are considering below the uni-directional deployment shown in Figure 4. However, the solution shall not be restricted to any particular deployment, i.e., it shall be usable in bi-directional deployments as we show in the previous Section 2.1.

Figure 4: HST FR2 generic uni-directional deployment scheme.

The signaling diagram of the proposed procedure is shown Figure 5. The core of the network-controlled UL timing adjustment mechanism is in the lower box.

Figure 5: Signaling diagram of fully network -controlled timing adjustment at inter-RRH beam switch.

In order to evaluate the total delay related to this procedure it is necessary to clarify how the UE acquires DL synchronization to the target RRH at the moment of the TCI state switching (12). If the UE is able to track the timing of the target RRH after it was detected, then PDCCH order (13) can be transmitted without any additional delays. However, if the UE still needs to synchronize to the target RRH after the TCI state switch command is received, then additional delays will be involved, e.g., for the reception of RSs for time/frequency synchronization from the target RRH. At least coarse DL synchronization shall be acquired to receive the PDCCH order.

Note, that the potential delay needed for the DL synchronization to the target RRH will be present both in the network-controlled and UE-based mechanisms. Hence, it is crustal to clarify this issue.

The legacy assumption that the source and target beams are QCLed is not true for non-collocated RRHs. DL synchronization to the target RRH might introduce additional delays both in the network-controlled and UE-based mechanisms.

RAN4 to discuss how DL synchronization can be established to the target RRH in the case of TCI state switches between the beams of non-collocated RRHs:

  - Option 1: The UE is capable of tracking timing of the beams of target RRH after it was detected

  - Option 2: UE needs to synchronize to the beam of target RRH after the TCI switch command is received

Assuming that the UE is synchronized in DL with the target RRH, the CF RA procedure can be triggered with the PDCCH order (13). It indicates to the UE that the same beam as in the TCI switch command (12) shall be selected for the RA preamble transmission. Hence, a contention free (CF) RA procedure can be used that include only two messages (14) and (15).

Network-controlled mechanism is based on already standardized procedures: TCI state switching, PDCCH Order and CF RA. Additional delay is introduced by RA but CF procedure is much faster than HO.

UE-based mechanism

In RAN4#101-e meeting outcomes the following implementation options for the one-shot large timing adjustment were listed [2]:

- Introduce a mechanism for one shot large uplink timing adjustment for FR2 HST scenarios with UE allowed to adjust uplink timing beyond Tq - FFS for conditions and additional network assistance for UE to apply one shot large uplink timing adjustment. - The following options can be considered for triggering condition and network assistance - Option 1: No condition except DL timing difference: - UE will apply one shot large timing adjustment if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - Option 2: TCI switching without network assistance: - UE will apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - Option 3: TCI switching with network assistance of indication of inter-RRH and UE large DL timing change detection - UE will apply one shot large timing adjustment on TCI switching between RRH occasion if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH. - Option 4: TCI switching with network assistance of indication of inter-RRH but without UE large DL timing change detection - UE will apply one shot large timing adjustment on TCI switching between RRH occasions - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH. - Performance degradation and impact to signalling design shall be discussed for above procedures - RAN4 will further discuss the accuracy performance and testing issues based on conclusion of above procedures

As far as we can see, in HST FR2 scenarios the jump in the propagation delay can happen only in the situations when the beams switching for the UE takes place in between non-collocated RRHs. Therefore, a safer solution would be to link the one-shot large timing adjustment to the TCI beam switching. Hence, Option 2 is more preferred by us than Option 1.

RAN4 to consider TCI state switch as a triggering condition for the one-shot large UL timing adjustment.

Next, it is also straightforward that the UE shall be capable of calculating the DL timing difference in between the source and target beams. Otherwise, it is not clear what will be the value of timing adjustment applied by the UE. Therefore, we do not see a clear reason for the support of Option 4.

RAN4 to consider UE large DL timing change above Tq as triggering condition for the one-shot large UL timing adjustment.

Option 3 also includes a proposal on the network assistance for the identification of inter-RRH beam switching. As already discussed in the previous meetings, such specification changes could be challenging due to a need to involve considerable resources from the other RAN WGs. Hence, we are doubting if this is possible in terms of Rel-16 WI.

Introduction of network assistance, such as changes in the MAC-CE or extensive signalling of SSB-RRH mapping, is challenging to be achieve in terms of Rel-16.

Additionally, there is still a number of open issues common to all of the options. For example, in our paper submitted at the previous meeting [4], we demonstrate that large one-shot timing adjustment performed on UE side results in the mismatch in between the information about timing advance  on the networks side and the actual timing advance applied at the UE. Therefore, existing RRM timing requirements are not applicable anymore if UE-based mechanism is used. Moreover, proper testing of a new UE-Based solution requires the design of a new test setup, where the signals are coming to the UE from different sources, what is hardly achievable in terms in Rel-17.

Existing RRM UE timing requirements will not be applicable anymore if one-shot UL timing adjustment is allowed due to the mismatch in between timing advance information at the UE and in the network. However, the design of a new testing setup will be challenging in terms of Rel17 SHT FR2 WI.

Conclusion

In this paper, we have analyzed the challenges and necessary conditions for the triggering of network-controlled and UE-based timing adjustment mechanisms in HST FR2 scenarios when a large jump in propagation delay happens.

The following observations and proposal are made:

On the configuration of UL TA mechanisms:

- There is no need in dynamic signaling for enabling/disabling UE one-shot large UL timing adjustment. Dynamic signaling will require more profound changes in the existing specifications and larger involvement of the other WGs what is likely not possible in terms of Rel-17 HST FR2 WI.

- RAN4 to define network signaling flag to enable/disable one-shot large UL timing adjustment beyond Tq at UE.

- RAN4 to use FR2 HST scenario flag as a pre-condition for one-shot large UL timing adjustment beyond Tq at UE.

The beam management/switching algorithms are network implementation specific. It cannot be guaranteed that in bi-directional deployments the switch in between the non-collocated RRHs cannot result in a significant jump in propagation delays.

RAN4 not to exclude bi-directional deployments while considering the solution to the problem of a jump in propagation delays.

Network signaling flag to enable/disable one-shot large UL timing adjustment beyond Tq at UE should be generic, i.e., not limited to a particular FR2 HST deployment and/or scenario.

On network-controlled mechanism:

The legacy assumption that the source and target beams are QCLed is not true for non-collocated RRHs. DL synchronization to the target RRH might introduce additional delays both in the network-controlled and UE-based mechanisms.

RAN4 to discuss how DL synchronization can be established to the target RRH in the case of TCI state switches between the beams of non-collocated RRHs:

  - Option 1: The UE is capable of tracking timing of the beams of target RRH after it was detected

  - Option 2: UE needs to synchronize to the beam of target RRH after the TCI switch command is received

Network-controlled mechanism is based on already standardized procedures: TCI state switching, PDCCH Order and CF RA. Additional delay is introduced by RA but CF procedure is much faster than HO.

On UE-based mechanism:

RAN4 to consider TCI state switch as a triggering condition for the one-shot large UL timing adjustment.

RAN4 to consider UE large DL timing change above Tq as triggering condition for the one-shot large UL timing adjustment.

Introduction of network assistance, such as changes in the MAC-CE or extensive signalling of SSB-RRH mapping, is challenging to be achieve in terms of Rel-16.

Existing RRM UE timing requirements will not be applicable anymore if one-shot UL timing adjustment is allowed due to the mismatch in between timing advance information at the UE and in the network. However, the design of a new testing setup will be challenging in terms of Rel17 SHT FR2 WI.

- References

- R4-2120359, Email discussion summary for [101-e][217] NR_HST_FR2_RRM_2, Moderator (Samsung), RAN4#101-e.

- R4-2120416, WF on FR2 HST RRM requirements (part 2), Samsung, RAN4#101-e.

- R4-2201666, [Draft] LS on network signalling for Rel-17 FR2 NR HST RRM, Nokia, RAN4#101-bis-e.

- R4-2119477, On UL TA in HST FR2 Deployments, Nokia, RAN4#101-e.

- Appendix: TP to the [Draft] LS on network signaling for Rel-17 FR2 NR HST RRM [R4-2201666]

1. Overall Description:

In the Rel-17 work item on NR support for high-speed train scenarios in FR2 (NR_HST_FR2), RAN4 has agreed to specify 2 sets of enhanced NR RRM requirements for UE moving at speed up to 350 km/h. As such, network signalling is needed for indicating to UE which one of the 2 sets of the enhanced NR RRM requirements applies per cell. The per cell signalling is provided to UE in both idle mode and connected mode.

RAN4 kindly requests RAN2 to design the above-mentioned network signalling to support the 2 specified sets of enhanced NR RRM requirements for Rel-17 FR2 HST.

In the Rel-17 work item on NR support for high-speed train scenarios in FR2 (NR_HST_FR2), RAN4 has agreed that it is up to network configuration to enable one shot large uplink timing adjustment mechanism. If one shot large uplink timing adjustment is disabled, existing uplink timing adjustment, i.e., RA based mechanism, and related existing RAN4 requirements will be applied when needed. Thus, network per cell signaling is needed for indicating to UE whether one shot large UL timing adjustment is enabled or not.

RAN4 kindly requests RAN2 to design the above-mentioned network signalling to enable UE one shot large UL timing adjustment for Rel-17 FR2 HST.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly requests RAN2 to design the above-mentioned network signalling to support the specified sets of enhanced NR RRM requirements for Rel-17 FR2 HST RRM.

RAN4 kindly requests RAN2 to design the above-mentioned network signalling to enable UE one shot large UL timing adjustment for Rel-17 FR2 HST.