3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2201893

Electronic Meeting, 17 January – 25 January 2022

Source:	Ericsson, AT&T

Title:	TP for TR 38.717-04-01 to include CA_n2A-n14A-n30A-n77A

Agenda Item:	5.10.1

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-04-01 to include CA_n2A-n14A-n30A-n77A.

# 2. Text Proposal

# ---Start of changes---

## 5.X	   CA_n2-n14-n30-n77

### 5.X.1	Operating bands for CA

Table 5.X.1-1: 4DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n2-n14-n30-n77 | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 5.X.2	Channel bandwidths per operating band for CA

Table 5.X.2-1: Supported channel bandwidths per CA configuration for 4DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n14A-n30A-n77A | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n2A-n14A-n30A-n77(2A) | - | n2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.3	Co-existence studies

For single uplink, the UE coexistence is already considered in the fallback combinations in TS 38.101-1.

### 5.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n2, n14, n30 and n77, the TIB,c and RIB,c values are shown in table 5.X.4-1 and table 5.X.4-2 refer to  DC_2-14-30_n77  in 38.101-3, respectively.

Table 5.X4-1: ΔTIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n14-n30-n77 | n2 | 0.6 |
|  | n14 | 0.5 |
|  | n30 | 0.3 |
|  | n77 | 0.8 |

Table 5.X.4-2: ΔRIB,c for 4DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n14-n30-n77 | n2 | 0.2 |
|  | n14 | 0.2 |
|  | n30 | 0 |
|  | n77 | 0.5 |

### 5.X.5	REFSENS requirements

Compared to its fall-back modes, there are no additional MSD requirements for this band combination.

---End of changes---

# Reference

- RP-211759 TR 38.717-04-01, Rel-17 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL,v0.7.0