TSG-RAN Working Group 4 (Radio) meeting #101-bis-e	R4-2201916

Electronic Meeting, 17 – 25 January 2022

Source:	Ericsson

Title:	FR2-2 DC/CA with FR1 anchor

Agenda item:	8.16.6

Document for:	Discussion

# 1	Discussion

In RAN #93, the WID for Extending current NR operation to 71GHz [1] was revised to include FR2-2 DC or CA with an anchor in FR1as follows:

- For the case of FR2-2 DC or CA with an anchor in FR1 the following three example band combinations shall be considered:

  - n79 + Nx

  - n77 + Nx

  - n41 + Nx

- where Nx is the 57-71 GHz band for unlicensed operation and the [66-71] GHz for licensed operation.

- RAN4 to further discuss the need for single or multiple bands relevant for FR2-2 licensed/unlicensed operation.

In RAN4, it is agreed that when new combination for CA or DC between an FR2-1 band and an existing FR1 combinations are added, there is no need to consider any MSD, harmonics, IMDs or any other issues as the FR2-1 frequency ranges are far apart from FR1. Therefore, it has been agreed that TP’s to TR’s are not needed and that new band combinations adding a FR2-1 band can be included in the specifications by using draft CR’s.

As FR2-2 frequencies are further away from FR1 compared to FR2-1 frequencies, it is reasonable to use the same approach also for FR2-2 CA/DC combinations with an anchor in FR1. There is no need to consider any MSD, harmonics, IMDs or any other issues for FR2-2 CA or DC combinations with an anchor in FR1.

# 3	Proposal

Proposal: CA/DC combinations with FR2-2 and with an anchor in FR1 can be added directly by draft CR’s just like it is done for CA/DC FR2-1 combinations with an anchor in FR1

# 4	References

- RP-213540, “WID: Extending current NR operation to 71GHz”, Qualcomm, Intel