3GPP TGS-RAN WG4 Meeting #101-Bis-e	R4-2201926
Electronic Meeting, January 17 – 25, 2022

Agenda item:	6.16.3.2

Source:	Intel Corporation

Title:	UE EIS requirements for FR2-2

Document for:	Discussion

# 1	Introduction

UE Rx requirement discussions in FR2-2 are ongoing [1,2]. Given the expected performance degradation of this frequency range, different array sizes are being considered in the power class evaluations [2]. During RAN4 #101-e, the FWA array size was agreed to range from 32 to 64 elements [3].

Agreement: If a single power class is defined for FWA in Rel-17, the number of antenna element assumption is anywhere in the range between 32 and 64 elements.

In this paper, we discuss the minimum peak EIS of handheld and FWA UEs for different antenna array sizes.

# 2	Discussion

## 2.1	Handheld UE

For PC3, we include minimum peak EIS derivations for the baseline 4-element antenna array, as well as 8 and 16 elements. Table 1 summarizes the

### 2.1.1 Minimum peak EIS

Table 1. PC3 minimum peak EIS evaluation for FR2-2

| Parameter | Unit | 4-elements | 8-elements | 16-elements |
| --- | --- | --- | --- | --- |
| Frequency range | GHz | 52.6 - 71 | 52.6 - 71 | 52.6 - 71 |
| Modulation |  | QPSK | QPSK | QPSK |
| Bandwidth | MHz | 100 | 100 | 100 |
| SNR | dB | -1 | -1 | -1 |
| Thermal noise - 10log[(k*T*BW)/1mW] | dBm | -94 | -94 | -94 |
| Noise figure | dB | 16 | 16 | 16 |
| Effective array gain - 10log(# ant.) + element gain + roll-off | dB | 7.0 | 10 | 13 |
| Total implementation loss | dB | -12.5 | -13.0 | -13.5 |
| Peak EIS (Minimum) | dBm | -73.5 | -76.0 | -78.5 |

For 100MHz CBW, the minimum peak EIS of the 4-element array is -73.5 dBm, while the 8-element is -76.0 dBm, and the 16-element is -78.5 dBm. As detailed in [4], the 8-element array provides the most viable compromise between improving performance and integration challenges.

Observation 1: PC3 minimum peak EIS (100MHz) for a 4-element array is -73.5 dBm. For 8 and 16-element arrays, the values are -76.0 dBm, and -78.5 dBm, respectively. Given the need for improved performance and size constraints of this form factor, an 8-element antenna array is a reasonable compromise.

Proposal 1: Use an 8-element antenna array assumption for handheld UEs in FR2-2 and define the single-band minimum peak EIS as -76.0 dBm (100MHz).

## 2.2	FWA UE

Since the number of elements for this UE was agreed to range from 32 to 64 elements, two array sizes were evaluated [3]. Table 2 summarizes the achieved minimum peak EIS of the 32-element and a 64-element antenna array.

### 2.2.1 Minimum peak EIS

Table 2. PC1 minimum peak EIS evaluation for FR2-2

| Parameter | Unit | 32-elements | 64-elements |
| --- | --- | --- | --- |
| Frequency range | GHz | 52.6 - 71 | 52.6 - 71 |
| Modulation |  | QPSK | QPSK |
| Bandwidth | MHz | 100 | 100 |
| SNR | dB | -1 | -1 |
| Thermal noise - 10log[(k*T*BW)/1mW] | dBm | -94 | -94 |
| Noise figure | dB | 16 | 16 |
| Effective array gain - 10log(# ant.) + element gain + roll-off | dB | 16 | 19 |
| Total implementation loss | dB | -12.8 | -13.5 |
| Peak EIS (Minimum) | dBm | -82.2 | -84.5 |

Proposal 2: Use a 32-elment array assumption for PC1 in FR2-2 and define the minimum peak EIS single-band requirement as -82.2 dBm (100MHz).

# 3	Conclusions

In this paper we discussed the FR2-2 minimum peak EIS requirement of PC1 and PC3 while considering different antenna array sizes. Additionally, we discussed Rx spurious emissions. The following observations and proposals have been made:

Handheld UE

Observation 1: PC3 minimum peak EIS (100MHz) for a 4-element array is -73.5 dBm. For 8 and 16-element arrays, the values are -76.0 dBm, and -78.5 dBm, respectively. Given the need for improved performance and size constraints of this form factor, an 8-element antenna array is a reasonable compromise.

Proposal 1: Use an 8-element antenna array assumption for handheld UEs in FR2-2 and define the single-band minimum peak EIS as -76.0 dBm (100MHz).

FWA UE

Proposal 2: Use a 32-elment array assumption for PC1 in FR2-2 and define the minimum peak EIS single-band requirement as -82.2 dBm (100MHz).

# 4	References

- R4-2114989, “WF on 60 GHz UE RF requirements,” Qualcomm, RAN4 #100-e, August 2021

- R4-2119929, “Email discussion summary for [101-e][129] NR_ext_to_71GHz_Part_2,” Qualcomm, RAN4 #101-e, November 2021

- R4-2120062	“WF on 60GHz UE RF requirements,” Qualcomm, RAN4 #101-e, November 2021

- R4-2201925, “UE Tx requirements for 52.6 to 71 GHz,” Intel Corporation, RAN4 #101-Bis-e, January 2022