3GPP TSG-RAN WG4 Meeting #101-bis-e 	R4-2201930

Online, 17 - 25 Jan 2021

Source: 	Huawei

Title: 	TP to TS 38.106 clause 6.1 and 6.2

Agenda Item:	6.5.2

Document for:	Approval

# Introduction

Many of the technical requirements for the NR RF repeater have been agreed and it is now time to draft the TS, the sub-clauses have been distributed between different sub-authors. This paper is for the conducted transmitter characteristic general section 6.1 and the repeater output power section 6.2,

# TP to TS 38.106 v.0.0.1

< Start of changes >

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

antenna connector: connector at the conducted interface of the repeater type 1-C

Maximum carrier output power: mean power level measured within the passband at the antenna connector.

pass band: [The frequency range in which the repeater operates in with operational configuration, this frequency range can correspond to one or several consecutive nominal channels, if they are not consecutive each subset of channels shall be considered as an individual pass band,  a repeater can have one or several pass bands,  all channels within the passband(s) shall belong to a single operator or collaborating operators.]

rated output power: mean power level associated with a pass band the manufacturer has declared to be available at the antenna connector.

repeater type 1-C:	NR repeater operating at FR1 with a requirement set consisting only of conducted requirements defined at individual antenna connectors.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

Prated,out	Maximum rated output power

Prated,in	Input power intended to produce the maximum rated output power (Prated,out)

Pmax,out	Maximum carrier output power measured per antenna connector

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

< next change >

## 6.1	General

Unless otherwise stated, the conducted transmitter characteristics are specified at the antenna connector for repeater type 1-C configuration in normal operating conditions.

Requirements apply in both DL and UL unless otherwise stated.

For the DL the antenna connector on the BS side is the input and the antenna connector on the UE side is the output.

For the UL the antenna connector on the UE side is the input and the antenna connector on the BS side is the output.

## 6.2	Repeater output power

### 6.2.1	General

The repeater conducted output power requirement is at the antenna connector.

The maximum rated output power of the repeater type 1-C shall be as specified in table 6.2.1-1 and table 6.2.1-2.

Table 6.2.1-1: repeater type 1-C DL transmission classes rated output power limits for repeater classes

| repeater class | Prated,out |
| --- | --- |
| Wide Area repeater | (Note) |
| Medium Range repeater | ≤ 38 dBm |
| Local Area repeater | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,out rated output power of the Wide Area repeater |  |

Table 6.2.1-2: repeater type 1-C UL transmission classes rated output power limits for repeater classes

| repeater class | Prated,out |
| --- | --- |
| Wide Area repeater | (Note) |
| Local Area repeater | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,out rated output power of the Wide Area repeater. |  |

### 6.2.2	Minimum requirement

The requirements shall apply at maximum gain, with NR signals in the pass band of the repeater at:

The level that produce the maximum rated output power per channel (Prated,in).

The level that produce the maximum rated output power per channel (Prated,in) plus 10dB

In normal conditions, the measured output power, Pmax,out shall remain within +2 dB and -2 dB of the rated output power Prated,out, declared by the manufacturer.

In extreme conditions, the measured output power, Pmax,out shall remain within +2.5 dB and -2.5 dB of the rated output power Prated,out, declared by the manufacturer.

< End of change >