3GPP TSG-RAN WG4 Meeting #101-bis-e                                      R4-2201942

Electronic Meeting, January 17-25, 2022

Source: 	Huawei, HiSilicon

Title: 	On ULFPTx and applicable MPR requirements for different PA configurations

Agenda Item:	6.7.3.2

Document for:	Approval

# Introduction

Relationship of ULFPTx and TxD is a phase 2 issue to be addressed in the TxD WI. The discussion in last meeting can be found in the WF [1].

This contribution provides our consideration on addressing the issue.

# Discussion

There are two options are under discussion in last RAN4 meeting, i.e.:

Option 1 (R4-2114967):

Option 2 (R4-2118135):

The main point is whether we need to couple ULFPTx with TxD for the applicable requirements. From the RAN1 reply LS in [2], clearly it clarified that TxD can be implemented for both ULFPTx mode-1 and mode-2. In other words, there is no one-to-one mapping relationship for the UE implementation architectures with the ULFPTx modes. Even though, it could have typical implementation for the different modes, but the specifications in RAN4 and RAN1 should be consistent.

Observation 1: There is no one-to-one mapping relationship between the UE implementation architectures and the ULFPTx modes.

The concern for the proposal of option 2 is that UE indicated with TxD, the applicable MPR requirements would be relaxed according to the specification. However, we doubt UE will deliberately to report TxD capability just for utilizing a set of relaxed requirements. Supporting TxD also needs development, it does not come for free. As discussed above, RAN4 cannot use ULFPTx modes to distinguish UE architectures, but is it necessary to introduce new UE capability other than TxD to do so? WE think that is not necessary, as no matter which new capability, UE may still indicate TxD capability, however, it is not the normal case. For simplicity and a workable way, distinguishing the applicable requirements based on TxD indication would be enough. Option 1 is preferred.

Proposal 1: It is proposed to distinguish the applicable requirements for 2Tx implementation just based on TxD indication.

# Conclusion

This contribution provides our view on ULFPTx and applicable MPR requirements for different PA configurations.

Observation 1: There is no one-to-one mapping relationship between the UE implementation architectures and the ULFPTx modes.

Proposal 1: It is proposed to distinguish the applicable requirements for 2Tx implementation just based on TxD indication.

Further clarification of the applicable requirements in clause 6.2D as proposed in R4-2114967 is preferred by us.

# References

[1] R4-2119974, WF on ULFPTx with TxD, vivo

[2] R1-2007245, Reply LS on feasibility of UL FPTx modes and transparent TxD for certain UE implementation, Huawei