3GPP TSG-RAN WG4 Meeting #101-bis-e                                      R4-2201950

Electronic Meeting, January 17-25, 2022

Source: 	Huawei, HiSilicon

Title: 	MPR for intra-band con-current operation

Agenda Item:	6.15.3.1

Document for:	Approval

# Introduction

A WF was agreed to further consider MPR for intra-band con-current operation in this meeting [1]:

This contribution provides our analysis on addressing the issue.

# Discussion

## Uu PC3 MPR for single carrier vs intra-band UL CA

For PC3 UE, the MPR values for single carrier and intra-band UL CA are different.

- For single carrier, MPR for 1T and 2T are the same

- For intra-band UL CA, MPR for 1T and 2T are different, 2T requirements are bit relaxed

TS 38.101-1 Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | ≤ 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

TS 38.101-1 Table 6.2A.2.1-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Observation 1: For singe Tx implementation, even for CA bandwidth class B, the MPR for CA is relaxed for 0.5-1.5dB for different modulation schemes.

Observation 2: For both singe Tx and dual Tx implementation, the requirements for PC3 UE for UL CA are the same.

## Uu PC2 MPR for single carrier vs intra-band UL CA

For PC2 UE, the MPR values for single carrier and intra-band UL CA are different.

- For single carrier, MPR for 1T and 2T are different, requirements for 2T are relaxed

- For intra-band UL CA, MPR for 1T and 2T are different, 2T requirements are bit relaxed

TS 38.101-1 Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

TS 38.101-1 Table 6.2A.2.1-1a: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | QPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | 16QAM | 2.5 | 4.01 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.51 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.5 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.0 | 5.01 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5.01 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. |  |  |  |  |  |

TS 38.101-1 Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 31 | 6.5 | 13 | 31 | 7.5 | 13.5 |
|  | QPSK | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 16QAM | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 64QAM | 5 | 6.5 |  | 5 | 7.5 |  |
|  | 256QAM | 6.5 | 7 |  | 6.5 | 7.5 |  |
| CP-OFDM | QPSK | 3.51 | 7 | 14 | 3.51 | 8 | 14.5 |
|  | 16QAM | 3.51 | 7 |  | 3.51 | 8 |  |
|  | 64QAM | 5 | 7 |  | 5 | 8 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 8 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

According to WF in [3], the MPR values are bit relaxed for 23+23dBm PA configuration, i.e.

|  | CP-OFDM | DFT-s-OFDM |
| --- | --- | --- |
| contiguous inner | +0.5 | +1 |
| contiguous outer | +0.5 | +1 |
| non-contiguous inner | +0.5 | +1 |
| non-contiguous outer 1 | +1 | +1 |
| non-contiguous outer 2 | +1 | +1 |

Observation 3: For singe Tx implementation, for CA bandwidth class B and DFT-s-OFDM, the MPR for CA continuous inner RB allocation is relaxed for 0.5-2 dB for different modulation schemes, while for outer RB allocation, the relaxation is larger, i.e. 1.5-3dB.

Observation 4: MPR for non-contiguous RB allocation are larger than those for contiguous RB allocation

Observation 5: For dual Tx implementation, the requirements for PC2 UE are further relaxed.

## V2X MPR for single carrier vs con-current operation

### Tentative values in the TR for con-current operation

#### PC3 MPR for single carrier vs intra-band con-current operation

Tables below list the MPR for PC3 NR V2X and tentative values for intra-band con-current operation.

TS 38.101-1 Table 6.2E.2.2-1: Maximum Power Reduction (MPR) for power class 3 NR V2X

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 2.5 |
|  | 16QAM | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.0 |  |

TR 38.785 Table 5.2.4.2.2-3: Contiguous RB allocation for Power Class 3

| Modulation | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  |  | inner | outer |  |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.0] |  |
|  | 16QAM | ≤ [1.0] | ≤ [1.0] |  |
|  | 64QAM | ≤ [1.0] | ≤ [1.0] |  |
|  | 256QAM | ≤ [1.5] | ≤ [1.5] |  |

TR 38.785 Table 5.2.4.2.2-4: Non-contiguous RB allocation for Power Class 3

| Modulation | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 64QAM | ≤ [1.0] | ≤ [1.5] | ≤ [3.0] |
|  | 256QAM | ≤ [1.5] | ≤ [2.0] | ≤ [3.0] |

#### PC2 MPR for single carrier vs intra-band con-current operation

Tables below list the MPR for PC2 NR V2X and tentative values for intra-band con-current operation.

TR 38.785 Table 5.1.2.2.1-1: Maximum Power Reduction (MPR) for power class 2 V2X (Contiguous PSCCH and PSSCH transmission)

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 2.5 |
|  | 16QAM |  |  |
|  | 64 QAM | ≤ 6.0 | ≤ 4.5 |
|  | 256 QAM | ≤ 7.0 |  |

TR 38.785 Table 5.2.4.2.2-5: Contiguous RB allocation for Power Class 2

| Modulation | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  |  | inner | outer |  |
| CP-OFDM | QPSK | ≤ [1.5] | ≤ [3.5] |  |
|  | 16QAM | ≤ [2.0] | ≤ [3.5] |  |
|  | 64QAM | ≤ [3.0] | ≤ [3.5] |  |
|  | 256QAM | ≤ [5.0] | ≤ [5.0] |  |

TR 38.785 Table 5.2.4.2.2-6: Non-contiguous RB allocation for Power Class 2

| Modulation | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  |  | inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [4.0] | ≤ [6.0] |
|  | 16QAM | ≤ [2.5] | ≤ [4.0] | ≤ [6.0] |
|  | 64QAM | ≤ [3.5] | ≤ [4.5] | ≤ [6.0] |
|  | 256QAM | ≤ [4.5] | ≤ [5.0] | ≤ [6.0] |

#### Observations on MPR for intra-band con-current operation

Observation 6: It is observed that the PC3 and PC2 MPR values for intra-band con-current operation are based on simulation assumptions in TR section 5.2.4.2.2. While for 2Tx requirements no matter for single carrier or UL intra-band CA, the requirements are mainly based on measurement, in which the reverse IMD impact has been considered.

Observation 7: MPR for the proposed intra-band con-current operation are quite smaller than those for single carrier, even the values are based on 2Tx assumptions.

### Further analysis and proposal

Though we appreciate the inputs for the simulation results, compare to MPR Uu single carrier as well as UL intra-band CA, we feel the values for intra-band con-current operation need to be revised. Based on our evaluation, we think that additional 0.5~1.0dB relaxation based on PC3 and PC2 MPR values for NR V2X single carrier should be considered. The proposed MPR values for intra-band con-current are listed in table 2.3.2-1/2 for PC3 and PC2 respectively.

Table 2.3.2-1: MPR for contiguous RB allocation for PC3 intra-band con-current operation

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations | Outer RB allocations |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64 QAM | ≤ 5.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.5 |  |

Table 2.3.2-2: MPR for non-contiguous RB allocation for PC3 intra-band con-current operation

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.5 |
|  | 64QAM | ≤ 5.0 | ≤ 6.5 |
|  | 256QAM | ≤ 8.0 |  |

Table 2.3.2-3: MPR for contiguous RB allocation for PC2 intra-band con-current operation

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Inner RB allocations | Outer RB allocations |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 6.0 |
|  | 16QAM |  |  |
|  | 64 QAM | ≤ 5.0 | ≤ 6.5 |
|  | 256 QAM | ≤ 7.5 |  |

Table 2.3.2-4: MPR for non-contiguous RB allocation for PC2 intra-band con-current operation

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 6.0 |
|  | 16QAM | ≤ 3.0 | ≤ 6.5 |
|  | 64QAM | ≤ 5.0 | ≤ 7.0 |
|  | 256QAM | ≤ 8.0 |  |

Proposal 1: It is proposed to agree the MPR values for PC3 and PC2 NR V2X intra-band con-current operation

# Conclusion

This contribution provides our further analysis on MPR for PC3 and PC2 NR V2X intra-band con-current operation. Based on the comparison with Uu single carrier as well as intra-band UL CA for both 1Tx and 2Tx, and further evaluation, we have the following observations and proposal.

Observation 1: For singe Tx implementation, even for CA bandwidth class B, the MPR for CA is relaxed for 0.5-1.5dB for different modulation schemes.

Observation 2: For both singe Tx and dual Tx implementation, the requirements for PC3 UE for UL CA are the same.

Observation 3: For singe Tx implementation, for CA bandwidth class B and DFT-s-OFDM, the MPR for CA continuous inner RB allocation is relaxed for 0.5-2 dB for different modulation schemes, while for outer RB allocation, the relaxation is larger, i.e. 1.5-3dB.

Observation 4: MPR for non-contiguous RB allocation are larger than those for contiguous RB allocation

Observation 5: For dual Tx implementation, the requirements for PC2 UE are further relaxed.

Observation 6: It is observed that the PC3 and PC2 MPR values for intra-band con-current operation are based on simulation assumptions in TR section 5.2.4.2.2. While for 2Tx requirements no matter for single carrier or UL intra-band CA, the requirements are mainly based on measurement, in which the reverse IMD impact has been considered.

Observation 7: MPR for the proposed intra-band con-current operation are quite smaller than those for single carrier, even the values are based on 2Tx assumptions.

Proposal 1: It is proposed to agree the MPR values for PC3 and PC2 NR V2X intra-band con-current operation in Tables 2.3.2-1 to 2.3.2-4.

# References

[1] R4-2119989, WF on MPR for intra-band V2X con-current operation, Huawei, HiSilicon, LG Electronics

[2] R4-2119954, WF on PC2 intra-band contiguous UL CA with UL MIMO, OPPO

[3] R4-2118081, TR 38.785 V0.4.0, LG Electronics.