3GPP TSG-RAN WG4 Meeting #101bis-e                                      R4-2201959

Electronic Meeting, January 17-25, 2022

Source: 	Huawei, HiSilicon

Title: 	Further study on DC location reporting

Agenda Item:	6.4.4

Document for:	Approval

# Introduction

During last RAN4 e-meeting, intensive discussion as recorded in [1] has been done with the outcome of detailed framework on the DC location reporting, which is informed to RAN2 in the agreed LS [2]. However, there are still some FFS points which can be found in the approved WF [3]. In this contribution, our further analysis and opinions will be provided.

# Discussion

As captured in the WF, the offset range still needs to be discussed:

Issue 1-2-4: What is the offset range? - Proposals - Option 1: restrict UE DC offset ranges for example +/-20MHz from the default DC location - Option 2: Largest offset is 1.5 GHz i.e. +/- 25000 sub carriers in a 60 kHz grid. - Option 3: Others. - Recommended WF - TBA Conclusion: FFS.

According to the definition of frequency separation class, a UE can indicate the maximum frequency span between lower edge of lowest component carrier and upper edge of highest component carrier per band combination. Taking V17.3.0 as an example, the maximum frequency span for intra-band non-contiguous UL CA is [600MHz] for FR1 or 2400MHz for FR2, respectively. Thus the current spec doesn’t support 3GHz frequency span introduced by the 1.5GHz offset, which is a spec impact.

Observation: Currently the specified maximum frequency span for intra-band non-contiguous UL CA is [600MHz] for FR1 or 2400MHz for FR2, respectively. Thus 3GHz frequency span introduced by the 1.5GHz offset is not supported, which is a spec impact.

From the implementation perspective, non-zero DC location shift could be caused by temperature impacts to the UE hardware. Normally, several subcarriers or RBs are enough to cover the shift. So the 1.5GHz range seems a waste to the signalling design, which is against the intention of this solution. Consequently, we think reuse the bit length of the original signalling is an appropriate solution, i.e., 12 bits. The mapping can be found as below.

| Bit sequence(12 bits) | DC location offset (Subcarrier index) corresponding to the smallest numerology in the configured or activated CCs/BWPs |
| --- | --- |
| 0000 0000 0000 | 0 |
| 0000 0000 0001 | 1 |
| …… | …… |
| 0111 1111 1111 | 2047 |
| 1000 0000 0000 | reserved |
| 1000 0000 0001 | -1 |
| …… | ..…. |
| 1111 1111 1110 | -2046 |
| 1111 1111 1111 | -2047 |

Proposal: Reuse the 12 bit length of the original signalling for DC location offset report > 2CCs and adopt the following mapping relationship.

| Bit sequence(12 bits) | DC location offset (Subcarrier index) corresponding to the smallest numerology in the configured or activated CCs/BWPs |
| --- | --- |
| 0000 0000 0000 | 0 |
| 0000 0000 0001 | 1 |
| …… | …… |
| 0111 1111 1111 | 2047 |
| 1000 0000 0000 | reserved |
| 1000 0000 0001 | -1 |
| …… | ..…. |
| 1111 1111 1110 | -2046 |
| 1111 1111 1111 | -2047 |

# Conclusion

In this contribution we discussed on the remaining issue on DC location reporting, we have the following observation and proposal:

Observation: Currently the specified maximum frequency span for intra-band non-contiguous UL CA is [600MHz] for FR1 or 2400MHz for FR2, respectively. Thus 3GHz frequency span introduced by the 1.5GHz offset is not supported, which is a spec impact.

Proposal: Reuse the 12 bit length of the original signalling for DC location offset report > 2CCs and adopt the following mapping relationship.

| Bit sequence(12 bits) | DC location offset (Subcarrier index) corresponding to the smallest numerology in the configured or activated CCs/BWPs |
| --- | --- |
| 0000 0000 0000 | 0 |
| 0000 0000 0001 | 1 |
| …… | …… |
| 0111 1111 1111 | 2047 |
| 1000 0000 0000 | reserved |
| 1000 0000 0001 | -1 |
| …… | ..…. |
| 1111 1111 1110 | -2046 |
| 1111 1111 1111 | -2047 |

# References

- R4-2119922, “Email discussion on suammay for [101-e][122] NR_RF_FR2_enh2_Part_3”, Moderator (Vivo), RAN4 #101-e, Electronic meeting, January 17-25, 2022.

- R4-2119965, “LS on DC location for >2CC”, Qualcomm, RAN4 #101-e, Electronic meeting, January 17-25, 2022.

- R4-2119964, “WF on DC-Location”, Vivo, RAN4 #101-e, Electronic meeting, January 17-25, 2022.