3GPP TSG-RAN WG4 Meeting#101-bis-e	R4-2201966

Online, 17th Jan – 25th Jan, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on Intra-cell Inter-user Interference Scenarios

Agenda item:	6.12.2.2

Document for:	Discussion

- Introduction

In RAN4#101-e, WF [1] was agreed and it still had some open issues. In this paper, we provide our views on those open issues for evaluating intra-cell inter-user interference scenarios.

- Test Parameters and Simulation Results

Precoding

It is more practical to choose orthogonal precoding for different UEs. Some companies already showed in the last meeting that random precoding can cause severe performance degradation. Therefore, we propose the following.

Proposal 1: Use random PMI selection for the target UE, and select the precoder for the interference UE to ensure orthogonality.

DMRS Scrambling ID

For MMSE-IRC receiver, it does not matter whether we use same or different scrambling ID for co-scheduled UEs. However, it will be more complex on network side to use different scrambling IDs for different co-scheduled UEs. Also, RAN4 is considering enhanced receivers for Rel-18 where UE will have to perform joint channel estimation of co-scheduled UEs, which will need the assumption of same scrambling ID. For fair comparison and to show gains compared to MMSE-IRC receiver in Rel-17, it will be better to use same scrambling ID for Rel-17 requirements as well. Therefore, we propose the following.

Proposal 2: Use same DMRS scrambling ID for all co-scheduled UEs.

Simulation Results

Table 2: SNR in dB at 70% of peak throughput assuming orthogonal precoding

| Case | Configuration | MMSE-MRC | MMSE-IRC |
| --- | --- | --- | --- |
| FDD: 10 MHz, 15kHz, TDLC300-100 ULA Low, 2x2, MCS13, Rank1 | 1+1 Same CDM Group |  |  |
| FDD: 10 MHz, 15kHz, TDLC300-100 ULA Low, 2x4, MCS13, Rank1 | 1+1 Same CDM Group |  |  |
| FDD: 10 MHz, 15kHz, TDLC300-100 ULA Low, 2x2, MCS13, Rank1 | 1+1 Different CDM Group | Inf | 14.05 |
| FDD: 10 MHz, 15kHz, TDLC300-100 ULA Low, 2x4, MCS13, Rank1 | 1+1 Different CDM Group | 12.42 | 6.83 |
| FDD: 10 MHz, 15kHz, TDLA30-10 ULA Low, 4x4, MCS13, Rank2 | 2+2 | Inf | 12.59 |

- Conclusions

This paper provides our views on open issues for evaluating intra-cell inter-user interference scenarios in RAN4. Following has been observed and proposed.

Proposal 1: Use random PMI selection for the target UE, and select the precoder for the interference UE to ensure orthogonality.

Proposal 2: Use same DMRS scrambling ID for all co-scheduled UEs.

# References

[1] R4-2120709, “Way Forward on MMSE-IRC for intra-cell inter-user interference”, 3GPP TSG RAN WG4 Meeting #101-e, Online, Nov 1 - 12, 2021.