3GPP TSG-RAN WG4 Meeting # 101bis-e 	R4-2201979

# Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on PBCH simulation assumptions for RedCap

Agenda item:	6.20.3.1.1

Document for:	Discussion

Introduction

At RAN4 #101-e, the discussion on RRM requirements for the WI on reduced capability NR (RedCap) devices [1] was continued [2]. Agreements and open issues on this matter were captured in the WF [3].

The simulation assumptions for RedCap RRM were confirmed including those for PBCH detection based on 1 Rx reception [4] to evaluate the basic SI reading (PBCH acquisition) time and the DMRS time index reading time.

This paper raises a need for clarification of the PBCH detection method, of the definitions for PBCH reading time and  DMRS time index reading time as well on the settings of PSS/SSS/PBCH sequence  parameters as listed in [4].

Discussion

The simulation assumptions for PBCH detection based on 1 Rx listed in Table 1 of [4] are replicated below.

Table 1: Parameters for basic SI reading (PBCH acquisition)

| Parameter | Unit | Value |
| --- | --- | --- |
| Carrier frequency | GHz | 4GHz; 30GHz |
| Subcarrier spacing | kHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz |
| Number of Tx antennas | - | 1 Assuming only 1Tx port is used |
| Number of Rx antennas | - | 1 |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 |
| SS burst set periodicity | ms | 20 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| RB Utilization | % | 100 |
| PBCH symbols within the SS block |  | PSS-PBCH-SSS-PBCH |
| Data and Control Power offset with respect to PSS and SSS | dB | 0 |
| PBCH power offset with respect to PBCH-DMRS | dB | 0 |
| PBCH-DMRS power offset with respect to PSS and SSS | dB | 0 |
| PSS and SSS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS sequences | - | According to the RAN1 agreements |
| PBCH-DMRS RE positions within the PBCH resource | - | According to the RAN1 agreements |
| PBCH Channel coding |  | Polar code with 512 length and 24bit DCRC |
| PBCH Modulation | - | QPSK |
| PBCH Payload (including the CRC) | bits | 56bit (CRC 24bit) |
| PBCH SNR | dB | -10: 0 dB, at least including -6dB, -8 dB |
| Propagation Condition | - | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns FR2: AWGN TDL-A 30ns |
| UE speed |  | FR1: 30 km/h FR2: 3 km/h |
| Detection Method |  | One shot detection (No combination for different PBCHs) |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |

The performance metric [4] is replicated below.

The proposed simulation assumptions are used to derive the basic SI reading (PBCH acquisition) time. Case1: PBCH reading Basic SI reading time = the number of SS-blocks required for 99% to successfully decode the PBCH. Case2: PBCH-DMRS time index reading (Without decode PBCH data) DMRS time index reading time = the number of SS-blocks required for 99% to successfully reading DMRS time index.

Detection method

According to Table 1, the detection method is described as “one shot detection without combination for different PBCHs”.

In our view, a more detailed description is needed here:

- Considering one shot detection, we assume that no soft combining between different PBCH instances is performed both for case 1 (PBCH reading) and case 2 (DMRS time index reading).

- We assume that a decoding attempt for case 1 and case 2 refers to a single PBCH instance only.

- We further assume, that PBCH detection is performed with the side condition of a given false alarm rate, e.g. Pfa = 1% .

- We further assume that no BLER performance target is set as side condition for PBCH detection.

Clarification of the detection method for PBCH reading and DMRS time index reading in the PBCH simulation assumptions is required.

PBCH reading time and DMRS time index reading time

In the above performance metrics, the PBCH reading time / DMRS time index reading time are defined as the number of subsequent SS-blocks required for 99% successful decoding/reading attempts.

In our view, a more detailed description is needed here:

- As SS/PBCH blocks are separated by SS burst set periodicity of 20 ms, this time interval needs to be multiplied with the number of subsequent attempts to achieve 99% successful PBCH decoding / DMRS time index reading.

- However, the time to receive the first PBCH instance needs to be fixed as well. We refer to PBCH  simulation assumptions discussed at RAN4 NR Ad-Hoc#3 (2017) in [5]:

It is important to note that, although multiple SS blocks can be configured within one single SS burst set periodicity, a blind UE may not assume hearing from another SS block until the next burst set. Thus we can define the mapping between worst PBCH acquisition time t and expected number n of needed PBCH acquisition attempts as , in which stands for the time spent until the first PBCH is acquired and stands for SS burst set periodicity. Figure 1 PBCH reading time

- The time to receive the first PBCH instance Tr is ambiguous. Since PBCH may be transmitted in the worst case in the last slot, we assume this time to be 5 ms (as listed in [5].

- Thus, the PBCH reading time / DMRS time index reading time will be:

t = 5 ms + n (99%) * 20 ms.

Clarification of the definition for PBCH reading time and DMRS time index reading time in the PBCH simulation assumptions is required.

Other comments

According to Table 1, following parameters should be selected “according to the RAN1 agreements”:

- PSS and SSS sequences

- PBCH-DMRS sequences

- PBCH-DMRS RE positions within the PBCH resource

The parameter setting is unclear and needs clarificatiom. Either a reference to RAN1 agreements should be provided or the parameters should be explicitely specified.

Clarification of the settings for PSS and SSS sequences, PBCH-DMRS sequences and PBCH-DMRS RE positions within the PBCH resource in the PBCH simulation assumptions is required.

Conclusion

This contribution raises a need for clarification of the PBCH detection method, of the definitions for PBCH reading time and DMRS time index reading time as well on the settings of PSS/SSS/PBCH sequence  parameters as listed in the simulation assumptions for RedCap PBCH detection based on 1 Rx reception [4].

The following proposals are made:

- Clarification of the detection method for PBCH reading and DMRS time index reading in the PBCH simulation assumptions is required.

Clarification of the definition for PBCH reading time and DMRS time index reading time in the PBCH simulation assumptions is required.

Clarification of the settings for PSS and SSS sequences, PBCH-DMRS sequences and PBCH-DMRS RE positions within the PBCH resource in the PBCH simulation assumptions is required.

- References

- RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson, RAN #92e

- R4-2120374, “Email discussion summary for [101-e][232] NR_redcap_RRM_1”, Moderator (Ericsson)

- R4-2120418, “WF on RedCap RRM requirements”, Ericsson

- R4-2115363, “Simulation assumptions for RedCap PBCH detection”, vivo

- R4-1709649, “ Link level simulation results for PBCH acquisition”, Huawei, HiSilicon