3GPP TSG-RAN WG4 Meeting # 101bis-e 	R4-2201984

# Electronic Meeting, January 17-25, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on BS demodulation requirements for Additional enhancements for NB-IoT

Agenda item:	8.9.6.2.2

Document for:	Discussion

Introduction

The Rel-17 work item for additional enhancements for NB-IoT and LTE-MTC was approved at TSG RAN#88-e [1]. At RAN4 #101bis-e, the technical discussion on demodulation requirements for this WI is started.

The WID1 [1] lists following objective for NB-IoT involving RAN4 effort:

- Specify 16-QAM for unicast in UL and DL, including necessary changes to DL power allocation for NPDSCH and DL TBS. This is to be specified without a new NB-IoT UE category. For DL, increase in maximum TBS of e.g. 2x the Rel-16 maximum, and soft buffer size will be specified by modifying at least existing Category NB2. For UL, the maximum TBS is not increased. [NB-IoT] [RAN1, RAN4] - Extend the NB-IoT channel quality reporting based on the framework of Rel-14—16, to support 16-QAM in DL. [NB-IoT] [RAN2, RAN1, RAN4] - Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements.

This contribution provides our view on BS demodulation requirements for NB-IoT 16QAM.

Discussion

16QAM support in RAN1

RAN1 has completed the PHY design on 16QAM for unicast in UL and DL [2]. No outstanding issue is known that would impact FRC channel selection for BS demodulation requirements.

16QAM BS RF requirements

BS RF performance requirements for 16QAM are almost complete. There is one open issue whether separate rated output power declaration for 16QAM may be made by vendors. It was agreed at RAN4 #101-e not to define 16QAM FRCs for BS receiver dynamic range tests [3]:

- New 16QAM FRCs for BS Rx dynamic range tests are not needed if they’re added for demodulation tests.

16QAM BS demodulation requirements

Regarding BS demodulation requirements, requirements for NPUSCH format 1 need to be defined.

Proposed new FRCs

We propose to reuse FRCs that are being discussed for verifying the receiver dynamic range in the RF session [4] for specifying BS demodulation requirements for NPUSCH format 1 for 3, 6 and 12 subcarriers with SCS=15 kHz.

The UL TBS agreed in RAN1 for 16QAM in NB-IoT uplink is provided in table 1 below.

Table 1: TBS indices are introduced for 16QAM in NB-IoT uplink

| I_TBS | I_RU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 |  |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 |  |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2536 |  |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 |  |  |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 |  |  |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2536 |  |  |
| 20 | 440 | 904 | 1384 | 1864 | 2344 |  |  |  |
| 21 | 488 | 1000 | 1480 | 1992 | 2536 |  |  |  |

It has also been agreed in RAN1 that repetition is not used for 16QAM in NB-IoT uplink, where it can be used for 3, 6 and 12 subcarriers NPUSCH format 1. Note that 16QAM cannot be used for 1 subcarrier NPUSCH format 1. Considering the agreements in RAN1, and to ensure satisfactory BS receiver performance with 3, 6 and 12 subcarriers for 16QAM in NB-IoT uplink, it is proposed to define the following new uplink FRCs for 16QAM for the BS demodulation requirements as shown in table 2 below.

Table 2: FRC parameters for 16QAM in NB-IoT uplink

| Reference channel | Ax-y | Ax-y | Ax-y |
| --- | --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 15 | 15 |
| Number of tones | 3 | 6 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM |
| Diversity | No | No | No |
| Frequency offset | 0 | 0 | 0 |
| I_TBS / I_RU | 19 / 3 | 19 / 3 | 19 / 3 |
| Payload size (bits) | 1736 | 1736 | 1736 |
| Allocated resource units | 4 | 4 | 4 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Coding rate (target) | 3/4 | 3/4 | 3/4 |
| Coding Rate | 0.76 | 0.76 | 0.76 |
| Code block CRC size (bits) | 0 | 0 | 0 |
| Number of code blocks – C | 1 | 1 | 1 |
| Total symbols per resource unit | 144 | 144 | 144 |
| Total number of bits per resource unit | 576 | 576 | 576 |
| Tx time (ms) | 16 | 8 | 4 |
| Frequency offset | 0 | 0 | 0 |
| Channel estimation length (ms) Note 1 | 12 | 8 | 4 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |

The following proposal is made.

To define the new uplink FRCs for 16QAM NPUSCH format 1 demodulation requirements in TS 36.104 as shown in table 2.

Preliminary Simulation Results

Preliminary link-level simulation has been performed in AWGN channel using the FRC parameters in table 2 above, and the simulation results for the SNIR required at 70% and 95% of maximum throughput are provided in Figure 1 below. It can be seen from the figure that the required SNIR are quite close for the different number of tones. Further simulation results, e.g., in fading channels, can be provided when the simulation parameters are agreed.

(a) 3 tones

(b) 6 tones

(c) 12 tones

Figure 1: SNIR required at 70% and 95% of maximum throughput for 16QAM in NB-IoT uplink

Propagation channel

For the propagation channel, it is proposed to align this to existing NPUSCH format 1 requirements in TS 36.104, which is to specify the performance for ETU 1Hz Low propagation channel.

The following proposal is made.

To select ETU 1Hz Low propagation channel for aligning to existing NPUSCH format 1 demodulation requirements in TS 36.104.

Conclusion

This contribution has provided our view on BS demodulation requirements for NB-IoT 16QAM.

The following proposals are submitted for discussion:

- To define the new uplink FRCs for 16QAM NPUSCH format 1 demodulation requirements in TS 36.104 as shown in table 2.

To select ETU 1Hz Low propagation channel for aligning to existing NPUSCH format 1 demodulation requirements in TS 36.104.

References

- RP-201306, WID revision: Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN #88e

- RP-213365, Status report for WI: Additional enhancements for NB-IoT and LTE-MTC, RAN #94e

- R4-2119948, WF on remaining issues for NB-IoT 16QAM, Huawei, HiSilicon, RAN4 #101-e

- R4-2117250, Proposals on BS RF requirements for support of 16QAM in NB-IoT, Nokia, Nokia Shanghai Bell, RAN4 #101-e