3GPP TSG-RAN WG4 Meeting # 101bis-e 	R4-2201989

# Electronic Meeting, 17th – 25th January, 2022

Title:	NTN system parameters remaining issues for n255

Agenda Item:	6.13.1.1

Source:	MediaTek Inc.

Document for:		Discussion

# Introduction

This document discusses potential ways forward for the remaining open issues on n255 band system parameters.

# Discussion

Channel spacing requirements

For n256, it was agreed for the channel spacing requirements to follow the TN requirements in section 5.4.1 of 38.101-1. We do not see any reason as to why this cannot also be applied for n255. Therefore, we propose that n255 channel spacing also follows the TN requirements in section 5.4.1 of 38.101-1.

Channel raster and NREF range

The channel raster for n256 has been agreed to be 100kHz. For n255 we propose to use the same channel raster value as n256. This would lead to the NREF range (same as for n24) below:

| NTN satellite band # | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |

SS block configuration and raster entries

For n24, the SS block and GSCN configurations are defined in the table below.

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n24 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |

For n256, only the 15kHz SS block configuration was agreed. Following the same approach for n255 would reduce UE cell search effort and time, but may cause spectrum inefficiency in case 30kHz SCS was used for other physical channels. We welcome feedback on whether to follow the n24 approach or the n256 approach for the SSB SCS.

# Proposal

Discuss and, if acceptable, agree to the points in bold in this contribution for n255 remaining system parameter issues.