3GPP TSG-RAN WG4 Meeting#101-bis-e	R4-2202006

Online, 17th Jan – 25th Jan, 2022

Source: 	Qualcomm Incorporated

Title: 	Views on FR1 HST PDSCH CA Tests

Agenda item:	6.8.3.2

Document for:	Discussion

- Introduction

In RAN4#99-e, simulation assumptions were agreed in [1] for defining HST requirements for CA scenarios. There were also some open issues mentioned in [2]. In this paper, we provide our views on FR1 HST PDSCH CA test cases.

- Release Independence

From UE implementation perspective, we don’t see any difficulty in extending the demod requirements for CA. Therefore, we propose the following.

Proposal 1: Use same rule for release independence of demod requirements, as agreed in RRM requirements.

- HST-SFN Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below.

Table 1: HST-SFN Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 9.91 | 12.41 | 7.66 | 10.16 |
| 10 | 10.05 | 12.55 | 8.04 | 10.54 |
| 15 | 10.37 | 12.87 | 8.46 | 10.96 |
| 20 | 10.59 | 13.09 | 8.67 | 11.17 |
| 25 | 10.68 | 13.18 | 8.77 | 11.27 |
| 30 | 10.95 | 13.45 | 8.97 | 11.47 |
| 40 | 10.94 | 13.44 | 8.97 | 11.47 |
| 50 | 10.92 | 13.42 | 8.98 | 11.48 |

Table 2: HST-SFN Simulation results for TDD 30kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 11.05 | 13.55 | 9.33 | 11.83 |
| 10 | 11.74 | 14.24 | 9.62 | 12.63 |
| 15 | 11.5 | 14 | 9.63 | 12.37 |
| 20 | 11.29 | 13.79 | 9.6 | 12.1 |
| 25 | 11.55 | 14.05 | 9.87 | 12.37 |
| 30 | 11.81 | 14.31 | 10.07 | 12.57 |
| 40 | 11.91 | 14.41 | 10.06 | 12.56 |
| 50 | 12.02 | 14.52 | 10.17 | 12.67 |
| 60 | 12.09 | 14.59 | 10.27 | 12.77 |
| 80 | 12.16 | 14.66 | 10.44 | 12.94 |
| 90 | 12.29 | 14.79 | 10.45 | 12.95 |
| 100 | 12.15 | 14.65 | 10.31 | 12.81 |

- HST-DPS Simulation Results

Based on simulation assumptions in [1], we provide the simulation results below.

Table 3: HST-DPS Simulation results for FDD 15kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 10.41 | 12.41 | 7.65 | 9.65 |
| 10 | 10.64 | 12.64 | 7.82 | 9.82 |
| 15 | 10.64 | 12.64 | 7.75 | 9.75 |
| 20 | 10.20 | 12.20 | 7.41 | 9.41 |
| 25 | 10.62 | 12.62 | 7.72 | 9.72 |
| 30 | 10.70 | 12.70 | 7.76 | 9.76 |
| 40 | 10.56 | 12.56 | 7.79 | 9.79 |
| 50 | 10.86 | 12.86 | 7.89 | 9.89 |

Table 4: HST-DPS Simulation results for TDD 30kHz

|  | 2x2 |  | 2x4 |  |
| --- | --- | --- | --- | --- |
| CBW (MHz) | Alignment SNR (dB) | Impairment SNR (dB) | Alignment SNR (dB) | Impairment SNR (dB) |
| 5 | 10.37 | 12.37 | 7.59 | 9.59 |
| 10 | 10.63 | 12.63 | 7.80 | 9.80 |
| 15 | 10.51 | 12.51 | 7.76 | 9.76 |
| 20 | 10.43 | 12.43 | 7.71 | 9.71 |
| 25 | 10.51 | 12.51 | 7.75 | 9.75 |
| 30 | 10.38 | 12.38 | 7.65 | 9.65 |
| 40 | 10.32 | 12.32 | 7.62 | 9.62 |
| 50 | 10.68 | 12.68 | 7.80 | 9.80 |
| 60 | 10.64 | 12.64 | 7.77 | 9.77 |
| 80 | 10.53 | 12.53 | 7.75 | 9.75 |
| 90 | 10.78 | 12.78 | 7.67 | 9.67 |
| 100 | 10.60 | 12.60 | 7.80 | 9.80 |

- Conclusions

This paper provides our views on release independence and simulation results for FR1 HST PDSCH CA requirements. Following has been proposed.

Proposal 1: Use same rule for release independence of demod requirements, as agreed in RRM requirements.

# References

[1] R4-2108635, “WF on UE demodulation for FR1 HST”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19 – May 27, 2021.

[2] R4-2120700, “WF on FR1 HST demodulation”, 3GPP TSG RAN WG4 Meeting #101-e, Online, Nov 1 - 12, 2021.