3GPP TSG-RAN WG4 Meeting # 101-bis-e	R4-2202024

# Electronic Meeting, January 17-25, 2022

Source: 	Huawei

Title: 	Further discussion on the BS RF requirements

Agenda Item:	8.8.2

Document for:	Discussion

# Introduction

Last meeting, we have provided initial analysis of the BS RF related aspects in [2]. Two proposals were formulated:

Proposal 1: based on the analysis of co-lo / co-ex requirements for the 700MHz NAM bands in EUTRA spec, initiate the work to specify related requirements also for the new 700MHz band.

Proposal 2: confirm that co-lo / co-ex requirements are applicable to Upper_700_MHz_A_LTE band, so that [] can be removed from the WID during next RAN meeting.

Based on the discussion in [3], there was no consensus reached on the BS co-location and BS co-existence requirements. At later state of RAN4#101-e meeting, a WF was proposed (not approved due to formal reasons), with the following proposals:

It shall be noted, that the above presented tables 1 and 2 are reflecting technical discussion and proposals from our contribution in [2]. Therefore in order to re-confirm our proposal from the previous RAN4 meeting, an updated version of contribution in [2] is re-submitted.

In this contribution we provide updated analysis of the co-location and co-ex requirements for BS RF.

# Discussion

Referring to the EUTRA spec, the following 700MHz range bands are already defined and applicable to NAM region:

Table 1 E-UTRA frequency bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode | Region /market |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD | Canada, USA |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD | Canada, USA |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD | USA |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | FDD | Canada, USA |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD | Canada, USA |
| 85 | 698 MHz – 716 MHz |  |  | 728 MHz | – | 746 MHz | FDD |  |
| Upper_700_MHz_A_LTE | 787 MHz – 788 MHz |  |  | 757 MHz | – | 758 MHz | FDD |  |

One of the objectives of the WID [1] was to study the need for co-existence and/or co-location requirements:

Study the need for any applicable co-location and/or co-existence requirements for the 700MHz NB-IoT band, considering regional regulations, and if needed specify the co-existence requirements

In general, whenever new bands are introduced, the following needs to be analysed (in relation to the co-location and co-existence requirements). For that reason, multiple BS RF specifications were listed in [] in the WID [1].

| Affected existing specifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |
| 36.101 |  | E-UTRA; UE Radio transmission and reception | RAN#95 Mar. 2022 | Core part |
| 36.104 |  | E-UTRA; Base Station (BS) radio transmission and reception | Same as above | Core part |
| 36.133 |  | E-UTRA; Requirements for support of RRM | Same as above | Core part |
| 36.141 |  | E-UTRA; Base Station (BS) conformance testing | Same as above | Perf part |
| [37.104] |  | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception | Same as above | Core part (for co-existence) |
| [37.105] |  | Active Antenna System (AAS) Base Station (BS) transmission and reception | Same as above | Core part (for co-existence) |
| [37.141] |  | E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing | Same as above | Perf part (for co-existence) |
| [38.101-1] |  | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | Same as above | Core part (for co-existence) |
| [37.145-1] |  | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: conducted conformance testing | Same as above | Perf part (for co-existence) |
| [37.145-2] |  | Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing | Same as above | Perf part (for co-existence) |
| [38.104] |  | NR; Base Station (BS) radio transmission and reception | Same as above | Core part (for co-existence) |
| [38.141-1] |  | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | Same as above | Perf part (for co-existence) |

Below we briefly analyse the 700MHz bands and related co-ex/co-location requirements.

1. Co-location requirements are RAN4 derived requirements (as opposed to the regulatory requirements). They are defined as:

- TX spurious emissions co-location requirements: These requirements may be applied for the protection of other BS receivers, i.e. the intention is to protect other BS from potential spur emissions from “our” BS. BS RF spec defined the frequency range over which that requirement applies. Those requirements are specified in section 6.6.4.4 of TS 36.104.

Table 2: Tx spur co-lo requirements for NAM 700MHz bands [TS 36.104]

| Type of co-located BS | Frequency range for co-location requirement | WA BS Maximum Level | MR BS Maximum Level | LA BS Maximum Level | Measurement Bandwidth |
| --- | --- | --- | --- | --- | --- |
| WA UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 699 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |
| WA UTRA FDD Band XIII or E-UTRA Band 13 or NR Band n13 | 777 - 787 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |
| WA UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 788 - 798 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |
| WA E-UTRA Band 17 | 704 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |
| WA E-UTRA Band 85 or NR band n85 | 698 - 716 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |

Please note that the levels are the same for all bands.

- RX co-location blocking: This additional blocking requirement may be applied for the protection of “our” BS receivers, when co-located with other BS. Those requirements are specified in section 7.6.2 of TS 36.104.

2. Co-existence requirements are regulatory requirements. They are defined as:

- Additional TX spurious emissions requirements: those requirements represent co-existence requirements, which are regulatory requirements. Those requirements are captured in section 6.6.4.3 of TS 36.104.

Table 3: Tx spur co-ex requirements for NAM 700MHz bands [TS 36.104]

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band XII or E-UTRA Band 12 or NR Band n12 | 729 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 12 or 85. |
|  | 699 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 12 or 85, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6). |
| UTRA FDD Band XIII or E-UTRA Band 13 or NR Band n13 | 746 - 756 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 13. |
|  | 777 - 787 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 13, since it is already covered by the requirement in clause 6.6.4.2. |
| UTRA FDD Band XIV or E-UTRA Band 14 or NR Band n14 | 758 - 768 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 14. |
|  | 788 - 798 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 14, since it is already covered by the requirement in clause 6.6.4.2. |
| E-UTRA Band 17 | 734 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 17. |
|  | 704 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 17, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6). |
| E-UTRA Band 85 or NR band n85 | 728 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 12, 29 or 85. |
|  | 698 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 85, since it is already covered by the requirement in clause 6.6.4.2. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band (Note 6). |

The above emission levels for the co-existence with 700MHz EUTRA BS are considered to be “typical” values – no good reason was identified so far to motivate why such requirements shall not be applicable to the new Upper_700_MHz_A_LTE band.

Based on the analysis of the co-location and co-existence requirements for 700MHz bands applicable for NAM region, the following is observed:

Observation: for all the NAM 700MHz bands, typical co-location and co-existence requirements are applied in TS 36.104 EUTRA spec.

Based on this the following is proposed:

Proposal 1: Define co-location and co-existence requirements for the new 700MHz NAM band in the EUTRA BS specifications.

Proposal 2: agree on the work-spilt for implementation of related requirements in affected BS RF specifications.

All in all, consideration of the co-ex, and co-location requirements is business-as-usual in RAN4. Those requirements, together with all other RF requirements, are to be completed before WI completion.

# Conclusion

Based on the above, the following proposals are formulated:

Proposal 1: Define co-location and co-existence requirements for the new 700MHz NAM band in the EUTRA BS specifications.

Proposal 2: agree on the work-spilt for implementation of related requirements in affected BS RF specifications

# References

[1]	RP-212618	New WID proposal: Introducing Upper 700 MHz Block A as a new stand-alone band for NB-IoT for Industrial IoT applications, WID

[2]	R4-2119473	Initial discussion on the BS RF requirements, RAN4#101-e

[3]	R4-2119938	Email discussion summary for [101-e][138] LTE_Upper_700MHz