3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2202035

# Electronic Meeting, 17-25 January 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Measurements for CA_n29-n71 MSD

Agenda Item:	5.8.1	NR inter band CA requirements without any FR2 band(s)	[NR_CADC_R17_2BDL_xBUL-Core]

Document for:	Approval

# Introduction

This document presents measurements of a band n71 power amplifier (PA) transmitter (Tx) noise emission level that falls into band n29. We propose MSD levels based on these measurements.

# Discussion

CA_n71-n29 is a low-band (LB) / low-band combination in which the aggressor (band n71 PA) is in close proximity of the band n29 victim. The resulting MSD may be considered as a case of “cross-band isolation” MSD. To the best of our knowledge, there are no commonly agreed assumptions for gNb radio unit deployment scenarios for LB/LB FDD-FDD combinations. We assume throughout this document both gNB radio units are co-located, implying that if the band n29 victim is operated at its reference sensitivity (REFSENS), then the aggressor is also operated at its REFSENS level. This implies that the aggressor UL RB configuration shall be configured with the agreed configurations specified in the REFSENS Table 7.3.2-3. According to the MSD simplification discussion taking place for the BCS4 discussion [1], we propose that the aggressor is configured with the highest supported UL CBW, i.e., 20MHz CBW for band n71, and UL carrier positioned closest to the victim’s band, i.e. for CA_n29-n71, n71 UL carrier set the highest UL channel.

Proposal 1: For FDD-FDD, LB/LB CA combinations, cross-band isolation MSD should be evaluated assuming co-located gNB radio units. This implies that both the aggressor and the victim are operated at their respective REFSENS levels. This means that the aggressor UL RB configuration LCRB shall be configured according to the Table 7.3.2-3 specifications.

## Power Amplifier Calibration

The PA is calibrated using a 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge to meet ACLR -30dBc with 1 dB MPR- PC3, and the following assumptions:

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local oscillator (LO) leakage: -28dBc;

- IQ image rejection: -28dB;

- Candidate waveforms to explore MSD levels, with xRBy denoting, LCRB of “x” RBs at RBstart position of “y”:

- 1RB105;

- 2RB104;

- 3RB103;

- 5RB101;

- 6RB100;

- 10RB96;

- 15RB91;

- 20RB86;

- 25RB81;

- 30RB76;

- 40RB66;

- 50RB56;

- 60RB46;

- 75RB31;

- 80RB26;

- 90RB16;

- 100RB6; and,

- We denote LCRB the number of modulated RBs.

- All waveforms are 20MHz channel bandwidth (CBW) modulated DFT-s-OFDM QPSK;

- The waveforms are filtered at the PA input port so that the waveform noise floor at the victim’s carrier frequencies is below thermal noise level;

- The band n71 20MHz CBW UL carrier is placed closest to the victim’s band n29;

## Measurement Results and MSD proposal

Figure 1 shows the measured n71 PA noise level falling into the following DL band carriers:

- n29 lowermost 5MHz channel (717-722MHz);

- n29 uppermost 5MHz channel (723-728MHz);

- n29 10MHz channel (717-727MHz);

- n71 own 20MHz DL channel to monitor self-desense as LCRB is increased.

Figure 1 n71 20MHz CBW PA Tx noise level falling in n29 and in n71 DL. The blue arrow indicates the n71 20MHz REFSENS UL RB configuration.

Based on proposal 1, n29 MSD with n71 LCRB that corresponds to the n71 20MHz REFSENS UL configuration, ie LCRB = 20. This is indicated by the blue arrow, and the measured noise levels are highlighted with a vertical dashed red-line in Figure 1. The n71 20MHz CBW noise levels plotted LCRB = 20 falling in each of the studied victim’s are:

- n29 lowermost 5MHz channel: -37.6 dBm;

- n29 uppermost 5MHz channel: -47.8 dBm;

- n29 10MHz channel: -36.8 dBm;

- n71 self-desense: -57dBm.

With -57dBm falling in its own DL band, the adoption of n71 UL LCRB = 20 ensures no self-desense occurs.

Based on these results, we propose to MSD evaluation using the RF Front-End parameters shown in Figure 2. For Tx-Rx isolation, we assume 40dB Tx-Rx triplexer isolation and support of the band n71 extension. The lower than usual Tx-Rx isolation is justified by the fact that the 600MHz extended band comes with a lower duplex gap, making triplexer Tx noise rejection to protect band n29 particularly challenging.

Figure 2 n29 MSD evaluation for CA_n29-n71.

Observation: For CA_n29-n71 with n71 UL aggressor configured at 20MHz CBW, LCRB=20RB with both modulated RBs and modulated carrier positioned closest to band n29, we observe the following MSD levels:

- n29 lowermost 5MHz channel: 19dB

- n29 uppermost 5MHz channel: 9.6dB

- n29 10MHz channel: 16.7dB

- no n71 self-desense.

Based on this observation, we make the following proposal.

Proposal 2: For NR intra-band uplink CA MSD core requirements and MSD specifications, adopt the yellow highlighted text and Table 7.3A.2.1-1 changes.

Table 6.x.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n71 | n29 | 19.0 | 16.7 |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n71 | n29 | 15 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). The UL carrier shall be as close as possible to the downlink operating band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we present measurements of n71 PA transmitter noise level falling in band n29 and propose cross-band isolation MSD levels assuming co-located band n71 / band n29 gNB radio units.

Proposal 1: For FDD-FDD, LB/LB CA combinations, cross-band isolation MSD should be evaluated assuming co-located gNB radio units. This implies that both the aggressor and the victim are operated at their respective REFSENS levels. This means that the aggressor UL RB configuration LCRB shall be configured according to the Table 7.3.2-3 specifications.

Proposal 2: For NR intra-band uplink CA MSD core requirements and MSD specifications, adopt the yellow highlighted text and Table 7.3A.2.1-1 changes.

Table 6.x.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n71 | n29 | 19.0 | 16.7 |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n71 | n29 | 15 | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). The UL carrier shall be as close as possible to the downlink operating band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# References

[1] 	R4-2119878	WF on how to improve MSD tables in Rel-17, 3GPP TSG-RAN WG4 e-meeting#101, Skyworks Solutions, Inc.