3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2202036

# Electronic Meeting, 17-25 January 2022

Source: 	Skyworks Solutions, Inc.

Title: 	Measurements for CA_n5-n28 MSD

Agenda Item:	5.8.1	NR inter band CA requirements without any FR2 band(s)	[NR_CADC_R17_2BDL_xBUL-Core]

Document for:	Approval

# Introduction

At RAN4#100 a text proposal for CA_n5-n28 was agreed in which MSD for band n28A ranges from 8.2dB at 5MHz CBW to 0.4dB at 30MHz [1]. This document presents measurements of a band n5 power amplifier (PA) transmitter (Tx) noise emission level that falls into band n28 for CA_n5-n28. We propose MSD levels based on these measurements.

# Discussion

CA_n5-n28 is a low-band (LB) / low-band combination in which the aggressor (band n5 transmitter) is in close-proximity of the band n28 victim. The resulting MSD may be considered as a case of “Cross-band isolation” MSD. To the best of our knowledge, there are no commonly agreed assumptions for gNb radio unit deployment scenarios for LB/LB FDD-FDD combinations. We assume throughout this document that both gNB radio units are co-located, implying that if the band n28 victim is operated at its reference sensitivity (REFSENS), then the aggressor is also operated at its REFSENS level. This implies that the aggressor UL RB configuration should configured with the agreed configurations specified in the REFSENS Table 7.3.2-3. According to the MSD simplification discussion taking place for the BCS4 discussion [2], we propose that the aggressor is configured with the highest supported UL CBW, i.e. 20MHz CBW for band n5, and UL carrier positioned closest to the victim’s band, i.e. for CA_n5-n28, n5 UL carrier set to the lowest UL channel.

Proposal 1: For FDD-FDD, LB/LB CA combination, cross-band isolation MSD should be evaluated assuming co-located gNB radio units. This implies that both the aggressor and the victim are operated at their respective REFSENS levels. This means that the aggressor UL RB configuration LCRB shall be configured according to the Table 7.3.2-3 specifications.

## Power Amplifier Calibration

The PA is calibrated using a 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge to meet ACLR -30dBc with 1 dB MPR- PC3, and the following assumptions:

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- Candidate waveforms to explore MSD levels, with xRBy denoting, LCRB of “x” RBs at RBstart position of “y”:

- 1RB0;

- 2RB0;

- 3RB0;

- 5RB0;

- 6RB0;

- 10RB0;

- 15RB0;

- 20RB0;

- 25RB0;

- 30RB0;

- 40RB0;

- 50RB0;

- 60RB0;

- 75RB0;

- 80RB0;

- 90RB0;

- 100RB0; and,

- We denote LCRB the number of modulated RBs.

- All waveforms are 20MHz channel bandwidth (CBW) modulated DFT-s-OFDM QPSK;

- The waveforms are filtered at the PA input port so that the waveform noise floor at the victim’s carrier frequencies is below thermal noise level;

- The band n5 20MHz CBW UL carrier is placed closest to the victim’s band n28;

## Measurement Results and MSD proposal

Figure 1 shows the measured n5 PA noise level falling into the following DL band carriers:

- band n28A uppermost 5,10,15,20,30MHz channels (denoted n28A 5M,10M,15M,20M,30M);

- n5 own 20MHz DL channel to monitor self-desense as LCRB is increased (denoted n5 self-desense).

Figure 1 n5 20MHz CBW PA Tx noise level falling in n28A and in n5 DL. The blue arrow indicates the n5 20MHz REFSENS UL RB configuration.

Based on proposal 1, n28 MSD is evaluated with n5 LCRB that corresponds to the n5 20MHz REFSENS UL configuration, i.e. LCRB = 20. This is indicated by the blue arrow, and the measured noise levels are highlighted with a vertical dashed red-line in Figure 1. The n71 20MHz CBW noise levels plotted LCRB = 20 falling in each of the studied victim’s are:

- band n28A uppermost 5,10,15,20,30MHz channels: -35.7,-34.5,-34.3,-34.2,-34.2 dBm respectively;

- n5 own 20MHz DL channel: -54 dBm.

With -54dBm falling in its own DL band, the adoption of n5 UL LCRB = 20 ensures no self-desense occurs.

Based on these results, we propose to MSD evaluation using the RF Front-End parameters shown in Figure 2. For Tx-Rx isolation, we assume 45dB Tx-Rx quad-plexer isolation.

Figure 2 n28 MSD evaluation for CA_n5-n28.

Observation: For CA_n5-n28 with n5 UL aggressor configured at 20MHz CBW, LCRB=20RB with both modulated RBs and modulated carrier positioned closest to band n28, we observe the following MSD levels:

- band n28A uppermost 5,10,15,20,30MHz channels: 17.5,15.8,14,11.7,2.9dB respectively;

- no n5 self-desense.

Based on this observation, we make the following proposal.

Proposal 2: For NR intra-band uplink CA MSD core requirements and MSD specifications, adopt the yellow highlighted text and Table 7.3A.2.1-1 changes.

Table 6.x.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n5 | n28 | 17.5 | 15.8 | 14.0 | 11.7 |  | 2.9 |  |  |  |  |  |  |  |

Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n5 | n28 | 15 | 201 | 201 | 201 | 201 |  | 201 |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). The UL carrier shall be as close as possible to the downlink operating band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we present measurements of n71 PA transmitter noise level falling in band n28 and propose cross-band isolation MSD levels assuming co-located band n71 / band n28 gNB radio units.

Proposal 1: For FDD-FDD, LB/LB CA combination, cross-band isolation MSD should be evaluated assuming co-located gNB radio units. This implies that both the aggressor and the victim are operated at their respective REFSENS levels. This means that the aggressor UL RB configuration LCRB shall be configured according to the Table 7.3.2-3 specifications.

Proposal 2: For NR intra-band uplink CA MSD core requirements and MSD specifications, adopt the yellow highlighted text and Table 7.3A.2.1-1 changes.

Table 6.x.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n5 | n28 | 17.5 | 15.8 | 14.0 | 11.7 |  | 2.9 |  |  |  |  |  |  |  |

Table 6.x.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n5 | n28 | 15 | 201 | 201 | 201 | 201 |  | 201 |  |  |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). The UL carrier shall be as close as possible to the downlink operating band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# References

[1] 	R4-2119869, TP to TR 38.717-02-01 Addition of CA_n5-n28, 3GPP TSG-RAN WG4 e-meeting#101, Nokia.

[2] 	R4-2119878	WF on how to improve MSD tables in Rel-17, 3GPP TSG-RAN WG4 e-meeting#101, Skyworks Solutions Inc.