3GPP TSG RAN WG4 Meeting #101-bis-e	R4-2200235

Online, 17th – 25th Jan. 2022

Agenda item:	5.15.1

Source:	SoftBank Corp., LG Electronics

Title:	TP for TR 37.717-21-11: EN-DC_8-41_n1

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_8-41_n1 was approved in RAN#93[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 2-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |

3	Co-existence

When Uplink EN-DC configuration is DC_41A_n1A, IMD5 of (B41 - n1) will fall into Rx band of Band 8. Thus, it is proposed to evaluate.

4	Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 4-1: ΔTIB of constituent 2 band combo.

| ΔTIB | B8+B41 | B8+n1 | B41+n1 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0.3 | 0.3 |  | 0.3 |
| B41 | 0.3 |  | 0 | 0.3 |
| n1 |  | 0.3 | 0 | 0.3 |

Table 4-2: ΔRIB of constituent 2 band combo.

| ΔRIB | B8+B41 | B8+n1 | B41+n1 | Proposed Value |
| --- | --- | --- | --- | --- |
| B8 | 0 | 0 |  | 0 |
| B41 | 0 |  | 0 | 0 |
| n1 |  | 0 | 0 | 0 |

Then the proposal for this 2LTE-1NR EN-DC is:

Table 4-3: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n1 | 8 | 0.3 |
|  | 41 | 0.3 |
|  | n1 | 0.3 |

Table 4-4: ΔRIB,c

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n1 | 8 | 0 |
|  | 41 | 0 |
|  | n1 | 0 |

5	Reference sensitivity exceptions

As mentioned in section 3, REFSENS exception due to IMD5 are expected. The following value is proposed.

Table 5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n1A DC_8A-41C_n1A | 41 | 2500 | 5 | 25 | 2500 | N/A | TDD | N/A |
|  | n1 | 1977 | 5 | 25 | 2167 | N/A | FDD | N/A |
|  | 8 | 886 | 5 | 25 | 931 | 4.5 | FDD | IMD5 |

6	Conclusion

This paper proposes study results of EN-DC 8-41_n1.

TP is shown in Annex.

7	Reference

[1]	RP-212174  WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon

Annex	Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_8-41_n1

5.x.1	Configurations for DC_8-41_n1

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_8A-41A_n1A DC_8A-41C_n1A | DC_8A_n1A DC_41A_n1A |

5.x.2	Co-existence studies

When Uplink EN-DC configuration is DC_41A_n1A, IMD5 of (B41 - n1) will fall into Rx band of Band 8. Thus, it is proposed to evaluate.

5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n1 | 8 | 0.3 |
|  | 41 | 0.3 |
|  | n1 | 0.3 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8-41_n1 | 8 | 0 |
|  | 41 | 0 |
|  | n1 | 0 |

5.x.4	Reference sensitivity exceptions

As mentioned above, IMD of B8 and n1 to Band41 Rx and IMD of B41 and n1 to Band8 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-41A_n1A DC_8A-41C_n1A | 41 | 2500 | 5 | 25 | 2500 | N/A | TDD | N/A |
|  | n1 | 1977 | 5 | 25 | 2167 | N/A | FDD | N/A |
|  | 8 | 886 | 5 | 25 | 931 | 4.5 | FDD | IMD5 |

[Unchanged Parts Skipped]