3GPP TSG-RAN WG4 Meeting #101-bis-e 												R4-2202238

Electronic Meeting, January 17-25, 2022

Agenda item:			9

Source:	Moderator (Apple)

Title:	Email discussion summary for [101-bis-e][138] NR_reply_LS_UE_RF

Document for:	Information

# Introduction

This email thread treats the following topics:

- BC for SDT in RRC_INACTIVE

- RAN5 response LS on LTE REFSENS exception simplification

- FR2 power control for NR-DC

# Topic #1: BC with SDC in RRC_INACTIVE

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201484 | ZTE | Draft LS: There are a need to define the Beam correspondence requirement in RRC_INACTIVE state for Configured Grant SDT and/or Random Access SDT and Rel-16 SSB only based eBC requirement could be used as baseline. |
| R4-2201973 | Qualcomm | Proposal: RAN4 to agree in principle on addressing beam correspondence outside connected more via an objective for Rel-18 FR2 enhancement. |

## Open issues summary

Note that in previous RAN4 meetings, we discussed whether there is a need to define the beam correspondence requirement in RRC_INACTIVE state for Configured Grant SDT and/or Random Access SDT in R17 and there was no agreement. So the plan at this meeting is not to re-discuss it again despite the contribution R4-2201484. Instead, it seems reasonable to focus the discussion on the proposal from R4-2201973.

### Sub-topic 1-1: RAN4 to agree in principle on addressing beam correspondence outside connected more via an objective for Rel-18 FR2 enhancement.

- Proposals (when picking an option, please state reasons)

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | (sorry for the obvious editorial failure in our proposal) We favor ‘RAN4 to agree in principle on addressing beam correspondence outside connected mode via an objective for Rel-18 FR2 enhancement’ if there is no agreement on how to respond to RAN1 |
| Nokia | Option 1: We support addressing beam correspondence outside connected mode via an objective for Rel-18 FR2 enhancement. We have proposed this also earlier as we think it is important functionality for FR2 system performance and operations that is now missing. Also RAN1 has come to the same conclusions of the need for extending beam correspondence requirements beyond RRC_CONNECTED as stated in RAN1’s LS to RAN4. |
| Sony | We support this proposal (option 1). |
| MediaTek | Option 2. RAN plenary would be a suitable place to discuss R18 objective. |
| Ericsson | Option 1. We have proposed requirements for initial access since Rel-15 (together with Sony) for this is crucial for basic FR2 performance, and no network assistance available. It is also important for SDT functionality and performance. |
| Apple | Option 2. It is OK for proponent companies to propose “BC outside connected mode” for R18, and this proposal will be subject to further discussion/consideration just like other RAN4 R18 proposals. |
| OPPO | Option 2, same view as Apple. |
| Huawei, HiSilicon | Option 2. R18 objective is a RAN level discussion, it is not appropriate to decide it in RAN4, especially in a bis meeting. |
| vivo | Option 2. |

### Sub-topic 1-2: If there is an agreement on Sub-topic 1-1, should RAN4 send a reply LS to close the discussion in R17?

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | Option 1, but we would need a supporting WF or other kind of agreement first. |
| Nokia | Option 1: If agreement is reached then RAN1 should be informed. |
| MediaTek | Option 1. If we agree not to introduce it in R17, it’s fine to inform RAN1. However, about R18, we shall discuss it in RAN plenary. |
| Ericsson | Option 1. |
| Apple | Option 1. Given R17 time window is closing, we propose to send a reply to RAN1 no matter whether there is an agreement or disagreement on sub-topic 1-1. |
| OPPO | Option 1, same as MTK |
| Huawei, HiSilicon | Option 1. We agree to send an LS to RAN1 focusing on the question only asked by RAN1, but certainly the content should be further discussed in RAN4. |
| vivo | Option 1. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section “Open issues summary” above.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | There are split views, with 4 companies (Qualcomm, Nokia, Sony, Ericsson) supporting option 1, 5 companies (MediaTek, Apple, OPPO, Huawei, vivo) supporting option 2. Recommendations for 2nd round: There is no need to further discuss it in the second round, given this issue has been discussed in the last few meetings and companies’ positions have remained unchanged. Instead, based on the comments, it seems OK that the proposal to address beam correspondence outside connected mode via an objective for Rel-18 FR2 enhancement can be submitted to the ongoing RAN4 R18 package discussion. |
| Sub-topic #1-2 | As there is no agreement reached for sub-topic 1-1, it is recommended to not send an LS based on the different views received in the first round, some saying that an LS can be sent only when there is an agreement on sub-topic 1-1, while others saying an LS can be sent to just inform RAN1 that RAN4 will not specify BC for RRC_INACTIVE mode. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: RAN5 response LS on LTE REFSENS exception simplification (R5-215803)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201247 | Huawei, HiSilicon | Proposal 1: RAN4 should agree option 2a which was proposed by RAN5 when we simplify the LTE REFSENS. And the decision should be provided to RAN5 in this meeting. Observation 1: For new Rel-17 band combinations, REFSENS test points have been removed if already covered by fall-back combination via small CR in TS 36.101. For legacy combinations, all redundant REFSENS test points should be removed and keep only the lowest order fall-back test points. One clarification that simplified REFSENS requirements in Rel-17 specifications could be release independently supported by earlier Ues is needed. Proposal 2: To implement the CR of LTE REFSENS simplification according to Table 1. Proposal 3: Considering the workload, a work split can be preferred to implement this Rel-17 CR in next RAN4 meeting. The draft LS is attached. |

## Open issues summary

### Sub-topic 2-1: Is Proposal 1 (RAN4 should agree option 2a which was proposed by RAN5 when we simplify the LTE REFSENS. And the decision should be provided to RAN5 in this meeting) agreeable?

Note: after discussion on proposals 1 and 2, we can discuss and revise the reply LS if needed

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Huawei | Option 1 |
| Nokia | Option 1 as it is according to RAN5 feedback. |
| Skyworks | Option 1 i.e, adopt option 2a proposed by RAN5 in LS Reply R4-2117027. |
| Apple | Option 1 |
| CHTTL | Option 2, one clarification question is that if the higher order test points are completely removed, how to ensure the performance of the high order CA combo? |

### Sub-topic 2-2: Is Proposal 2 (To implement the CR of LTE REFSENS simplification according to Table 1) agreeable?

- Proposals

  - Option 1: Yes

  - Option 2: No (please also suggest possible revisions)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Huawei | Option 1 |
| Nokia | Option 1: Table 1 is aligned as far we have understood with approved WF R4-2016940 from Skyworks, Nokia and Huawei. |
| Skyworks | Option 1; we share the same views with Nokia: this is in-line with the initiative the three companies co-signed in the agreed WF R4-2016940, prompted by the initial R4-2016007 proposals, and later on pursued by Nokia and Skyworks in R4-2110817. |
| Apple | Option 1 |
| CHTTL | Option 2, probably carefully review is needed before removing the table if some of the 2 band requirement is missed but specify in the highest order. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section “Open issues summary” above.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Four companies supported Option 1 and one company supported Option 2 and asked a clarification question. Recommendations for 2nd round: Can proponent companies address the clarification question in order to seek an agreement? Also, a revised LS will be assigned to Huawei. If there is an agreement, the LS can be sent. |
| Sub-topic#2-2 | Four companies supported Option 1 and one company supported Option 2 and commented that careful review is needed before removing the table. Recommendations for 2nd round: Can proponent companies address the comment from the opposing company in order to seek an agreement? |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: FR2 power control for NR-DC

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2201277 | OPPO | Observation 1: FR2 NR-DC hasn’t been discussed in RAN4, but NR CA conclusion can be taken as reference. Observation 2: Hardware are shared by CBM inter-band UL CA, thus it is not independent power control. Observation 3: It was agreed that for total power handling the UE power consumption should be addressed, and no matter what kind of solutions defined in the end, it is clear that the two bands with IBM capability are not independent of each other. Observation 4: For IBM inter-band UL CA, it is not independent power control. Proposal 1: It is proposed to inform RAN1 that FR2 intra-band or inter-band NR DC power control are not independent. Draft LS is attached. |

## Open issues summary

### Sub-topic 3-1: Is the draft LS agreeable? Please also provide your detailed comments if any revision is needed.

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |
| Qualcomm | Option 2: Not agreeable. Thank you Oppo for the good paper and draft LS. Observation 3 in R4-2202177 may not be correct, however. ‘Total power concept’ is not agreeable to many companies in discussion of FR2+FR2 inter-band ULCA. The LS presumes that IBM UEs will be subject to restrictions based on ‘total power concept’. We do not see a physical reason that forces mutual dependency of UL powers during inter-band ULCA operation. See R4-2201967 for further details. |
| Nokia | LS is not agreeable. RAN4 has not agreed total power concept, instead many companies including Nokia have strongly questioned the concept. Total power concept would make UL CA useless. |
| MediaTek | We think it is true that FR2 power control is NOT fully independent. For example, single-chain for inter-band CA. It would be good to further discuss LS content based on R4-2201277. |
| Ericsson | Option 2. We agree that the NR DC can be based on NR CA. However, RAN4 requirements for inter-band UL CA are not completed. The power control is independent as specified in 38.213, but the PCMAX for the CA configuration limits the total power and implies dependence for any CA configuration. Then the UE can apply P-MPR on each of the UL serving cells for various reasons (not only due to unduly frying the user). PCMAX is specified in an implementation specific plane of reference for intra-band UL CA in FR2, but not yet specified for inter-band UL CA. The total power concept is not agreed by RAN4. |
| Apple | In general, we share a similar view that for inter-band CA, even for the case of IBM, there is a total power limitation for the UE. This is because to deal with UE power dissipation/heating, there should be some limit on how high the UE TX power (max. TRP) can be in real implementation. Whether this implementation constraint needs to be explicitly considered in standards can be further discussed. For instance, if there is a power limit for each constituent band, then as long as the TX power per band does not exceed the limit, power control can be considered independent. But before such a limit is specified, it may be hard to conclude independent power control is the case regardless. In addition, since the LS reply is on power control for NR-DC, we need to clearly mention that conclusion/derivation of the inter-band CA can apply to NR-DC, which is missing in the draft LS. |
| OPPO | Thanks for the comments, regarding total power concept, actually it is not the intention to introduce the total power limitation in the spec as in FR1 did. However, total power concept means UE may not be able to transmit the power in one band without considering the other band Tx power due to the reasons like Apple has commented power consumption and heating issues which exists already in todays even single band FR2 UE. From implementation perspective, we don’t believe the two bands can transmit max power as single band can do. Total power control even not defined in spec, it will be controlled in UE itself. Therefore, in RAN1 power control design, this should be clear no matter finally RAN4 define total power limit or not. |
| Huawei, HiSilicon | We support the proposal and think that total power should be considered to address the power consumption, heat sink issues. The implementation issues mentioned by OPPO should be considered when we have following discussion for total power concept. |
| vivo | In our last meeting paper R4-2118291, a complete analysis and two choices of LS versions based on how “total power concept” would be treated were provided. This concept would be still under discussion in this meeting. Actually, this is a Rel-16 LS and are outside the scope of this agenda which is specific for Rel-17, so no paper was submitted in this meeting. Like other Rel-15/Rel-16 LS, it is proposed not treat this LS and postpone this topic to next meeting. |

## Companies views’ collection for 1st round

### Open issues

Comments are collected in section “Open issues summary” above.

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Three companies (Qualcomm, Nokia, Ericsson) supported Option 2, and four companies (MediaTek, Apple, OPPO, Huawei) were positive toward the views in R4-2201277 and commented the draft LS needs to be revised. Vivo commented “this is a Rel-16 LS and are outside the scope of this agenda which is specific for Rel-17.” After vivo’s comment, moderator checked and it is indeed a Rel-16 LS, as stated in R4-2107605(R1-2104018). Apologies for not detecting it earlier. Recommendations for 2nd round: Since R4-2201277 concerns a Rel-16 LS, it should not be treated in this meeting following RAN4 chair’s guidance. Therefore, there is no further discussion in the second round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| Response LS to RAN5 on LTE REFSENS Exceptions Simplification | Huawei, HiSilicon | To: TSG RAN WG5 |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2201484 |  |  | Noted |  |
| R4-2201973 |  |  | Noted |  |
| R4-2201247 |  |  | Noted |  |
| R4-2201277 |  |  | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Skyworks Solutions Inc. | Laurent Noel | laurent.noel@skyworksinc.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)