3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202244

# Electronic meeting, January 17th-25th, 2022

Source:		UIC

Title:			Proposed changes for clause 7.2 in 3GPP TR 38.852

Agenda item:		5.5.4

Document for:	Approval

# Introduction

Clause 7.2 establishes the necessary context for UE RF in the RMR 1900 context. The proposed text updates will improve readability and context.

# Proposed changes

***********************************************1st Change*******************************************************

### 7.2.1		Transmitter characteristics

The UE requirements for RMR 1900 are summarized in the upcoming tables applicable for 3GPP TS 38.101-1 [2].

Table 7.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n101 |  |  |  |  |  |  | 23 | ±2 |

Table 7.2.1-2: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n101 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 41, 42, 43, 50, 51, 52, 65, 67, 68, 69, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |

********************************************End of Changes****************************************************