3GPP TSG-RAN WG4 Meeting #101bis-e 												R4-2202260

Electronic Meeting, 17 – 25th January 2022

Agenda item:			5.2

Source:	Apple Inc.

Title:	WF on introduction of the full unlicensed band

Document for:	Information

# Introduction

3GPP Rel-16 NR-U WI specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  5GHz is a well-known band for the unlicensed operation, but 6GHz is a relative new band usage of which was approved recently in different regulatory regions. While the 6GHz band for the US is already part of the Rel-16 core functionality, current 3GPP specifications do not support it for other countries, such as South Korea and Canada, which have finalised their regulatory requirements only recently. Thus, RAN#92 meeting approved a new WI aim of which is to enable support of the 6GHz unlicensed band for those countries and regions that have finalised recently the corresponding regulatory requirements.

This document presents a way forward based on the agreements made during the [101-e][102] NR_6GHz_unlic_full email discussion (RAN4#101bis meeting).

# Way forward

## Summary of the NS values

The following summary is proposed that shows which NS values can be re-used and which new NS values are needed.

Agreements:

- NS_53 can be re-used for LPI Colombia.

- Endorse the following table with updated NS values (also included into TP in R4-2202263).

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 2 |  |  |  |
| Canada | NS_54 | NS_x1 (new) | TBD |
| Brazil | N/A | NS_53 | TBD |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | TBD |
| Colombia |  | NS_53 |  |
| Region 3 |  |  |  |
| South Korea | N/A | NS_x2 (new) | TBD |

## Band n96 and its applicability

As per the agreement from the RAN4#100 meeting, band n96 can be used not only in US, but also in other countries where the full frequency band is used for unlicensed operation. Thus, it was suggested to revise the existing band n96 NOTE so that it does not limit band n96 applicability only to US. As a related aspect, it was also discussed starting from which release the corresponding NOTE should be changed.

No conclusion was reached during the RAN4#101bis meeting.

Candidate options:

-	Package 1:

-	The NOTE 14 is kept but the wording is changed to "This band is only applicable subject to regional and/or country specific restrictions".

-	The same change is applied to band n96 in Rel-16 and Rel-17.

- Package 2:

-	The NOTE 14 is kept but the wording is changed to "This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions ".

-	The same change is applied to band n96 in Rel-16 and Rel-17.

NOTE:	One company holds the opinion that Rel-16 specifications should not be changed at all.

## UE RF aspects

Agreements:

-	For LPI:

-	Content of the draft running CR in R4-2202262 is technically endorsed.

-	For VLP:

-	PC5 +20dBm (with the corresponding A-MPR values) can be used to support VLP. PC5 VLP can be used already in Rel-17.

-	A new power class PC6 [+14dBm], if considered, is subject for further discussions in Rel-18

-	PC5 VLP A-MPR values for South Korea from R4-2202261 (LGE) are taken as a starting point but need more analysis for the edge and mid channels. The corresponding technical input is anticipated next meeting.

## BS RF aspects

Agreement:

-	It was further confirmed that there is no BS RF impact (the corresponding TP is agreed in R4-2201515).