3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202267

# Electronic meeting, January 17-25, 2022

Source:		Ericsson

Title:			WF on UE RF requirements for 6GHz licensed band

Agenda item:		6.13.3.4

Document for:		Approval

# Tx requirements

## Maximum output power

Tentative agreement:

Specify PC3 1 Tx (FFS if 2Tx) as first priority and default power class.

Additionally specify PC2 (1Tx) if time allows (2Tx is FFS)

Note: FFS if any clarification from RCC is needed on UE max output power, this is handled in the WF on general aspects.

## MPR

Way Forward:

FFS, also pending on ACLR and SEM.

Transceiver and PA performance as well as trace loss and component losses for this frequency range could also be further evaluated for next meeting.

Further investigate if current MPR should be maintained, relaxed, or tightened.

## Output power dynamics

Agreement:

Requirements specified in TS 38.101-1 sub-clause 6.3 apply for the new 6GHz licensed band

## Transmit signal quality

Agreement:

Requirements specified in TS 38.101-1 sub-clause 6.4 apply for the new 6GHz licensed band.

## Occupied bandwidth

Agreement:

Requirement specified in TS 38.101-1 sub-clause 6.5.1 shall also apply for the new 6GHz licensed band.

## Spectral emission mask

Tentative agreement:

Option 1: Adopt the SEM requirement defined in TR 38.921 for 6425-7125MHz assuming UE PC3 is

specified.

Option 2: Adopt SEM as defined in sub-clause 6.5.2.2 of TS 38.101-1 assuming UE PC3 is specified

## ACLR

Tentative agreement:

Option 1: Adopt 26 dB ACLR for 6.425 - 7.125 GHz band (PC3 UE).

Option 2: Adopt 30 dB ACLR for 6.425 – 7.125 GHz band (PC3 UE).

## Spurious emission – general

Agreement:

Requirements specified in TS 38.101-1 sub-clause 6.5.3.1 apply for the new 6GHz licensed band.

## Transmit intermodulation

Agreement:

Requirements specified in TS 38.101-1 sub-clause 6.5.4 apply for the new 6GHz licensed band.

# Rx requirements

## REFSENS

Way forward:

Companies are encouraged to provide further analysis on the UE NF for this frequency range and check the following options:

  - Option 1: Same REFSENS as n78 and n79 for the new band

  - Option 2: Adopt 10.5 dB NF for 6GHz NR band.

  - Option 3: FFS

## Maximum input level

Agreement:

Requirements specified in TS 38.101-1 sub-clause 6.3 apply for the new 6GHz licensed band.

## ACS

Tentative agreement:

Option 1: Adopt 32 dBc adjacent channel selectivity (ACS) for 6.425 - 7.125 GHz band.

Option 2: Adopt 33 dB adjacent channel selectivity (ACS) for 6.425 – 7.125 GHz band in accordance with sub-clause 7.5 of TS 38.101-1

## Blocking

Way forward

Further investigate the blocking requirement.

Note: FFS if any clarification from RCC is needed, this is handled in the WF on general aspects.

## Intermodulation

Agreement:

Requirement specified in TS 38.101-1 sub-clause 7.8 shall also apply for the new 6GHz licensed band.

## Spurious emissions

Agreement:

Requirement specified in TS 38.101-1 sub-clause 7.9 shall also apply for the new 6GHz licensed band.

# References

- R4-2200150, Discussion on UE RF requirements for 6GHz licensed band, CATT

- R4-2201305, Discussion on UE RF requirements for 6G license band, Xiaomi

- R4-2201332, 6GHz licensed band - UE requirements, Ericsson

- R4-2201505, UE RF requirements, Huawei, HiSilicon, CMCC, China Unicom, OPPO

- R4-2201506, Draft CR for 38.101-1: 6GHz NR licensed band, Huawei, HiSilicon,CMCC, China Telecom, China Unicom, OPPO

- R4-2201826, draft CR on introduction of 6GHz UE RF requirements for 38.101-1, ZTE Corporation

- R4-2201855, 6 GHz licensed band, Qualcomm Incorporated