3GPP TSG-RAN WG4 Meeting #101-bis-e	R4-2202268

# Electronic meeting, January 17th – 25th, 2022

Source:		CATT

Title:			WF on BS RF requirements for 6GHz licensed band

Agenda item:		5.3

Document for:	Approval

- BS RF requirements

1.1 BS type

Specify BS type 1-C, BS type 1-H and type 1-O for the 6GHz licensed band

1.2 Conducted Tx requirements

The following proposals are agreeable. OBUE and spurious are need to be agreed together with ΔfOBUE.

- Table 1.2-1 conducted Tx requirements

| Requirement | WF |
| --- | --- |
| BS output power | Requirement specified in TS 38.104 sub-clause 6.2.3 shall also apply for the new 6GHz licensed band. |
| RE power control dynamic range | Requirement specified in TS 38.104 sub-clause 6.3.2 shall also apply for the new 6GHz licensed band. |
| Total power dynamic range | Requirement specified in TS 38.104 sub-clause 6.3.3 shall also apply for the new 6GHz licensed band. |
| Transmit OFF power | Requirement specified in TS 38.104 sub-clause 6.4.1 shall also apply for the new 6GHz licensed band. |
| Transient period | Requirement specified in TS 38.104 sub-clause 6.4.2 shall also apply for the new 6GHz licensed band. |
| Frequency error | Requirement specified in TS 38.104 sub-clause 6.5.1 shall also apply for the new 6GHz licensed band. |
| Modulation quality | Requirement specified in TS 38.104 sub-clause 6.5.2 shall also apply for the new 6GHz licensed band. |
| Time alignment error | Requirement specified in TS 38.104 sub-clause 6.5.3 shall also apply for the new 6GHz licensed band. |
| Occupied bandwidth | Requirement specified in TS 38.104 sub-clause 6.6.2 shall also apply for the new 6GHz licensed band. |
| ACLR | Based on the coexistence study, an ACLR value of 38dB shall apply for the band. |
| OBUE | For Wide BS, the OBUE mask for 6425-7125MHz in TR 38.921 sub-clause 6.1.2 shall apply. For Medium range and Local area BS, see table 1.2-2a, 1.2-2b and 1.2-3.. |
| Transmit spurious | The requirement in sub-clause 6.1.4 of TR 38.921 shall apply for licensed band 6425-7125MHz. |
| Transmitter intermodulation | Requirement specified in TS 38.104 sub-clause 6.7.3 shall also apply for the new 6GHz licensed band. |

- Table 1.2-2a. MR BS UEM limit values for 6425-7125MHz, 31< Prated,x  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | Prated,x-53dB-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | Prated,x - 60dB | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | Min(Prated,x - 60dB, -25dBm) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,x -60dB, -25dBm)/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

- Table 1.2-2b. MR BS UEM limit values for 6425-7125MHz, Prated,x  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -22dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -29 dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -29dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

- Table 1.2-3. LA BS UEM limit values for 6425-7125MHz,

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -30dBm-750f_offsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -37 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -37 dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -37dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

1.3 Conducted Rx requirements

The following proposals are agreeable. Narrow band blocking and intermodulation requirements are not applicable; wanted and interfering signal levels for ICS and dynamic range requirement should be updated with 1dB higher NFs.

- Table 1.3-1 conducted Rx requirements

| Requirement | WF |
| --- | --- |
| Reference sensitivity | Reference sensitivity requirements in TS 38.104 sub-clause 7.2.2 shall be updated considering the agreed Noise Figure (6 dB for Wide Area BS, 11 dB for Medium Range BS and 14 dB for Local Area BS). |
| Dynamic range | Wanted and interfering signal levels for dynamic range requirement should be updated with 1dB higher NFs. |
| ACS | Based on TR 38.921 sub-clause 6.2.4, an ACS value of 42dB shall be specified. |
| In-band blocking | The in-band blocking has been agreed in TR 38.921 sub-clause 6.2.3 shall be specified |
| Out-of-band blocking | Requirement specified in TS 38.104 sub-clause 7.5.2 shall also apply for the new 6GHz licensed band. For ΔfOOB requirement, see Issue 4-1-8A. New NOTE proposed in R4-2201508 is to be discussed. |
| Receiver spurious | Requirement specified in TS 38.104 sub-clause 7.6.4 shall also apply for the new 6GHz licensed band. |
| Receiver intermodulation | Requirement specified in TS 38.104 sub-clause 7.7.2 shall also apply for the new 6GHz licensed band. |
| In-channel selectivity | Wanted and interfering signal levels for ICS requirement should be updated with 1dB higher NFs. |

1.4 OTA Tx requirements

The following proposals are agreeable. OBUE and spurious need are to be agreed together with ΔfOBUE.

- Table 1.4-1 OTA Tx requirements

| Requirement | Suggested limits |
| --- | --- |
| Radiated transmit power | Requirement specified in TS 38.104 sub-clause 9.2.2 shall also apply for the new 6GHz licensed band. |
| OTA BS output power | Requirement specified in TS 38.104 sub-clause 9.3.2 shall also apply for the new 6GHz licensed band. |
| OTA RE power control dynamic range | Requirement specified in TS 38.104 sub-clause 9.4.2.2 shall also apply for the new 6GHz licensed band. |
| OTA total power dynamic range | Requirement specified in TS 38.104 sub-clause 9.4.3.2 shall also apply for the new 6GHz licensed band. |
| OTA transmit OFF power | Requirement specified in TS 38.104 sub-clause 9.5.2.2 shall also apply for the new 6GHz licensed band. |
| OTA transient period | Requirement specified in TS 38.104 sub-clause 9.5.3.2 shall also apply for the new 6GHz licensed band. |
| OTA frequency error | Requirement specified in TS 38.104 sub-clause 9.6.1.2 shall also apply for the new 6GHz licensed band. |
| OTA modulation quality | Requirement specified in TS 38.104 sub-clause 9.6.2.2 shall also apply for the new 6GHz licensed band. |
| OTA time alignment error | Requirement specified in TS 38.104 sub-clause 9.6.3.2 shall also apply for the new 6GHz licensed band. |
| OTA occupied bandwidth | Requirement specified in TS 38.104 sub-clause 9.7.2.2 shall also apply for the new 6GHz licensed band. |
| OTA ACLR | The requirements are referring to conducted requirement. No change is needed. |
| OTA OBUE | The requirements are referring to conducted requirement. No change is needed. |
| OTA transmit spurious | The requirements are referring to conducted requirement. No change is needed. |
| OTA transmitter intermodulation | Requirement specified in TS 38.104 sub-clause 9.8.2 shall also apply for the new 6GHz licensed band. |

1.5 OTA Rx requirements

The following proposals are agreeable. Narrow band blocking and intermodulation requirements are not applicable; wanted and interfering signal levels for ICS and dynamic range requirements should be updated with 1dB higher NFs.

- Table 1.5-1 OTA Rx requirements

| Requirement | Suggested limits |
| --- | --- |
| OTA sensitivity | Requirement specified in TS 38.104 clause 10.2.1 shall also apply for the new 6GHz licensed band. |
| OTA reference sensitivity | The requirements are referring to conducted requirement. No change is needed. |
| OTA dynamic range | The requirements are referring to conducted requirement. |
| OTA ACS | The requirements are referring to conducted requirement. No change is needed. |
| OTA in-band blocking | The requirements are referring to conducted requirement. No change is needed. |
| OTA out of band blocking | Requirement specified in TS 38.104 sub-clause 10.6.2 shall also apply for the new 6GHz licensed band. |
| OTA receiver spurious | Requirement specified in TS 38.104 sub-clause 10.7.2 shall also apply for the new 6GHz licensed band. |
| OTA receiver intermodulation | Requirement specified in TS 38.104 sub-clause 10.8.2 shall also apply for the new 6GHz licensed band. |
| OTA in-channel selectivity | Wanted and interfering signal levels for OTA ICS requirement should be updated with 1dB higher NFs. |

1.6 ΔfOBUE

FFS

1.7 ΔfOOB

FFS

2 References

- R4-2200153, Analysis on BS requirements for operation in 6GHz band, CATT

- R4-2201331, 6GHz licensed band - BS requirement, Ericsson

- R4-2201449, Discussion on BS RF requirements for 6425-7125MHz, ZTE

- R4-2201507, BS RF requirements, Huawei, HiSilicon, China Unicom, CMCC